3.  Programming Interface

This chapter contains information needed to write custom software drivers for the EXM-23.  If using standard VGA drivers, skip this chapter.

The EXM-23 is mapped into both the memory space and I/O space.  In the memory space, the mapping is as follows:

| Range |  | Content |
| --- | --- | --- |
| 000A0000 | 000BFFFF | Video RAM |
| 000C0000 | 000C7FFF | Video BIOS ROM |

Table 2. Video Memory Mapping.

The EXM-23 defines the following registers in the I/O space:

| Address | Read | Write |
| --- | --- | --- |
| 3B0 | Reserved for MDA/Hercules | Reserved for MDA/Hercules |
| 3B1 | Reserved for MDA/Hercules | Reserved for MDA/Hercules |
| 3B2 | Reserved for MDA/Hercules | Reserved for MDA/Hercules |
| 3B3 | Reserved for MDA/Hercules | Reserved for MDA/Hercules |
| 3B4 | CRTC Index | CRTC Index |
| 3B5 | CRTC Data | CRTC Data |
| 3B6 | Reserved for MDA/Hercules | Reserved for MDA/Hercules |
| 3B7 | Reserved for MDA/Hercules | Reserved for MDA/Hercules |
| 3B8 | Hercules Mode Register (Mode) | Hercules Mode Register (Mode) |
| 3B9 | -- | Set Light Pen FF (ignored) |
| 3BA | Status Register (STAT) | Feature Control Register (FCR) |
| 3BB | -- | Clear Light Pen FF (ignored) |
| 3BC | Reserved for system parallel port |  |
| 3BD | Reserved for system parallel port |  |
| 3BE | Reserved for system parallel port |  |
| 3BF | Hercules Configuration Register (HCFG) | Hercules Configuration Register (HCFG) |

Table 3. Monochrome Mode I/O Map.

| Address | Read | Write |
| --- | --- | --- |
| 3C0 | Attribute controller index/data | Attribute controller index/data |
| 3C1 | Attribute controller index/data | Attribute controller index/data |
| 3C2 | Feature Read Register (FCR) | Miscellaneous Output Register |
| 3C3 | Video Subsystem Enable (VSE) (MC/PI/LB) | Video Subsystem Enable (VSE) MC/PI/LB) |
| 3C4 | Sequencer index | Sequencer index |
| 3C5 | Sequencer data | Sequencer data |
| 3C6, 83C6 | Color Palette Mask | Color Palette Mask |
| 3C7, 83C7 | Color Palette State | Color Palette Read Mode Index |
| 3C8, 83C8 | Color Palette Write Mode Index | Color Palette Write Mode Index |
| 3C9, 83C9 | Color Palette Data | Color Palette Data |
| 3CA | Feature Read Register (FEAT) | -- |
| 3CB | -- |  |
| 3CC | Miscellaneous Output Register | -- |
| 3CD | -- |  |
| 3CE | Graphics Controller Index | Graphics Controller Index |
| 3CF | Graphics Controller Data | Graphics Controller Data |

Table 4. I/O Map Used By Both Monochrome and Color Modes.

| Port Address | Read | Write |
| --- | --- | --- |
| 3D0 | -- | -- |
| 3D1 | -- | -- |
| 3D2 | -- | -- |
| 3D3 | -- | -- |
| 3D4 | CRTC Index | CRTC Index |
| 3D5 | CRTC Data | CRTC Data |
| 3D6 | CHIPSÔ Extensions Index | CHIPSÔ Extensions Index |
| 3D7 | CHIPSÔ Extensions Data | CHIPSÔ Extensions Data |
| 3D8 | CGA Mode Register (MODE) | CGA Mode Register (MODE) |
| 3D9 | CGA Color Register (COLOR) | CGA Color Register (COLOR) |
| 3DA | Status Register (STAT) | Feature Control Register (FCR) |
| 3DB | -- | Clear Light Pen FF (ignored) |
| 3DC | -- | Set Light Pen FF (ignored) |
| 46E8 | -- | Setup Control (ISA bus only) |

Table 5. Color Mode Registers.

Hercules, CGA, MDA and EGA mode registers are included for completeness, but the Chips & Technologies specification sheet for the 65535 chip set will probably be required.

| Register | Register Name | Bits | Access | I/O Port - MDA/Herc | I/O Port - CGA | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| ST00 (STAT) | Display Status | 7 | R | 3BA | 3DA |  |
| CLPEN | Clear Light Pen Flip Flop | 0 | W (n/a) | 3BB (ignored) | 3DB (ignored) | ref only; no light pen |
| SLPEN | Set Light Pen Flip Flop | 0 | W (n/a) | 3B9 (ignored) | 3DC (ignored) | ref only; no light pen |
| MODE | CGA/ MDA/ Herc | 7 | RW | 3B8 | 3D8 |  |
| COLOR | CGA Color Select | 6 | RW | n/a | 3D9 |  |
| HCFG | Hercules Config | 2 | W R | 3BF 3D6-3D7 index 14 | n/a n/a | XR14 |
| RX, R0-11 | '6845' registers | 0-8 | RW | 3B4-3B5 | 3D4-3D5 |  |
| XRX, XR0-7F | Extension Registers | 0-8 | RW | 3D6-3D7 | 3D6-3D7 |  |

Table 6. Register Summary - CGA, MDA, and Hercules Modes.

| Register | Register Name | Bits | Access | I/O Port - Mono | I/O Port - Color | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| MSR | Misc Output | 7 | W | 3C2 | 3C2 |  |
| FCR | Feature Control | 3 | W | 3BA | 3DA |  |
| ST00 (FEAT) | Feature Read Input Stat 0 | 4 | R | 3C2 | 3C2 |  |
| ST01 (STAT) | Display Status Input Stat 1 | 7 | R | 3BA | 3DA |  |
| CLPEN | Clear Light Pen Flip Flop | 0 | W (n/a) | 3BB (ignored) | 3DB (ignored) | ref only; no light pen |
| SLPEN | Set Light Pen Flip Flop | 0 | W (n/a) | 3B9 (ignored) | 3DC (ignored) | ref only; no light pen |
| SRX, SR0-7 | Sequencer | 0-8 | RW | 3C4-3C5 | 3C4-3C5 |  |
| CRX, CR0-3F | CRT Controller | 0-8 | RW | 3B4-3B5 | 3D4-3D5 |  |
| GRX, GR0-8 | Graphics Controller | 0-8 | RW | 3CE-3CF | 3CE-3CF |  |
| ARX, AR0-14 | Attributes Controller | 0-8 | RW | 3C0-3C1 | 3C0-3C1 |  |
| XRX, XR0-7F | Extension Registers | 0-8 | RW | 3D6-3D7 | 3D6-3D7 |  |

Table 7. Register Summary - EGA Mode.

| Register | Register Name | Bits | Access | I/O Port - Mono | I/O Port - Color | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| VSE | Video Subsystems Enable | 1 | RW | 3C3 if MC/PI/LB | 3C3 if MC/PI/LB | Disabled by XR70 bit-6 |
| SETUP | Setup Control | 2 | W | 46E8 if ISA | 46E8 if ISA | Disabled by XR70 bit-7 |
| ENABLE | Global Enable | 1 | RW | 102 if ISA/MC | 102 if ISA/MC | Setup Only |
| MSR | Misc output | 7 | W R | 3C2 3CC | 3C2 3CC |  |
| FCR | Feature Control | 3 | W R | 3BA 3CA | 3DA 3CA |  |
| ST00 (FEAT) | Feature Rd (status 0) | 4 | R | 3C2 | 3C2 |  |
| ST01 (STAT) | Display stat (status 1) | 6 | R | 3BA | 3DA |  |
| CLPEN | Clear Light Pen flip flop | 0 | W (n/a) | 3BB (ignored) | 3DB (ignored) | ref only; no pen |
| SLPEN | Set Light Pen flip flop | 0 | W (n/a) | 3B9 (ignored) | 3DC (ignored) | ref only; no pen |
| DAC MASK | Color palette pixel mask | 8 | RW | 3C6, 83C6 | 3C6, 83C6 |  |
| DAC STATE | Color Palette State | 2 | R | 3C7, 83C7 | 3C7, 83C7 |  |
| DACRX | Color palette read index | 8 | W | 3C7, 83C7 | 3C7, 83C7 |  |
| DACWX | Color palette write index | 8 | RW | 3C8, 83C8 | 3C8, 83C8 |  |
| DAC DATA | Color palette data 0-FF | 3x6 3x8 | RW | 3C9, 83C9 | 3C9, 83C9 |  |
| SRX | Sequencer | 0-8 | RW | 3C4-3C5 | 3C4-3C5 |  |
| CRX | CRT Ctrlr | 0-8 | RW | 3B4-3B5 | 3D4-3D5 |  |
| GRX | Graph Ctrlr | 0-8 | RW | 3CE-3CF | 3CE-3CF |  |
| ARX | Attrib Ctrlr | 0-8 | RW | 3C0-3C1 | 3C0-3C1 |  |
| XRX | Exten Ctrlr | 0-8 | RW | 3D6-3D7 | 3D6-3D7 |  |

Table 8. Register Summary - VGA Mode.

## Standard EXM Registers

The Devices ID and Configuration Option Byte 1 registers are described below:

|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 | I/O port |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Device ID Reg | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 100h |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Config Option Byte 1 Reg |  |  |  | Cden | 102h |
| --- | --- | --- | --- | --- | --- |

| Config Option Byte 2 Reg |  | Reserved |  |  | 103h |
| --- | --- | --- | --- | --- | --- |

| Config Option Byte 3 Reg |  |  |  | BL | 104h |
| --- | --- | --- | --- | --- | --- |

Figure 6. Additional I/O Registers.

Device ID and OB1 are standard EXM registers for device identification and configuration.  The EXM-23 responds to accesses to these ports only if EXM expansion interface line -EXMID is asserted.  An 8-bit read from I/O address 100h returns the device ID value EE.  A read/write configuration register appears at I/O address 102h.

CDEN specifies whether the EXM is disabled or enabled.  Writing a 0 to this bit clears the flat panel display.

BL (Backlight) at address 104h specifies whether the backlight is on or off. This register is not accessible from the EXM Setup Screen and is not saved in CMOS RAM. A debug example is provided below:

debug <return>

-o 96 slot number <return>	(insert the number of the slot the card resides in)

-o 104 FF <return>		(this turns the light off)

-o 104 FE<return>		(this turns the light on)

The backlight defaults to "on" with a hardware reset.