# 4. Connectors

Because flat panels vary greatly from product to product, standard cables are not possible. Instead, the connector part numbers and pin-outs are included here.

## Mini-D-50 Connector

The female mini D-50 connector (3M 10250-52B2VE or equivalent) mates with a male 3M 10150-6000EC or equivalent. The pin-out is below.

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | F+5V | 26 | GND |
| 2 | F+12V | 27 | PD8 |
| 3 | +VADJ | 28 | PD9 |
| 4 | -VADJ | 29 | GND |
| 5 | ~LITEON | 30 | PD10 |
| 6 | GND | 31 | PD11 |
| 7 | M | 32 | GND |
| 8 | FLM | 33 | PD12 |
| 9 | GND | 34 | PD13 |
| 10 | LP | 35 | GND |
| 11 | ~SCLK | 36 | PD14 |
| 12 | GND | 37 | PD15 |
| 13 | SCLK | 38 | GND |
| 14 | GND | 39 | PD16 |
| 15 | PD0 | 40 | PD17 |
| 16 | PD1 | 41 | GND |
| 17 | GND | 42 | RSVD(PD18) |
| 18 | PD2 | 43 | RSVD(PD19) |
| 19 | PD3 | 44 | GND |
| 20 | GND | 45 | RSVD(PD20) |
| 21 | PD4 | 46 | RSVD(PD21) |
| 22 | PD5 | 47 | GND |
| 23 | GND | 48 | RSVD(PD22) |
| 24 | PD6 | 49 | RSVD(PD23) |
| 25 | PD7 | 50 | GND |

Table 9. EXM-23 Mini-D-50 Pin-out.

Notes to Table 9:

1.    F+5V is a fused version of +5V from the backplane.

2.    F+12V is a fused version of +12V from the backplane.

3.    +VADJ and -VADJ contrast adjustment accessed at the panel trim pot.

4.    ~LITEON is used to turn the backlight off via register 104h (0 on, 1 off).

5.    FLM is the flat panel equivalent of VSYNC.

6.    SCLK (Shift Clk) is the pixel clock for flat panel data.

7.    ~SCLK (Inverted Shift Clk) is the pixel clock normally for EL panels.

8.    PD0 - PD17 are the eighteen bits of flat panel data output.

9.    RSVD (PD18 - PD23) are reserved pins for future expansion.

## For more information, refer to Appendix A.

## Trimpot

Since adjusting the power serves to adjust the contrast on the flat panel, contrast adjustment is provided with a nominal 50K ohm 20-turn trimpot which adjusts the +VADJ and -VADJ in parallel; for example, when the +VADJ is at +15V, the -VADJ terminal is at -15V. The trimpot is depicted in Figure 1 on page 3.

## Fuse

All power from the backplane to the front panel is fused with a socketed 2A Schurter SMD fuse (part number 3402.0012.2).

Figure 7. EXM-23 Fuse Location.

NOTES