Appendix A:  
Video Modes and Supported Panels

The EXM-23 will support color 640 x 400 and color 640 x 480 panels from:  Epson (LCD), Fujitsu (plasma), Hitachi (LCD, Color LCD*), Kyocera (LCD*), Optrex (LCD), Panasonic (Plasma), Planar (EL), Sanyo (LCD), Sharp (LCD, Color TFT, EL* some), Toshiba (LCD).

The EXM-23 will support monochrome panels from: Epson (LCD), Fujitsu (plasma), Hitachi (LCD), Kyocera (LCD*), Optrex (LCD), Panasonic (Plasma), Planar (EL), Sanyo (LCD), Sharp (LCD, EL*some), Toshiba (LCD).

* External circuitry required

Interface tables for the EXM-23 and the following panels are included here:

Sharp Monochrome LCD (BIOS setting for Monochrome - default)

640 x 480 	LM64P80

Sharp Passive Color LCD  (BIOS setting for Passive Color)

640 x 480 	LM64C08P

Sharp 512 Color TFT (BIOS setting for Active Color)

640 x 480 	LQ10DH11

Optrex Monochrome LCD (BIOS setting for Electroluminescence)

640 x 200 	DMF651AN

Optrex Monochrome LCD (Contact RadiSys for more information about this panel)

640 x 200 	DMF651AN

Seiko Monochrome LCD (Contact RadiSys for more information about this panel)

320 x 240 	G321EX5R000

Sharp Monochrome LCD (Monochrome BIOS - default)

640 x 480 	LM64P80

| LM64P80 | Signal Name | EXM23 | Signal Name |
| --- | --- | --- | --- |
| 1 | S | 8 | FLM |
| 2 | CP1 | 10 | LP |
| 3 | CP2 | 13 | SCLK |
| 4 | DISP | 6 | GND |
| 5 | VDD | 1 | F+5V |
| 6 | VSS | 9 | GND |
| 7 | VEE | 4 | -VADJ |
| 8 | DU0 | 19 | PD3 |
| 9 | DU1 | 18 | PD2 |
| 10 | DU2 | 16 | PD1 |
| 11 | DU3 | 15 | PD0 |
| 12 | DL0 | 25 | PD7 |
| 13 | DL1 | 24 | PD6 |
| 14 | DL2 | 22 | PD5 |
| 15 | DL3 | 21 | PD4 |

Sharp Passive Color LCD (Use Passive Color BIOS)

640 x 480 	LM64C08P

| LM64C08P | Signal Name | EXM23 | Signal Name |
| --- | --- | --- | --- |
| CN1 |  |  |  |
| 1 | YD | 8 | FLM |
| 2 | LP | 10 | LP |
| 3 | XCK | 13 | SCLK |
| 4 | DISP | 1 | F+5V |
| 5 | VDD | 1 | F+5V |
| 6 | VSS | 6 | GND |
| 7 | VEE | 3 | +VADJ |
| 8 | DU0 | 31 | PD11 |
| 9 | DU1 | 30 | PD10 |
| 10 | DU2 | 28 | PD9 |
| 11 | DU3 | 27 | PD8 |
| 12 | DU4 | 19 | PD3 |
| 13 | DU5 | 18 | PD2 |
| 14 | DU6 | 16 | PD1 |
| 15 | DU7 | 15 | PD0 |
| CN2 |  |  |  |
| 16 | VSS | 9 | GND |
| 17 | DL0 | 37 | PD15 |
| 18 | DL1 | 36 | PD14 |
| 19 | DL2 | 34 | PD13 |
| 20 | DL3 | 33 | PD12 |
| 21 | DL4 | 25 | PD7 |
| 22 | DL5 | 24 | PD6 |
| 23 | DL6 | 22 | PD5 |
| 24 | DL7 | 21 | PD4 |
| 25 | VSS | 12 | GND |

Sharp 512 Color TFT (Use Active Color BIOS)

640 x 480 	LQ10DH11

| LQ10DH11 | Signal Name | EXM23 | Signal Name |
| --- | --- | --- | --- |
| 1 | GND | 6 | GND |
| 2 | CK | 13 | SCLK |
| 3 | R1 | 34 | PD13 |
| 4 | R0 | 33 | PD12 |
| 5 | GND | 6 | GND |
| 6 | R2 | 36 | PD14 |
| 7 | G1 | 27 | PD8 |
| 8 | G0 | 25 | PD7 |
| 9 | GND | 9 | GND |
| 10 | G2 | 28 | PD9 |
| 11 | B1 | 19 | PD3 |
| 12 | B0 | 18 | PD2 |
| 13 | GND | 12 | GND |
| 14 | B2 | 21 | PD4 |
| 15 | GND | 14 | GND |
| 16 | HSYNC | 10 | LP |
| 17 | VCC | 1 | F+5V |
| 18 | VSYNC | 8 | FLM |
| 19 | VDD | 2 | F+12V |
| 20 | GND | 17 | GND |
| 21 | NC |  |  |
| 22 | NC |  |  |

Sharp Grey Scale EL (Use Electroluminescence BIOS)

640 x 400 	LJ640U34

| LJ640U34 | Signal Name | EXM23 | Signal Name |
| --- | --- | --- | --- |
| A1 | D0 | 15 | PD0 |
| B2 | D1 | 16 | PD1 |
| A2 | CKD | 11 | ~SCLK |
| B2 | GND | 6 | GND |
| A3 | HORIZ. SYNC | 10 | LP |
| B3 | GND | 9 | GND |
| A4 | VERT SYNC | 8 | FLM |
| B4 | NC |  |  |
| A5 | GND | 12 | GND |
| B5 | GND | 14 | GND |
| A6 | NC |  |  |
| A6 | NC |  |  |
| A7 | VL (+5V) | 1 | F+5V |
| B7 | VL (+5V) | 1 | F+5V |
| A8 | VD (+12V) | 2 | F+12V |
| B8 | VD (+12V) | 2 | F+12V |

Optrex Monochrome LCD (Contact RadiSys for more information)

640 x 200 	DMF651AN

| DMF651AN | Signal Name | EXM23 | Signal Name |
| --- | --- | --- | --- |
| 1 | FLM | 8 | FLM |
| 2 | LP | 10 | LP |
| 3 | CP | 13 | SCLK |
| 4 | M | 7 | M |
| 5 | VADJ | 4 | -VADJ |
| 6 | VCC | 1 | F+5V |
| 7 | VSS | 6 | VSS |
| 8 | VEE | 4 | -VADJ |
| 9 | D0 | 15 | PD0 |
| 10 | D1 | 16 | PD1 |
| 11 | D2 | 18 | PD2 |
| 12 | D3 | 19 | PD3 |

Seiko Monochrome LCD (Contact RadiSys for more information)

320 x 240 	G321EX5R000

| G321EX5R000 | Signal Name | EXM23 | Signal Name |
| --- | --- | --- | --- |
| 1 | FLM | 8 | FLM |
| 2 | M | 7 | M |
| 3 | CL1 | 10 | LP |
| 4 | CL2 | 13 | SCLK |
| 5 | D OFF | 6 | GND |
| 6 | D0 | 15 | PD0 |
| 7 | D1 | 16 | PD1 |
| 8 | D2 | 18 | PD2 |
| 9 | D3 | 19 | PD3 |
| 10 | VDD | 1 | F+5V |
| 11 | VSS | 9 | GND |
| 12 | VLC | 4 | -VADJ |
| 13 | VO | * |  |
| 14 | FG | 12 | GND |

* -VADJ------------GND

VO