# 1.  Product Description

This manual contains the information required to install the EXM-24 intelligent serial communications module.

The EXM-24 is an RS-232 eight-port intelligent serial communications board designed to be 100% compatible with DigiBoard PC/8i software and accessories. The eight serial ports are accessed via a 78-pin connector off the front panel of the EXM-24, typically intended for mating with an 'octacable' which splits each port out into an individual connector.

An on-board 80C186 16MHz microprocessor off-loads much of the serial transmission management from the host CPU, hence the designation 'intelligent'. Communication between the host CPU and the on-board processor is via on-board dual-ported SRAM. The 80C186 also has access to private on-board SRAM. The private SRAM is 128 KBytes. The dual-ported SRAM is 256 KBytes.

The EXM-24 ships with an eight-port "octacable" adapter. The software interface to the EXM-24 is via drivers supplied by DigiBoard to RadiSys under a license agreement. These drivers are available for DOS and Windows. For drivers and software using a different operating system, contact DigiBoard directly at (612) 943-9020. Their DigiLine BBS phone number is (612) 943-0812 (up to 9600 baud, 8,N,1)

Multiple EXM-24 boards may operate within the same system, provided each board's I/O address and any IRQ used are unique. Memory addresses across boards need not be unique, as memory access to a board can be enabled/disabled via software.

Port Performance

The EXM-24 supports data transfer rates up to 115.2K baud. The number of ports operating at any given rate depends on multiple factors, i.e. the individual port baud rates, the synchronization of the data transfers among ports, etc. Typically, 1 to 2 ports may be run at 115.2K baud, or 1 to 3 at 57.6K baud, or all eight at 38.4K baud or less.

Serial communications are controlled by 85C30 SCCs and associated line drivers/receivers. Typical output voltage swings are -6.5 volts to +7.3 volts. Outputs are short-circuit protected from -30 volts to +30 volts, and RS-232 voltage levels (+/-30 volts) are accepted at the inputs.

On-board Processor Interface

The interface to the on-board 80C186 microprocessor is via on-board dual-ported memory, and through an I/O port address.

## Specifications

Table 1 defines the environmental and electrical specifications of the EXM-24.

| Characteristic |  | Value |
| --- | --- | --- |
| Environmental |  |  |
| Temperature | operating | 0° to 60° C (* see below) |
|  | storage | -40° to 85° C |
| Humidity | operating | 5% - 95% (non-condensing) |
|  | storage | 5% - 95% (non-condensing) |
| Vibration | operating | .015"displacement with 2.5 Gs peak (max) acceleration over 5 to 2000 Hz |
|  | storage | .030" displacement with 5 Gs peak (max) acceleration over 5 to 2000 Hz |
| Shock | operating | 30g 11 msec duration, half-sine shock pulse |
|  | storage | 50g 11 msec duration, half-sine shock pulse |
| Electrical |  |  |
| Current | maximum | 5V @ 0.7A |
|  | typical | 5V @ 0.4A |

Table 1. EXM-24 Environmental and Electrical Specifications.

* Upper temperature limit degrades 2° C per 1000 ft. elevation.  Maximum  
   elevation 10,000 ft.