# 2.  Installation

Before installing the EXM-24, unpack and inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-24, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Insertion into an EXM Carrier

Insertion of the EXM-24 into an EXM carrier is straightforward.  Remove a blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the 
EXM-24 into the card guides.  Firmly press the EXM-24 front panel to ensure that the module is properly seated in the subplane and secure it with the thumbscrews. DO NOT OVERTIGHTEN.

*	make sure that power to your system is off.  this exm is not designed to be inserted or removed from a live 
system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

*	insert it with adequate continuous force rather than tapping or hammering on it.

## Configuring the BIOS Setup

Once the EXM-24 is inserted into the carrier, turn the system on and boot it up. The EXM configuration data in the EPC to which the EXM-24 is connected now must be modified to recognize and enable the card and select from the available options.  Invoke the BIOS setup function from the command line by pressing the CTRL-ALT-ESC keys simultaneously.

### Enabling the EXM module

Once in the setup program, a menu displays specifying which function keys are available for further configuration.  Press the F2 function key to invoke the EXM menu.  The screen display resembles the figure below.

ID OB1 OB2       
 Slot 0    FF  00  00
      1    ED  01  00
      2    D7  0D  0D
      3    DE  01  00
      4    F5  05  00
      5    7D  05  00

Figure 1. EXM Setup Screen.

The EXM Setup Screen displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.  An example of configuration data for the EXM-24 is shown in bold in slot 2. The displayed data shows SLOT, ID, OB1 and OB2.  These are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See the diagram below to determine which EXM slot the EXM-24 occupies.  Note that dashed lines indicate EXM slots that may not be available on all systems.

EXM Slots

µ §

Figure 2. EXM Slot Numbering.

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

All slots not occupied by an EXM module should show an ID of FF and OB1/OB2 of   00  00  indicating that no EXM is present.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-24 should be set to D7h.

OB1 is a hexadecimal value derived by combining the following:

| >64K (bit 7) | IRQ 2-0 (bits 6-4) | I/O 2-0 (bits 3-1) | Card Enable (bit 0) |
| --- | --- | --- | --- |
| 0 - 64K 1 - >64K | 000 - none 001 - IRQ11 010 - IRQ12 011 - IRQ3 100 - IRQ4 101 - IRQ5 110 - IRQ15 111 - IRQ7 | 000 - none 001 - 100h 010 - 110h 011 - 120h 100 - 200h 101 - 220h 110 - 300h 111 - 320h | 0 - disable 1 - enable |

Figure 3. OB1 Register.

OB1 contains the card enable bit, specifies the I/O port base address, sets the IRQ, and controls the dual port memory setting. For example, a typical OB1 value is 0Dh (00001101) indicating the card is enabled, uses I/O address 300h, uses no IRQ, and acts as if only 64K of dual port memory is available to conserve memory space.

OB2 is a hexadecimal value derived by combining the following:

| A23 (bit 7) | A22 (bit 6) | A21 (bit 5) | A20 (bit 4) | A19 (bit 3) | A18 (bit 2) | A17/ >256K (bit 1) | A16/ >128K (bit 0) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x | x | x | x | x | x | 0 - 256K 1 - reserved | 0 - 128K 1 - >128K |

Figure 4. OB2 Register.

OB2 holds two bits to size the memory and memory base address lines A16 through A23. There are 24 bits available for address matching. Note that boards cannot be mapped into high memory for host systems containing 16 Megabytes or more of memory.

There are several restrictions that apply when using DOS/Windows systems. Under DOS, if using memory address A000, you cannot utilize 256 color VGA display. A memory address of D000 conflicts with the EXM-17 PCMCIA adapter. In addition, the EXM-10A Ethernet controller, the EXM-25 Token Ring controller, and the EXM-28 Arcnet controller may also conflict with D000. EMC systems can safely use E000;  VME/VXI systems cannot.

Address space memory must be placed above existing memory if it is to be placed above the 1 MByte boundary.

Note that under MS Windows, a memory size of 256K will work. Make sure you exclude this memory from Windows access using the EMMExclude= statement under the [386Enh] heading in your SYSTEM.INI file. For more information, refer to the Microsoft Windows Operating System manual.

Sizing the Memory

Use the chart below to determine the settings for your system. Note that once OB1 Bit 7 is set to zero, OB2 Bit 1 and OB2 Bit 0 are no longer used to size the memory, but are used instead to set the Base Memory Address. The same rule applies when OB2 Bit 0 is set to zero to set the system for 128K - OB2 Bit 1 is no longer used to size the memory, but is used instead to set the Base Memory Address.

| OB1 Bit 7 (64K) | OB2 Bit 0 (A16/128K) | OB2 Bit 1 (A16/256K) | Resulting Memory Size |
| --- | --- | --- | --- |
| 0 | - | - | 64K |
| 1 | 0 | - | 128K |
| 1 | 1 | 0 | 256K |
| 1 | 1 | 1 | Do not use |

Table 2. Sizing the Memory.

Setting the Base Memory Address

Register OB2 corresponds to the eight upper address bits decoded in the 24-bit address space. Address bits A15 - A0 are implied zero by default, based on a 64K boundary.

For example, if using a 64K memory size based at 0xD000:0, use the following settings:

OB1 Bit 7 = 0;  OB2 = 0D, or 00001101

For a 128K memory size based at 40000:0, use the following settings:

OB1 Bit 7 = 1;  OB2 = 40h, or 01000000

For a 256K memory size based at 80000:0, use the following settings:

OB1 Bit 7 = 1;  OB2 = 81h, or 10000001

For example, a typical OB2 value is 0Dh (00001101) indicating 64K memory size with the memory base address at 0D0000h.

### Software Support

The EXM-24 ships with DigiBoard DOS/Windows software disks and manuals under an agreement with RadiSys. Follow the installation instructions for drivers and functions found in the DigiBoard manuals that refer to the PC/8i board specifically or the PC/Xi board generally. Note that the DigiBoard software includes a substantial text file which provides detailed information about the DigiBoard drivers and functions.

For software drivers and manuals supporting operating systems other than DOS/Windows, DigiBoard can be contacted directly at the address and phone number listed in Chapter 4, Programming Interface, on page 13.