# 3. Connector

## DB-78 Connector

The EXM-24 has a single DB-78 connector on the faceplate. Grounding of the shell is performed by the jackscrews holding the face plate to the connector. The shell is grounded to chassis ground, not VSS.

Octacable (eight port) external interface cables are available from RadiSys, splitting the board connector into individual port connectors.

The pin-out is listed in Table 3.

| Pin | Signal | Pin | Signal | Pin | Signal | Pin | Signal |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | DTR8 | 21 | RTS5 | 40 | TXD5 | 60 | RTS8 |  |  |
| 2 | TXD6 | 22 | DTR5 | 41 | RTS6 | 61 | DTR7 |  |  |
| 3 | DTR6 | 23 | CD6 | 42 | DSR6 | 62 | RTS7 |  |  |
| 4 | CTS6 | 24 | RI6 | 43 | CD5 | 63 | TXD7 |  |  |
| 5 | DSR5 | 25 | CTS5 | 44 | RI5 | 64 | TXD8 |  |  |
| 6 | CD8 | 26 | DSR8 | 45 | CTS8 | 65 |  |  |  |
| 7 | RI8 | 27 | RXD8 | 46 | RXD7 | 66 |  |  |  |
| 8 | RXD6 | 28 | RXD5 | 47 | RI7 | 67 |  |  |  |
| 9 | CTS7 | 29 | DSR7 | 48 | CD7 | 68 | GND |  |  |
| 10 | TXD4 | 30 | TXD1 | 49 | DTR1 | 69 | GND |  |  |
| 11 | TXD3 | 31 | RTS2 | 50 | TXD2 | 70 | GND |  |  |
| 12 | RTS3 | 32 | DTR2 | 51 | RTS1 | 71 | GND |  |  |
| 13 | DTR3 | 33 | CD2 | 52 | DTR4 | 72 | GND |  |  |
| 14 | RTS4 | 34 | DSR2 | 53 | CTS2 | 73 | GND |  |  |
| 15 | RI2 | 35 | CD1 | 54 | DSR1 | 74 | GND |  |  |
| 16 | CTS1 | 36 | RI1 | 55 | RXD1 | 75 | GND |  |  |
| 17 | RXD2 | 37 | RXD3 | 56 | RXD4 | 76 |  |  |  |
| 18 | CD4 | 38 | DSR4 | 57 | CTS4 | 77 |  |  |  |
| 19 | RI4 | 39 | CD3 | 58 | DSR3 | 78 |  |  |  |
| 20 | RI3 |  |  | 59 | CTS3 |  |  |  |  |

Table 3. EXM-24 RS-232 DB-78 Pin-out.

NOTES