4.  Programming Interface

This chapter contains information about the EXM-24 registers. To write custom software for the EXM-24, you will also need the FEPOS Developer's Kit for the 80186, available from DigiBoard.

If you are using the EXM-24 software supplied on the distribution diskette,  or some other applications that enable access to the serial ports, you can ignore this chapter.

## Standard EXM Registers

The Device ID and Configuration Option Byte registers are described below:

|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 | I/O port |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Device ID Reg | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Config Option Byte 1 Reg | >64K | IRQ2 | IRQ1 | IRQ0 | I/O2 | I/O1 | I/O0 | Cden | 102 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Config Option Byte 2 Reg | A23 | A22 | A21 | A20 | A19 | A18 | A17 | A16 | 103 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 5.  Standard EXM Registers.

These are standard EXM registers for device identification and configuration.  The EXM-24 responds to accesses to these ports only if EXM expansion interface line 
-EXMID is asserted (by writing the slot number to I/O address 96h).  An 8-bit read from I/O address 100h returns the device ID value D7h.  Read/write configuration registers appear at I/O address 102h and 103h.

I/O addresses below 100h should not be used, due to the potential for conflict with other devices.

OB2 controls the memory start address, lines A16 through A23, depending on memory size.

CDEN specifies whether the EXM is disabled or enabled.  If disabled, the EXM does not respond to any I/O or memory addresses and does not assert an interrupt output; it only responds to reads from I/O port 100h and reads from and writes to I/O port 102h and 103h, and then only if EXM expansion interface line -EXMID is asserted.

### I/O Space

The EXM-24 resides within the first 1024 bytes (1 Kbyte) of I/O space, specified by address lines A0 - A9.

### Buffering

There is no DMA mode for the EXM-24.

## On-board Processor Interface

Interface to the on-board 80C186 microprocessor is via on-board dual-ported memory, and through an I/O port address.

## Software Compatibility

The EXM-24 is compatible with all DigiBoard drivers. Applicable drivers are provided with the hardware for installation on the host computer.

The core of the DigiBoard driver software is the FEPOS (Front End Processor Operating System). The EXM-24 has no BIOS of its own. The FEPOS performs this function.

Different limitations are placed on the FEPOS by different operating systems. Some of the major restrictions are listed below:

·	Under the 1MB boundary: Under the 1MB boundary, only 128k may be used. Due to driver and other considerations, only one address range is possibly available for the full 128K, between C0000h and DFFFFh (depending on video mode used, if any), even though the EXM-24 can be configured for other addresses.  If VGA video is present, only 64K can be used.

For systems that cannot fit the entire 128K of memory into the address space, there is a configuration bit that directs the EXM-24 to act as if only 64k were installed. This also potentially opens the A0000h to AFFFFh region (again, depending on video usage).

For applications with multiple boards, each may be configured in the same memory space. Individual boards may have their memory alternately enabled or disabled. Under DOS, the EXM-24 on-board dual-port memory must be addressed under the 1MB boundary, and FEPOS features are accessed via Interrupt 14.

·	16 Bit Device: Because the EXM-24 is a 16-bit device, care must be taken when using it in the same 128K block as an 8-bit device. The EXM-24 dual port memory must be disabled (via the I/O port) before accessing the 8-bit device.

For a higher level of software support, third party software is available from at least one vendor, GreenLeaf Software, headquartered in Dallas, Texas. This is support software designed to run on the host CPU and interface with the DigiBoard FEPOS. Since the EXM-24 is DigiBoard compatible, GreenLeaf Software will work with the EXM-24.

DigiBoard also has separate products they provide at an additional charge to customers that wish to write their own drivers. These are the Drivers Writer's Guide and the FEPOS Tool Kit.  DigiBoard can be contacted using the following information:

DigiBoard

6400 Flying Cloud Drive

Eden Prairie, MN   55344

(612) 943-9020

BBS: (612) 943-0812 (8,N,1; up to 9600 baud)

NOTES