EXM-25 Hardware Design Specification

4/16 Mbps Token Ring Adapter

Bohdan Tashchuk

RadiSys Corporation

# Introduction

The EXM-25 is a Token Ring interface adapter card. It is fully compatible with ANSI/IEEE Std. 802.5-1992 and International Standard ISO/IEC 8802-5:1992.

The adapter uses the National Semiconductor DP8025 TROPIC chip. This IC is an almost complete single-chip Token Ring controller. It is manufactured by IBM and distributed by National. The chip is IBM 802.5 certified. It supports both 4 Mbit and 16 Mbit Token Rings, and is fully compatible with all IBM software.

The adapter has 64 Kbytes of onboard static RAM. This memory is used as transmit and receive buffers and for TROPIC scratch memory. It is dual-ported to the EXMbus thru a variable sized window. The adapter also has an 8 Kbyte onboard boot ROM.

The EXM-25 has two front panel connectors. A female DB-9 connector provides STP (Shielded Twisted Pair), while an RJ45 connector provides UTP (Unshielded Twisted Pair). Only one of these two interfaces may be used at a time.

# Configuration

Configuration of the EXM-25 uses the same mechanism as all other RadiSys EXM products. The configuration I/O address space is active only when the EXM bus ~EXMID signal is asserted.

In the two Option Byte registers the default bit settings after an EXMbus reset are shown in bold. However, most of these defaults don't matter since the system BIOS will initialize these two registers from the CMOS configuration memory.

TROPIC reads most of its configuration information while its RESET input is asserted. For this reason an EXMbus write to either Option Byte register will cause RESET to be asserted to the TROPIC for approximately 73 milliseconds. This ensures that any writes to the registers are immediately seen by the TROPIC chip.

## 100h: Device ID Register

On a read from I/O address 100h the EXM-25 returns its assigned device ID value of D6h. Writes to this address are ignored.

## 102h: Option Byte 1 Register

This read/write register, together with the Option Byte 2 register, configure the adapter. A write to this register forces the TROPIC chip to be reset. The bit definitions are:

| Bit | Definition |
| --- | --- |
| 0 | This bit is standard across all EXMs. It is CDEN, the CarD ENable. 0: The adapter is disabled, except for configuration registers. The RESET input to the TROPIC is asserted. This completely disconnects TROPIC from the EXMbus. The adapter doesn't respond to I/O accesses except to the configuration registers. The adapter BIOS ROM is removed from the EXMbus address space. The adapter tristates all IRQ signals. 1: The RESET input to the TROPIC is negated approximately 73 milliseconds after this bit is set. This allows the adapter to function normally. WARNING: the negation delay means that the adapter BIOS ROM will not be visible to the system BIOS configuration routines immediately after this bit is set. This may cause the system BIOS to not detect the adapter BIOS. |
| 1 | Read-only status bit. 0: TROPIC is operating. 1: The RESET input to the TROPIC is asserted. There are two ways this can happen. Either CDEN is 0 (see previous bit definition), or one of the two Option Byte registers was written less than 73 milliseconds ago. |
| 2 | 0: The adapter is configured for 8-bit EXMbus operation. 1: The adapter is configured for 16-bit EXMbus operation. |
| 3 | 0: This is the primary Token Ring adapter. It responds to I/O addresses a20h thru a23h. Note that address bits SA11 and SA10 are ignored by many I/O devices, so these addresses may be considered to be aliased at 220h thru 223h. 1: This is the secondary Token Ring adapter. It responds to I/O addresses a24h thru a27h (may be aliased at 224h thru 227h). In addition to the above I/O addresses, the TROPIC also responds to I/O writes to addresses 2f2h, 2f3h, 2f6h, and 2f7h (depending on IRQ level selected). These are used by IBM drivers to control interrupt circuitry. |
| 5:4 | Shared RAM page size: 00: 8 KBytes 01: 16 KBytes 10: 32 KBytes 11: 64 KBytes The adapter has 64 Kbytes of onboard RAM. These bits control the size of the EXMbus window into this shared RAM. The base address of this RAM is not controlled via Option Register bits. It is programmable via driver writes to locations in the ACA (see adapter BIOS ROM base address definition following). |
| 6 | 0: The Token Ring data rate is 4 Mbps. 1: The Token Ring data rate is 16 Mbps. |
| 7 | undefined |

## 103h: Option Byte 2 Register

This read/write register, together with the Option Byte 1 register, configure the adapter. A write to this register forces the TROPIC chip to be reset. The bit definitions are:

| Bit | Definition |
| --- | --- |
| 1:0 | IRQ level used by the TROPIC to communicate with its driver: 00: IRQ9 (nee IRQ2) 01: IRQ3 10: IRQ6 WARNING: this level will conflict with a floppy disk controller. 11: IRQ7 The TROPIC responds to I/O writes to addresses 2f2h, 2f3h, 2f6h, and 2f7h. These are used by IBM drivers to control interrupt circuitry for IRQ9, IRQ3, IRQ6, and IRQ7 respectively |
| 6:2 | Bits 17 thru 13 of the adapter BIOS ROM base address. The ROM must be in the lowest megabyte of memory. Address bits 19 and 18 are forced to 11. The ROM occupies 8 Kbytes. The top 512 bytes are the Attachment Control Area (ACA), which isn't simply ROM but has a much more complex definition. WARNING: many of these settings will conflict with other devices on the EXMbus. 00000 thru 00011: C0000h thru C6000h WARNING: will probably conflict with VGA BIOS 00100: C8000h 00101: CA000h 00110: CC000h 00111: CE000h 01000: D0000h 01001: D2000h 01010: D4000h 01011: D6000h 01100: D8000h 01101: DA000h 01110: DC000h 01111: DE000h 10000 thru 10111: E0000h thru EE000h WARNING: may conflict with VMEbus window 11000 thru 11111: F0000h thru FE000h WARNING: will probably conflict with system BIOS |
| 7 | undefined |