# 1.  Product Description

This manual contains the information you need to install and use the EXM-25 expansion module.

The EXM-25 is a Token Ring interface adapter card. It is designed to be compatible with ANSI/IEEE standard 802.5-1992 and with International Standard ISO/IEC 8802-5:1992.

The EXM-25 uses the National Semiconductor DP8025 TROPIC Token Ring controller chip. The chip is manufactured by IBM and distributed by National, and is IBM 802.5 certified. It supports both 4 Mbit and 16 Mbit Token Ring configurations, and is fully compatible with all IBM Token Ring software.

The adapter has 64 Kbytes of onboard static RAM. This memory is used as transmit and receive buffers and for TROPIC scratch memory. It is dual-ported to the EXM expansion interface through a variable-sized window. The adapter also has an 
8 Kbyte onboard boot ROM.

The EXM-25 has two front panel connectors. A female DB-9 connector provides a shielded twisted pair (STP) connection. An RJ-45 connector provides an unshielded twisted pair (UTP) connection. Only one of these two interfaces may be used at a time.

Diagnostics software is shipped on a disk with this product, but no Token Ring software drivers are supplied with the EXM-25. However, the EXM-25 is 100% compatible with the IBM Token Ring adapters for the ISA bus. Thus, any of the widely available Token Ring software packages compatible with the IBM adapters may be used with the EXM-25.

## Specifications

The following table defines the environmental and electrical specifications of the 
EXM-25.

| Characteristic |  | Value |
| --- | --- | --- |
| Environmental Specifications |  |  |
| Temperature | operating | 0°C to 60°C ambient |
|  | storage | -40°C to 85°C |
| Humidity | operating | 5% to 95% non condensing |
|  | storage | 5% to 95% non condensing |
| Altitude | operating | 10,000 ft (3,000 m) |
|  | storage | 50,000 ft (15,000 m) |
| Vibration | operating | 0.015 inch (0.38 mm) peak to peak displacement with 2.5 g peak (max) acceleration over 5 to 2000 Hz |
|  | storage | 0.030 inch (0.76 mm) peak to peak displacement with 5.0 g peak (max) acceleration over 5 to 2000 Hz |
| Shock | operating | 30 g, 11 ms duration, half-sine shock pulse |
|  | storage | 50 g, 11 ms duration, half-sine shock pulse |
| Electrical Specifications |  |  |
| Current | typical | +5V @ 530 mA |
|  | maximum | +5V @ 600 mA |

Table 1. EXM-25 Environmental and Electrical Specifications.