# 2.  Installation

µ §

Before installing your EXM-25, you should unpack and inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-25, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Insertion in an EXM Carrier

Insertion of the EXM-25 into an EXM carrier is straightforward.  Remove any blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the 
EXM-25 into the card guides.  Firmly press the EXM-25 front panel to ensure that it has mated with the rear connector, and secure it with the thumbscrews. DO NOT OVERTIGHTEN.

*	make sure that power to your system is off.  the exm is not designed to be inserted or removed from a live system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

## Configuration Using BIOS Setup Screen

The EXM configuration data in the EPC to which the EXM-25 is connected needs to be modified to recognize and enable the card and select from the available options.  Invoke the BIOS setup function by pressing the CTRL-ALT-ESC keys simultaneously.

### Enabling the EXM module

Once in the setup program, a menu displays specifying which function keys are available for further configuration.  Press the F2 function key to invoke the EXM menu.  The screen display resembles Figure 1 below.

ID OB1 OB2       
 Slot 0    FF  00  00
      1    F5  05  00
      2    D6  55  20
      3    DE  01  00
      4    ED  01  00
      5    7D  05  91

Figure 1. EXM Setup Screen.

The setup screen displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.  The displayed data shows SLOT, ID, OB1 and OB2 for each installed EXM.

All slots not occupied by an EXM module should show an ID of FF and OB1/OB2 of   00  00  indicating that no EXM is present. A typical value for the EXM-25 is shown in bold letters in slot 2.

Slot, OB1 and OB2 are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See the diagram below to determine which EXM slot the EXM-25 occupies.  Note that dashed lines indicate EXM slots that may not be available on all systems.

EXM Slots

µ §

Figure 2. EXM Slot Numbering.

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-25 is D6h.

OB1 and OB2 are two bytes of option information.  OB1 is a hexadecimal value derived by combining the following:

| Reserved (bit 7) | Data Rate (bit 6) | Shared RAM page size (bits 5 - 4) | Adapter Select (bit 3) | Data Width (bit 2) | Read-only status bit (bit 1) | Card Enable (bit 0) |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 - 4 Mbps 1 - 16 Mbps | 00 - 8 Kbytes 01 - 16 Kbytes 10 - 32 Kbytes 11 - 64 Kbytes | 0 - Primary 1 - Secondary | 0 - 8-bit 1 - 16-bit | 0 - TROPIC operating 1 - TROPIC reset | 0 - disable 1 - enable |

Table 2. OB1 Configuration Bit Definition.

If the Card Enable bit (bit 0) is disabled, the RESET input to the TROPIC is asserted. This completely disconnects the TROPIC from the EXM expansion interface. When enabled, the RESET input to the TROPIC is negated approximately 18 milliseconds after this bit is set.

The read-only status bit (bit 1) is set to 1 (TROPIC reset) if the card enable bit is set to 0 or either of the two OB registers was written less than 18 milliseconds earlier.

Bit 2 configures the EXM-25 as either an 8-bit or a 16-bit card. Older versions of the EPC BIOSes do not work in 16-bit mode. The BIOSes that are EXM-25-compatible are the following:

EPC		BIOS Level

EPC-3 		2.17.01

EPC-4 		2.17.03

EPC 21/22 	3.05.02

All others require 8-bit configuration. Note that mixing 8-bit and 16-bit EXM cards in the same system can result in configuration errors.

Bit 3 determines if the EXM-25 is the primary or secondary Token Ring adapter. When the primary adapter, the module responds to I/O addresses A20h through A23h. Since EXM expansion interface address bits SA11 and SA10 are ignored by many I/O devices, these addresses may be aliased at 220h through 223h. When the secondary adapter, the EXM-25 responds to I/O addresses A24h through A27h, which may be aliased at 224h through 227h.

When using one card in a system, it must be setup as the primary adapter. When using two cards in one system, one card must be set up as the primary adapter and one card must be set up as the secondary adapter. Each card requires separate ROM areas.

Bits 4 and 5 are used to set the shared RAM page size. Due to the implementation of the standard Token Ring device driver, 16K is the preferred setting. Regardless of the size selected, the address must be on a 16K boundary - even if it is only 8K.

For instance, a typical OB1 value is 55h (01010101), representing card enabled, TROPIC operating, 16-bit mode, primary adapter, 16KBytes shared RAM page size, and 16 Mbps data rate.

OB2 is a hexadecimal value derived by combining the following:

| Reserved (bit 7) | Adapter BIOS ROM Base Address (bits 6 - 2) | IRQ (bits 1- 0) |
| --- | --- | --- |
| 0 | Valid settings are: 001-00: C8000h 001-01: CA000h 001-10: CC000h 001-11: CE000h 010-00: D0000h 010-01: D2000h 010-10: D4000h 010-11: D6000h 011-00: D8000h 011-01: DA000h 011-10: DC000h 011-11: DE000h 100-00: E0000h 100-01: E2000h 100-10: E4000h 100-11: E6000h 101-00: E8000h 101-01: EA000h 101-10: EC000h 101-11: EE000h | 00 - IRQ9(2) 01 - IRQ3 10 - IRQ6 11 - IRQ7 |

Table 3. OB2 Configuration Bit Definition.

Use of any value not listed WILL cause this card to fail and MAY cause the system to fail as well.

In VME or VXI systems, do not use base address E000h-EE000h. This address WILL conflict with access to the VMEbus.

Do not use the EXM-25 in slot 6 with an EPC-23 (if using BIOS rev. 3.05.00 or earlier).

Note that when using the 64Kbytes RAM option, the address must be set on the page boundary, such as D0000h or E0000h.  For example, when using the Novell NET.CFG file, insert the line "MEM #2 D0000" in the link file options.

The IRQ level defined in bits 1 - 0 is used by the TROPIC chip to communicate with its driver. Make sure that the IRQ chosen for the EXM-25 does not conflict with any other modules that use an IRQ.  Some common IRQs to avoid are the following:

IRQ3		if using COM2

IRQ6		if using floppy disk controller

IRQ7		if using LPT1

The TROPIC responds to I/O writes to address 2F2h, 2F3h, 2F6h, and 2F7h. These are used by IBM drivers to control interrupt circuitry for IRQ9(2), IRQ3, IRQ6, and IRQ7, respectively. For instance, a typical OB2 value is 20h (00100000), representing adapter BIOS ROM base address D0000h and IRQ9(2).

The OB registers are not used to configure the shared RAM address. This is done by the software drivers.