# 3. Connectors

## DB-9 Connector

The female DB-9 connector provides shielded twisted pair (STP) connection. The pin-out is provided below:

| Pin | Signal |
| --- | --- |
| 1 | RX+ (MIC R) |
| 2 | Ground |
| 3 | +5V |
| 4 | Ground |
| 5 | TX- (MIC B) |
| 6 | RX- (MIC G) |
| 7 | Ground |
| 8 | Ground |
| 9 | TX+ (MIC O) |

Table 4. DB-9 Pin-out.

The DB-9 connector is the boundary of the physical layer (PHY) of the Token Ring adapter. This connector is not part of the ISO standard, which defines the interface at the Medium Interface Connector (MIC) rather than at the PHY. A PHY/MIC cable (not supplied) must be used to connect the PHY and the MIC. This cable is readily available because this is the method used by IBM Token Ring adapters to connect to the MIC.

The fused +5V and GROUND signals may be used to supply power to a media adapter (not supplied) that plugs directly into the DB-9 connector. These signals are not used by the MIC and are not part of the ISO standard. They are provided for IBM compatablility.

The DB-9 connector is metal. The outside of the connector is a SHIELD. It connects to the outside SHIELD of the PHY/MIC cable.

## RJ-45 Connector

The RJ-45 connector provides an unshielded twisted pair (UTP) connection. The pin-out is  defined in the following table:

µ §

| RJ45 | Signal |
| --- | --- |
| 1 | NC |
| 2 | NC |
| 3 | TX- |
| 4 | RX+ |
| 5 | RX- |
| 6 | TX+ |
| 7 | NC |
| 8 | NC |

Table 5. RJ-45  Connector Pin-out.

## Termination

Note that termination of Token Ring networks is not an issue. Termination is automatic for Token Ring because cards cannot be daisy-chained.

## Fuse

The +5V fuse on the EXM-25 is not user-replaceable.