# 4.  Programming Interface

This chapter contains EXM configuration registers information and technical references needed to write custom software drivers for the EXM-25. Anyone using standard Token-Ring drivers can skip reading this information.

The EXM-25 defines the following registers in the I/O space:

|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 | I/O port |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Device ID Reg | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Config Option Byte 1 Reg | 0 | Data Rate | RAM Page Size | adapter select | Data Width | Read Only | Cden | 102 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Config Option Byte 2 Reg | Res | Adapter BIOS ROM base address | IRQ Select | 103 |
| --- | --- | --- | --- | --- |

Configuration Option Byte 2 bits 6-2 (Adapter BIOS ROM base address) and bits 
1-0 (IRQ Select) must be set to match your Token Ring and device driver parameters. Consult the manual associated with the particular operating system or application program for these details.

IBM publishes extensive documentation regarding Token Ring hardware and software.

The National Semiconductor Local Area Network Databook (1993 edition) has a number of applicable datasheets and application notes that explain how to program the TROPIC chip. Please refer to the DP8025 TROPIC Chip data sheet for more detailed information.

## Diagnostics

Included with the EXM-25 is a floppy disk containing the diagnostics routine TOKNDIAG.EXE. This executable file provides a basic functionality test of the token ring hardware. TOKNDIAG.EXE can be used with or without a cable connecting the card to a token ring hub. When used with a cable, the diagnostics routine tests functionality of the network connection. When used without a cable, the diagnostics routine tests local functionality.