# 1.  Product Description

This manual contains the information required to install and use the EXM-26 multiport serial communications module.

The EXM-26 is an RS232/RS422 four or eight port non-intelligent serial communications module. The serial ports are accessed via a 78-pin connector on the front panel, typically intended for mating with a 'quadracable' (four ports) or an 'octacable' (eight ports), splitting each port out into an individual connector (typically a DB-25). The board is offered in four or eight port versions, where all the ports use the same communications protocol, i.e., RS422 or RS232. RS232 boards differ from RS422 boards only in the drivers used, and the signals available. The RS422 signal set is limited to TXD, RXD, RTS, and CTS.

Each port is 16550 Universal Asynchronous Receiver/Transmitter (UART) compatible, and is controlled independently from the others. Each port has available 16-byte transmit and receive FIFOs, to reduce CPU interrupts. Individual modem control signals exist for each port. However, to maintain compatibility with existing software, each port powers up in 16450 UART emulation mode (i.e., the FIFOs are disabled, and the device mimics a 16450).

Individual ports share an interrupt to the CPU. The available IRQ lines are IRQ9(2), IRQ3-7, IRQ11-12, or IRQ15. Any combination of ports may be masked off from generating interrupts, and only one port may interrupt the CPU at a time. A shared interrupt identification register exists to identify the source of an interrupt to the CPU. Interrupt allocation is sequential; each port has the opportunity to interrupt in turn before a second interrupt can be generated by the same port.

Serial port addressing is based on eight sequential bytes in the I/O space 100h to 3FCh. The base address of the first port is user configurable, and must be on an eight byte boundary. Additional ports may be either addressed in contiguous eight byte segments, or aliased to addresses above 3FCh sharing the same lower 10 address bits.

Multiple EXM-26 boards may be operating within the same system, provided each board's I/O space and any IRQ used are unique.

The EXM-26 supports data transfer rates up to 56K baud. The number of ports operating at any given rate depends on multiple factors, i.e., the individual port baud rates, the synchronization of the data transfers among ports, the speed of the host CPU, etc.

Serial communications are controlled by 16C550 UARTs and associated line drivers/receivers. RS-232 voltage levels (± 30 volts) are accepted at the inputs for the RS-232 boards.

## Specifications

Table 1 defines the environmental and electrical specifications of the EXM-26.

| Characteristic |  | Value |
| --- | --- | --- |
| Environmental |  |  |
| Ambient | operating | 0° to 60° C (* see below) |
| Temperature | storage | -40° to 85° C |
| Humidity | operating | 5% - 95% (non-condensing) |
|  | storage | 5% - 95% (non-condensing) |
| Vibration | operating | .015"displacement with 2.5 g peak (max) acceleration over 5 to 2000 Hz |
|  | storage | .030" displacement with 5 g peak (max) acceleration over 5 to 2000 Hz |
| Shock | operating | 30g 11 msec duration, half-sine shock pulse |
|  | storage | 50g 11 msec duration, half-sine shock pulse |
| Electrical (both RS-232 and RS-422 versions) |  |  |
| Current | maximum | 5V @ 0.45A (eight ports) 0.35A (four ports) |
|  | typical | 5V @ 0.35A (eight ports) 0.25A (four ports) |

Table 1. EXM-26 Environmental and Electrical Specifications.

* Upper temperature limit degrades 2° C per 1000 ft. elevation.  Maximum  
   elevation 10,000 ft.