# 2.  Installation

µ §

Before installing the EXM-26, unpack and inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-26, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Insertion in an EXM Carrier

Insertion of the EXM-26 into an EXM carrier is straightforward.  Remove a blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the EXM-26 into the card guides.  Firmly press the EXM-26 front panel to ensure that the module is properly seated in the subplane and secure it with the thumbscrews.

*	make sure that power to your system is off.  this exm is not designed to be inserted or removed from a live 
system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

*	insert it with adequate continuous force rather than tapping or hammering on it.

## Configuring the BIOS Setup

Once the EXM-26 is inserted into the carrier, turn the system on and boot it up. The EXM configuration data in the EPC to which the EXM-26 is connected now must be modified to recognize and enable the card and select from the available options.  Invoke the BIOS setup function from the command line by pressing the CTRL-ALT-ESC keys simultaneously.

### Enabling the EXM module

Once in the setup program, a menu displays specifying which function keys are available for further configuration.  Press the F2 function key to invoke the EXM menu.  The screen display resembles the figure below.

ID OB1 OB2       
 Slot 0    DC  00  00
      1    DB  C1  00
      2    D5  C1  E0
      3    DE  00  39
      4    F5  05  00
      5    DC  F5  00

Figure 1. EXM Setup Screen.

The EXM Setup Screen displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.  An example of configuration data for the EXM-26 is shown in bold in slot 2. The displayed data shows SLOT, ID, OB1 and OB2.  These are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See the diagram below to determine which EXM slot the EXM-26 occupies.  Note that dashed lines indicate EXM slots that may not be available on all systems.

EXM Slots

µ §

Figure 2. EXM Slot Numbering.

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

For VME/VXI, note that all slots are listed on the screen even if the actual system configuration does not have all the possible EXM slots.  All slots not occupied by an EXM module should show an ID of FF and OB1/OB2 of  00  00  indicating that no EXM is present.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-26 should be set to D5h.

OB1 is a hexadecimal value derived by combining the following:

| Bit 7 I/O BaseA9 | Bit 6 I/O BaseA8 | Bit 5 I/O BaseA7 | Bit 4 I/O BaseA6 | Bit 3 I/O BaseA5 | Bit 2 I/O BaseA4 | Bit 2 I/O BaseA3 | Bit 1 Card Enable |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x | x | x | x | x | x | x | 0 disable 1 enable |

Figure 3. OB1 Register.

OB1 contains the card enable bit and specifies the I/O port base address using I/O address bits A9-A3. For example, a typical OB1 value is C1h (11000001) indicating the card has 4 or 8 serial ports based at 300h and the card is enabled.

The I/O port base address is specified using I/O address bits A9 - A3. For example, to specify a base address of 300h, the following bits would be set:

µ §

The first two bits and the last three bits are assumed.  Make sure that the base I/O address chosen for the EXM-26 does not conflict with any other modules that use a base I/O address (or range). Some common base I/O address ranges to avoid are the following:

1F0-1FF		IDE disk controller

278-27F		Parallel port (LPT2)

2E8-2EF	Serial port (COM4)

2F8-2FF		Serial port (COM2)

378-37F		Parallel port (LPT1)

3E8-3EF	Serial port (COM3)

3F0-3F7		Floppy disk controller

3F8-3FF		Serial port (COM1)

OB2 is a hexadecimal value derived by combining the following:

| Bit 7 reserved | Bit 6 reserved | Bit 5 reserved | Bit 4 Share | Bit 3 IRQ | Bit 2 IRQ | Bit 1 IRQ | Bit 0 IRQ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 0 disable 1 enable | see Table 2 | see Table 2 | see Table 2 | see Table 2 |

Figure 4. OB2 Register.

OB2 holds the interrupt setting and the share bit. Bits 7-4 are set to either 1111 or 1110, depending on whether the share bit (bit 4) is enabled.

The share bit is used to conserve the lower 1 Kbyte of I/O space. When you set the share bit equal to one, only 1 port is located in the lower 1 Kbyte of I/O space and the other ports are paged out to the same offset in each succeeding page. Thus when the share bit is enabled, the system uses 8 bytes of the lower 1 Kbyte, and when disabled, the system uses 64 bytes.

Table 2 below shows the possible settings for bits 3-0 in OB2.

| Hex Value | Setting | IRQ Configuration |
| --- | --- | --- |
| 0 | 0000 | none |
| 1 | 0001 | none |
| 2 | 0010 | reserved |
| 3 | 0011 | IRQ3 |
| 4 | 0100 | IRQ4 |
| 5 | 0101 | IRQ5 |
| 6 | 0110 | IRQ6 |
| 7 | 0111 | IRQ7 |
| 8 | 1000 | reserved |
| 9 | 1001 | IRQ9(2) |
| A | 1010 | reserved |
| B | 1011 | IRQ11 |
| C | 1100 | IRQ12 |
| D | 1101 | reserved |
| E | 1110 | reserved |
| F | 1111 | IRQ15 |

Table 2. OB2 Interrupt Settings.

Make sure that the IRQ chosen for the EXM-26 does not conflict with any other modules that use an IRQ.  Some common IRQs to avoid are the following:

IRQ2		
	IRQ3		usually COM2

IRQ4		usually COM1

IRQ5		usually LPT2

IRQ6		usually floppy disk controller

IRQ7		usually LPT1

For example, a typical OB2 value is E0h (11100000) indicating the card is set for no sharing and no interrupts.