3.  Programming Interface

This chapter contains information needed to write software for the EXM-26. Use this information if you are programming your own application, or are using one of the many commercially-available software libraries, such as the one provided by Greenleaf Software Inc. (Dallas, Texas).

To use this chapter to write software for the EXM-26, some expertise in software programming is assumed.

## Theory of Operations

Each port on the EXM-26 behaves independently.  As such, these UART ports operate like COM ports except the addresses are dependent on each other, and all ports share a single interrupt to the CPU. Other than that, each port is uniquely controllable.

### I/O Space

The EXM-26 resides within the first 1024 bytes (1 Kbyte) of I/O space, specified by address lines A9 - A0. To minimize usage of this space, the EXM-26 can use address lines A12 - A10 to make eight "copies" of its share of the I/O space. The EXM-26 sees this as 64 bytes, but other boards using only address lines A9 - A0 "see" only 8 bytes. This preserves I/O space.

The I/O space areas decoded by A12 - A10 are not contiguous.

## General I/O Base vs. Port Base

The I/O base is the start address in I/O space of the first port. In non-share mode, each sequential port starts at [('base address')+ (8 X (port - 1))]. In share mode, each sequential port starts at [(A12 ® A10) + 1] = (port #) for port number and A9 - A0 for base address. Refer to Standard EXM Registers for more information.

The contiguous setup is represented by 64 bytes starting at the base address. Refer to Standard EXM Registers for more information.

Sharing is determined by eight bytes in the lower 1K of I/O space. These bytes are mapped into higher I/O space by setting A12 ® A10 equal to [port # - 1].  Refer to Standard EXM Registers for more information.

## Interrupt Control and Identification

When an interrupt is chosen via the configuration register, the shared interrupt mask/identification register is enabled. To be compatible with other boards of a similar nature, this register is addressed at the base address + 7h. This means that the UART 'Scratchpad' register (see Table 3 for a full description of the UART registers) is not available for port 0 whenever an IRQ line is configured.

### I/O base address + 7h, Write Only (Shared Interrupt Mask Register)

The masked port register is an eight-bit, write-only register at base address + 7h. 
A 1 in any bit position disables interrupt generation capability for that port. The default is all zeros.

Base Address + 7h, Write Only

| MP7 | MP6 | MP5 | MP4 | MP3 | MP2 | MP1 | MP0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

### I/O Base address + 7h, Read Only (Shared Interrupt ID Register)

The shared interrupt identification register is a read only register also at base address + 7h. When an interrupt has been generated, the lower three bits will identify the port generating the interrupt, and hold that value until the interrupt is cleared. If no interrupt is pending, data read from this register has no meaning.

Base Address + 7h, Read Only

| 0 | 0 | 0 | 0 | 0 | IP2 | IP1 | IP0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Interrupt allocation is sequential, and is implemented through a continuous looping counter on the board. When a port generates an unmasked interrupt, and the counter reaches the number for that port, the counter enables the IRQ, updates the Shared Interrupt Identification Register, and remains at that count until the interrupt is cleared. The counter then restarts with one guaranteed increment. Therefore, each port has the opportunity to interrupt in turn before a second interrupt can be generated by the same port.

## Special Note

Configuring the IRQ selection bits in the OB2 register to 'none' (0000 or 0001) disables all access to the Shared Interrupt Mask and Identification Registers, allowing the 'Scratchpad' register for port 0 to be used.

However, using one of the reserved configurations for the IRQ selection bits will enable access to the shared Interrupt Mask and Identification Register, even though no IRQ line is enabled. Access to the Port 0 'Scratchpad' register will be disabled. Masking and identification of interrupts will function as if an IRQ line were enabled.

This setup can be useful in a polling environment where an interrupt is not desired, but some central polling location is needed. Instead of polling each individual UART for data, the UARTs can be set up to generate interrupts, but no interrupt enabled on the EXM expansion interface. The Shared Interrupt Identification Register can be polled to determine when data is available in any UART.

Note that in using such a scheme, the Shared Interrupt Identification Register will always contain some value, even when an interrupt has not been generated from a UART. Thus an individual UART designated by the register should be checked for valid data before processing.

## Standard EXM Registers

The Device ID and Configuration Option Byte registers are described below:

|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 | I/O port |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Device ID Reg | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Config Option Byte 1 Reg | A9 | A8 | A7 | A6 | A5 | A4 | A3 | Cden | 102 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Config Option Byte 2 Reg | Res | Res | Res | Share | I3 | I2 | I1 | I0 | 103 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 5. Additional I/O Registers.

These are standard EXM registers for device identification and configuration.  The EXM-26 responds to accesses to these ports only if EXM expansion interface line 
-EXMID is asserted (by writing the slot number to I/O address 96h).  An 8-bit read from I/O address 100h returns the device ID value D5.  A read/write configuration register appears at I/O address 102h.

I/O addresses below 100h should not be used, due to the potential for conflict with other devices. Care must also be taken to leave enough room for all port I/O addresses under the 3FFh maximum address. For practical purposes, this means a maximum unshared base address of 3E0h for four ports, 3C0h for eight ports, or a maximum shared base address of 3F8 (four or eight ports). See below.

OB2 holds the interrupt setting and the address configuration (either unshared or shared). The interrupt choice represents the IRQ line that all ports on the board are to share. This may be further controlled via the interrupt mask register and through registers within the UARTs themselves.

A 0 in the Share bit indicates that the ports are to be unshared, contiguous eight-byte blocks from the base address. For example, if the base address were 300h, the addresses for four ports would be 300h-307h (port 1), 308h - 30Fh (port 2), 310h - 317h (port 3), and 318 - 31Fh (port 4). This covers the contiguous range 300h - 31Fh.

A 1 in the Share bit indicates that the ports are to share the eight-byte block represented by the lower 10 base address lines (A9 - A0), and be distinguished by the upper three address lines (A12, A11, and A10). For example, if the base address were 300h, the base addresses for four ports would be 300h - 307h (port 1), 700h - 707h (port 2), B00h - B07h (port 3) and F00h - F07h (port 4). Since many boards don't decode address lines above A9, the purpose of this option is to allow all ports to be addressed in 'one' eight-byte I/O block in the lower 1 Kbyte of the I/O space, precluding the need to find up to 64 contiguous free bytes.

CDEN specifies whether the EXM is disabled or enabled.  If disabled, the EXM does not respond to any I/O or memory addresses and does not assert an interrupt output; it only responds to reads from I/O port 100h and reads from and writes to I/O port 102h and 103h, and then only if EXM expansion interface line -EXMID is asserted.

### Buffering

The EXM-26 powers up in "16450-compatible" mode, which does not use a FIFO buffer. By toggling a bit in the FCR register of the UART, the EXM-26 uses the FIFO buffer.

There is no DMA mode for the EXM-26.

## TL 16C554 UART

The TL 16C554 integrated asynchronous communication chip consists of four 
TL 16C550 UARTs (one for each of the four ports) plus steering logic. 
Port 1 is the first 16C550 on the TL16C554, Port 2 is the second 16550 on the chip, and so on. The four-port EXM-26 has one TL16C554;  the eight-port EXM-26 has two TL16C554 chips.

In the FIFO mode, each UART transmitter and receiver is buffered with a 16-Byte FIFO to reduce the number of interrupts to the CPU.

In the TL 16C450 mode, holding and shift registers eliminate the need for precise synchronization between the CPU and serial data. Regardless of the mode used, programmable baud rate registers allow the division of any input reference clock by 1 to 216-1 and generates an internal 16X clock.

The TL 16C554 adds or deletes standard asynchronous communication bits (stop, start, and parity) to or from the serial data stream.

The TL 16C554 UART chip also features independently controlled transmit, receive, line status, and data set interrupts.

Other features of the TL 16C554:

· Fully programmable serial interface characteristics:

5, 6, 7 or 8-bit characters.

even, odd, or No-parity bit

1, 1-1/2 or 2-stop bit generation

variable baud selection

· False-start bit detection

· Complete status reporting capabilities

· Line break generation and detection

· Internal diagnostic capabilities:

Loop back controls for communications link fault isolation

Break, parity, overrun, framing error simulation

· Full prioritized interrupt system controls

· Modem control functions (~CTS, ~RTS, ~DSR, ~DTR, ~RI, ~DCD)

The UART contains a programmable Baud Generator that is capable of taking any clock input from DC to 8.0 MHz and dividing it by any divisor from 2 to 216-1. The EXM-26 uses a 1.8432 MHz clock. The output frequency of the Baud Generator is 16X the baud [divisor # = (frequency input) ¸ (baud rate) X 16)]. Two 8-bit latches store the divisor in a 16-bit binary format. These Divisor Latches must be loaded during initialization to ensure proper operation of the Baud Generator. Upon loading either of the Divisor Latches, a 16-bit baud counter is immediately loaded.

Decimal divisors to use with the EXM-26 frequency of 1.8432 MHz are listed in Table 3. For baud rates of 38,400 and below, the error obtained is minimal. The accuracy of the desired baud rate is dependent on the divisor chosen. Do not use a divisor value of less than 2.

| Baud Rate Desired | Divisor (N) Used to Generate 16X Clock | Percent Error Difference Between Desired and Actual |
| --- | --- | --- |
| 50 | 2304 | - |
| 75 | 1536 | - |
| 110 | 1047 | 0.026 |
| 134.5 | 857 | 0.058 |
| 150 | 768 | - |
| 300 | 384 | - |
| 600 | 192 | - |
| 1200 | 96 | - |
| 1800 | 64 | - |
| 2000 | 58 | 0.690 |
| 2400 | 48 | - |
| 3600 | 32 | - |
| 4800 | 24 | - |
| 7200 | 16 | - |
| 9600 | 12 | - |
| 19200 | 6 | - |
| 38400 | 3 | - |
| 56000 | 2 | 2.860 |

Table 3. Baud Rate Divisor Selections.

## Description of UART Registers

Refer to the following table for a map of the UART registers. For a full description of the TL 16C554 UART, refer to the data sheet from Texas Instruments.

| Add-ress | Register Mnemonic | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | RBR read only | Data bit 7 (MSB) | Data bit 6 | Data bit 5 | Date bit 4 | Data bit 3 | Data bit 2 | Data bit 1 | Data bit 0 |
| 0 | THR write only | Data bit 7 | Data bit 6 | Data bit 5 | Data bit 4 | Data bit 3 | Data bit 2 | Data bit 1 | Data bit 0 |
| 0 | DLL | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| 1 | DLM | Bit 15 | Bit 14 | Bit 13 | Bit 12 | Bit 11 | bit 10 | Bit 9 | Bit 8 |
| 1 | IER | 0 | 0 | 0 | 0 | EDDSI Enable modem status interrupt | ERLSI Enable receiver line status interrupt | ETBEI Enable transmit holding register empty interrupt | ERBFI Enable received data avail interrupt |
| 2 | FCR write only | RCVR TriggerMSB | RCVR Trigger LSB | reservd | reservd | DMA mode select | XMIT FIFO reset | RCVR FIFO reset | FIFO enabled |
| 2 | IIR read only | FIFOs enabled | FIFOs enabled | 0 | 0 | INT ID bit 2 | INT ID bit 1 | INT ID bit 0 | 0 if INT pending |
| 3 | LCR | DLAB Divisor latch access bit | Set break | Stick parity | EPS Even parity select | PEN Parity enable | STB Number of stop bits | WLSB1 Word length select bit 1 | WLSB0 Word length select bit 0 |
| 4 | MCR | 0 | 0 | 0 | Loop | Enable external INT (or OUT2) | OUT1 | RTS Request to send | DTR Data terminal ready |
| 5 | LSR | Error in RCVR FIFO | TEMT transmit empty | THRE transmit holding register empty | BI Break Inter-rupt | FE Framing error | PE Parity Error | OE Overrun Error | DR Data Ready |
| 6 | MSR | DCD Data carrier detect | RI Ring Indicator | DSR Data Set Ready | CTS Clear to send | DDCD Delta data carrier detect | TERI Trailing edge ring indicator | DDSR Delta data set ready | DCTS Delta clear to send |
| 7 | SCR scratchpad | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |

Table 4. Serial Channel Accessible Registers.

### Receive Bit Register (RBR)

This register is used to read incoming data. The register is read-only and is only accessible if the DLAB bit (bit 7 of the LCR register) is not set.

### Transmit Holding Register (THR)

Write to this register to transmit data. The register is write-only and is only accessible if the DLAB bit (bit 7 of the LCR register) is not set.

### Divisor Latches (DLL and DLM)

Two 8-bit divisor latch registers store the divisor in a 16-bit binary format. These divisor latch registers must be loaded during initialization. Upon loading either of the divisor latches, a 16-bit baud counter is immediately loaded. This prevents long counts on initial load. The baud rate generator on the EXM-26 uses 1.8432 MHz to generate standard baud rates from 50 to 56K bps.

The DLL register is only accessible if the DLAB bit (bit 7 of the LCR register) is set.

The DLM register is only accessible if the DLAB bit (bit 7 of the LCR register) is set.

### Interrupt Enable Register (IER)

This register enables the five types of UART interrupts. Each interrupt can individually activate the interrupt (INTR) output signal. It is possible to totally disable the interrupt system by resetting bits 0 through 3 of the Interrupt Enable Register (IER). Similarly, setting bits of the IER register to a logic 1 enables the selected interrupt(s). Disabling an interrupt prevents it from being indicated as active in the IIR and from activating the INTR output signal. All other system functions operate in their normal manner, including the setting of the Line Status and MODEM Status Registers.

The IER register is only accessible if the DLAB bit (bit 7 of the LCR register) is not set.

Bit 0: This bit enables the Received Data Available Interrupt (and timeout interrupts in the FIFO mode) when set to logic 1.

Bit 1: This bit enables the Transmitter Holding Register Empty Interrupt when set to logic 1.

Bit 2: This bit enables the Receiver Line Status Interrupt when set to logic 1.

Bit 3: This bit enables the MODEM Status Interrupt when set to logic 1.

Bits 4 through 7: These four bits are always logic 0.

### FIFO Control Register (FCR)

This is a write only register at the same location as the IIR (the IIR is a read only register). This register is used to enable the FIFOs, clear the FIFOs and set the RCVR FIFO trigger level.

Bit 0: Writing a 1 to FCR0 enables both the XMIT and RCVR FIFOs. Resetting FCR0 will clear all bytes in both FIFOs. When changing from FIFO mode to 16C450 mode and vice versa, data is automatically cleared from the FIFOs. This bit must be a 1 when other FCR bits are written to or they will not be programmed.

Bit 1: Writing a 1 to FCR1 clears all bytes in the XMIT FIFO and resets its counter logic to 0. The shift register is not cleared. The 1 that is written to this bit position is self-clearing.

Bit 2: Writing a 1 to FCR2 clears all bytes in the RCVR FIFO and resets its counter logic to 0. The shift register is not cleared. The 1 that is written to this bit position is self-clearing.

Bit 3: This bit has no use on the EXM-26. Either value can be used.

Bit 4, 5: FCR4 to FCR5 are reserved for future use. Either value can be used.

Bit 6, 7: FCR6 and FCR7 are used to set the trigger level for the RCVR FIFO interrupt. When 7,6 = 00, the RCVR FIFO trigger level is 1 byte. When 7,6 = 01, the trigger level is 4 bytes. When 7,6 = 10, the trigger level is 8 bytes. When 7,6 = 11, the trigger level is 14 bytes.

### Interrupt ID Register (IIR)

In order to provide minimum software overhead during data character transfers, the UART prioritizes interrupts into four levels and records these in the Interrupt Identification Register. The four levels of interrupt conditions in order of priority are Receiver Line Status, Received Data Ready; Transmitter Holding Register Empty; and MODEM Status.

When the CPU accesses the IIR, the UART freezes all interrupts and indicates the highest priority pending interrupt to the CPU. While this CPU access is occurring, the UART records new interrupts, but does not change its current indication until the access is complete.

Bit 0: This bit can be used in a prioritized interrupt environment to indicate whether an interrupt is pending. When bit 0 is logic 0, an interrupt is pending and the IIR contents may be used as a pointer to the appropriate interrupt service routine. When bit 0 is logic 1, no interrupt is pending.

Bits 1 and 2: These two bits of the IIR are used to identify the highest priority interrupt pending.

Bit 3: In the 16450 mode this bit is 0. In the FIFO mode this bit is set along with bit 2 when a timeout interrupt is pending.

Bits 4,5: These two bits of the IIR are always logic 0.

Bits 6,7: These two bits are set when FCR0 = 1.

### Line Control Register (LCR)

The system programmer specifies the format of the asynchronous data communications exchange and sets the Divisor Latch Access bit via the Line Control Register (LCR). The programmer can also read the contents of the Line Control Register. The read capability simplifies system programming and eliminates the need for separate storage in system memory of the line characteristics.

Bits 0,1: These two bits specify the number of bits in each transmitted or received serial character. The encoding of these bits is as follows:

| Bit 1 | Bit 2 | Character Length |
| --- | --- | --- |
| 0 | 0 | 5 bits |
| 0 | 1 | 6 bits |
| 1 | 0 | 7 bits |
| 1 | 1 | 8 bits |

Bit 2: This bit specifies the number of Stop bits transmitted and received in each serial character. If bit 2 is logic 0, one Stop bit is generated in the transmitted data. If bit 2 is a logic 1 when a 5-bit word length is selected via bits 0 and 1, one and a half Stop bits are generated. If bit 2 is a logic 1 when either a 6-, 7- or 8-bit word length is selected, two Stop bits are generated. The Receiver checks the first Stop-bit only, regardless of the number of Stop bits selected.

Bit 3: This bit is the Parity Enable bit. When bit 3 is a logic 1 and bit 4 is a logic 0, an odd number of logic 1s is transmitted or checked in the data word bits and Parity bit. When bit 3 is a logic 1 and bit 4 is a logic 1, an even number of logic 1s is transmitted or checked.

Bit 4: This bit is the Even Parity Select bit. When bit 3 is a logic 1 and bit 4 is a logic 0, an odd number of logic 1s is transmitted or checked in the data word bits and parity bit. When bit 3 is a logic 1 and bit 4 is a logic 1, an even number of logic 1s is transmitted or checked.

Bit 5: This bit is the Stick Parity bit. When bits 3, 4, and 5 are logic 1, the Parity bit is transmitted and checked as a logic 0. If bits 3 and 5 are 1, and bit 4 is a logic 0, then the Parity bit is transmitted and checked as a logic 1. If bit 5 is a logic 0, Stick Parity is disabled.

Bit 6: This bit is the Break Control bit. It causes a break condition to be transmitted to the receiving UART. When it is set to a logic 1, the serial output (SOUT) is forced to the Spacing (logic 0) state. The break is disabled by setting bit 6 to a logic 0. The Break Control bit acts only on SOUT and has no effect on the transmitter logic. This feature enables the CPU to alert a terminal in a computer communications system.

Bit 7: This bit is the Divisor Latch Access Bit (DLAB). It must be set high (logic 1) to access the Divisor Latches of the Baud Generator during a Read or Write operation. It must be set low (logic 0) to access the Receiver buffer, the Transmitter Holding Register, or the Interrupt Enable Register.

### Modem Control Register (MCR)

This register controls the interface with the MODEM or data set (or a peripheral device emulating a MODEM). The contents of the MODEM Control Register are described below.

Bit 0: This bit controls the Data Terminal Ready (DTR) output. When set to 1, this bit turns on the DTR. When set to 0, this bit turns off the DTR.

Bit 1: This bit controls the Request to Send output. Bit 1 affects the RTS output in a manner identical to that described above for bit 0.

Bit 2: This bit has no effect on the EXM-26 operation except in local loopback mode.

Bit 3: This bit enables interrupts to the CPU from the port.

Bit 4: This bit provides a local loopback feature for diagnostic testing of the UART. When bit 4 is set to logic 1, the following occurs: the transmitter Serial Output (SOUT) is set to the Marking (logic 1) state; the receiver Serial Input (SIN) is disconnected; the output of the Transmitter Shift Register is "looped back" into the Receiver Shift Register Input; the four MODEM Control inputs (DSR, CTS, RI, and DCD) are disconnected; and the four MODEM Control outputs (DTR, RTS, OUT1 and OUT2) are internally connected to the four MODEM Control inputs, and the MODEM Control output pins are forced to their inactive state (high). In the diagnostic mode, data that is transmitted is immediately received. This feature allows the processor to verify the transmit-end and received-data paths of the UART.

In the diagnostic mode, the receiver and transmitter interrupts are fully operational. Their sources are external to the part. The MODEM Control interrupts are also operational, but the interrupt's sources are now the lower four bits of the MODEM Control Register instead of the four MODEM control inputs. The interrupts are still controlled by the Interrupt Enable Register.

Bits 5 through 7: These bits are permanently set to logic 0.

### Line Status Register (LSR)

This register provides status information to the CPU concerning the data transfer.

Bit 0: This bit is the receiver Data Ready (DR) indicator. Bit 0 is set to a logic 1 whenever a complete incoming character has been received and transferred into the Receiver Buffer Register or the FIFO. Bit 0 is reset to a logic 0 by reading all of the data in the Receiver Buffer Register or the FIFO.

Bit 1: This bit is the Overrun Error (OE) indicator. Bit 1 indicates that the data in the Receiver Buffer Register was not read by the CPU before the next character was transferred into the Receiver Buffer Register, thereby destroying the previous character. The OE indicator is set to a logic 1 upon detection of an overrun condition and reset whenever the CPU reads the contents of the Line Status Register. If the FIFO mode data continues to fill the FIFO beyond the trigger level, an overrun error will occur only after the FIFO is full and the next character has been completely received in the shift register.  OE is indicated to the CPU as soon as it happens. The character in the shift register is overwritten, but it is not transferred to the FIFO.

Bit 2: This bit is the Parity Error (PE) indicator. Bit 2 indicates that the received data character does not have the correct even or odd parity, as selected by the even-parity-select bit. The PE bit is set to a logic 1 upon detection of a parity error and is reset to a logic 0 whenever the CPU reads the contents of the Line Status Register. In the FIFO mode this error is associated with the particular character in the FIFO it applies to. This error is revealed to the CPU when its associated character is at the top of the FIFO.

Bit 3: This bit is the Framing Error (FE) indicator. Bit 3 indicates that the received character did not have a valid Stop bit. Bit 3 is set to a logic 1 whenever the Stop bit following the last data bit or parity bit is detected as a logic 0 bit (Spacing level). The FE indicator is reset whenever the CPU reads the contents of the Line Status Register. In the FIFO mode this error is associated with the particular character in the FIFO it applies to. This error is revealed to the CPU when its associated character is at the top of the FIFO. The UART will try to resynchronize after a framing error. To do this it assumes that the framing error was due to the next start bit, so it samples this "start" bit twice and then takes in the "data."

Bit 4: This bit is the Break Interrupt (BI) indicator. Bit 4 is set to a logic 1 whenever the receive data input is held in the Spacing (logic 0) state for longer than a full word transmission time (that is, the total time of Start bit + data bits + Parity + Stop bits). The BI indicator is reset whenever the CPU reads the contents of the Line Status Register. In the FIFO mode this error is associated with the particular character in the FIFO it applies to. This error is revealed to the CPU when its associated character is at the top of the FIFO. When a break occurs only one zero character is loaded into the FIFO. The next character transfer is enabled after SIN goes to the marking state and receives the next valid start bit.

Bit 5: This bit is the Transmitter Holding Register Empty (THRE) indicator. Bit 5 indicates that the UART is ready to accept a new character for transmission. In addition, this bit causes the UART to issue an interrupt to the CPU when the Transmit Holding Register Empty Interrupt enable is set high. The THRE bit is set to a logic 1 when a character is transferred from the Transmitter Holding Register into the Transmitter Shift Register. The bit is reset to logic 0 concurrently with the loading of the Transmitter Holding Register by the CPU. In the FIFO mode this bit is set when the XMIT FIFO is empty; it is cleared when at least 1 byte is written to the XMI T FIFO.

Bit 6: This bit is the Transmitter Empty (TEMPT) indicator. Bit 6 is set to a logic 1 whenever the Transmitter Holding Register (THR) and the Transmitter Shift Register (TSR) are both empty. It is reset to a logic 0 whenever either the THR or TSR contains a data character. In the FIFO mode this bit is set to one whenever the transmitter FIFO and shift register are both empty.

Bit 7: In the TL16C450 Mode this is a 0. In the FIFO mode LSR7 is set when there is at least one parity error, framing error or break indication in the FIFO. LSR7 is cleared when the CPU reads the LSR, if there are no subsequent errors in the FIFO. NOTE: The Line Status Register is intended for read operations only. Writing to this register is not recommended as this operation is only used for factory testing. In the FIFO mode the software must load a data byte in the Tx FIFO via Loopback Mode in order to write to LSR2-LSR4. LSR0 and LSR7 can't be written to in FIFO mode.

### Modem Status Register (MSR)

This register provides the current state of the control lines from the MODEM (or peripheral device) to the CPU. In addition to this current-state information, four bits of the MODEM Status Register provide change information. These bits are set to a logic 1 whenever a control input from the MODEM changes state. They are reset to logic 0 whenever the CPU reads the MODEM Status Register.

The contents of the MODEM Status Register are described below.

Bit 0: This bit is the Delta Clear to Send (DCTS) indicator. Bit 0 indicates that the CTS input to the chip has changed state since the last time it was read by the CPU.

Bit 1: This bit is the Delta Data Set Ready (DDSR) indicator. Bit 1 indicates that the DSR input to the chip has changed state since the last time it was read by the CPU.

Bit 2: This bit is the Trailing Edge of Ring Indicator (TERI) detector. Bit 2 indicates that the RI input to the chip has changed from a low to a high state, corresponding to an ON/OFF transition at the board RI input.

Bit 3: This bit indicates the state of the Clear to Send (CTS) input. If bit 4 (loop) of the MCR is set to a 1, this bit is equivalent to RTS in the MCR.

Bit 4: This bit indicates the state of the Clear to Send (CTS) input. If bit 4 (loop) of the MCR is set to a 1, this bit is equivalent to RTS in the MCR.

Bit 5: This bit indicates the state of the Data Set Ready (DSR) input. If bit 4 of the MCR is set to a 1, this bit is equivalent to DTR in the MCR.

Bit 6: This bit indicates the state of the Ring Indicator Input (RI). If bit 4 of the MCR is set to a 1, this bit is equivalent to OUT 1 in the MCR.

Bit 7: This bit indicates the state of the Data Carrier Detect (DCD) input. If bit 4 of the MCR is set to a 1, this bit is equivalent to OUT 2 in the MCR.

### Scratchpad Register (SCR)

This 8-bit Read/Write Register does not control the UART in any way. It is intended as a scratchpad register to be used by the programmer to hold data temporarily.

## Programming the UART

The TL 16C554 is an enhanced quad-version of the popular National Semiconductor  16550AF universal asynchronous receiver/transmitter (UART). Each channel performs serial to parallel conversion on data characters received from peripheral devices or modems and parallel to serial conversion on data characters transmitted by the CPU.

The complete status of each channel of the quad UART can be read at any time during functional operation by the CPU. The information obtained includes the type and condition of the operation being performed and any error conditions encountered.

The TL 16C554 quad UART can be placed in an alternate FIFO mode which activates the internal FIFOs, allowing 16 bytes (plus 3 bits or error data per byte in the receiver FIFO) to be stored in both receive and transmit modes. To minimize system overhead and maximize system efficiency, all logic is on the chip.

A typical initialization of a UART would take the following steps To initialize multiple ports, repeat these steps for each port.

1.	Set up the LCR register to enable the baud registers (DLL and DLM) by setting the DLAB bit equal to 1.

2.	Set the baud rate by writing the divisor value into the DLL and DLM registers according to the settings in Table 4.

3.	Clear the DLAB bit in the LCR; set up the word length, stop bit, and parity.

4.	Set up the Interrupt Registers (if desired) using the IER register and bit 3 of the MCR register.

5.	Set up any FIFOs if needed using the FCR register. You can choose to have FIFOs on some ports but not others.

6.	To avoid collecting invalid data, clear (discard) any existing data from the RBR register by performing a read.

7.	Check the LSR register for any errors. Consult Table 4 for more information.

8.	Transmit/Receive data as required by writing to the THR register and reading the RBR register.

### Example

The following is a segment of sample "C" code for initializing a port. The header and main statement are omitted. This example initializes port 2, assuming an EXM base address of 300h. The values used in the #define statements are offsets from the base.

#define BASE 0x0300;

#define RBR 0;

#define DLL 0;

#define IER 1;

#define DLM 1;

#define FCR 2;

#define IIR 2;

#define LCR 3;

#define MCR 4;

#define LSR 5;

.

.

.

int portbase = BASE + (8); /* Set port 2 base */

.

.

.

(void)outp (port_base + LCR,0x80); /*Set DLAB bit)            */

(void)outp (port_base + DLL,0x0C); /*Set 9600 baud            */

(void)outp (port_base + DLM,0x00);

(void)outp (port_base + LCR,0x03); /*Set word,stop bits, etc. */

(void)outp (port_base + IER,0x00); /*No interrupts            */

(void)outp (port_base + MCR,0x00); /*Disable interrupts       */

(void)outp (port_base + FCR,0x00); /*No FIFOs (16C450 mode)   */

(void)inp (port_base + RBR);       /*Clear Rx data register   */