# 4. Connectors

## DB-78 Connector

The EXM-26 has a single DB-78 connector on the faceplate. Grounding of the shell is performed by the jackscrews holding the face plate to the connector. The shell is grounded to chassis ground, not VSS.

ESD and EMI suppression is built in for all external signal connections.

All RS422 +/- signal pairs are terminated on-board through 235 ohms. Therefore there are no provisions for RS-485 type multi-drop capabilities.

Quadracable (four port) or octacable (eight port) external interface cables are available, splitting the board connector into individual port connectors. Whether or not a cable was shipped with your EXM-26 depends on how the product was ordered.

The pin-out for the RS-232 is listed in Table 5. The pin-out for the RS-422 is listed in Table 6.

| Pin | Signal | Pin | Signal | Pin | Signal | Pin | Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | DTR8 | 21 | RTS5 | 40 | TXD5 | 60 | RTS8 |
| 2 | TXD6 | 22 | DTR5 | 41 | RTS6 | 61 | DTR7 |
| 3 | DTR6 | 23 | CD6 | 42 | DSR6 | 62 | RTS7 |
| 4 | CTS6 | 24 | RI6 | 43 | CD5 | 63 | TXD7 |
| 5 | DSR5 | 25 | CTS5 | 44 | RI5 | 64 | TXD8 |
| 6 | CD8 | 26 | DSR8 | 45 | CTS8 | 65 |  |
| 7 | RI8 | 27 | RXD8 | 46 | RXD7 | 66 |  |
| 8 | RXD6 | 28 | RXD5 | 47 | RI7 | 67 |  |
| 9 | CTS7 | 29 | DSR7 | 48 | CD7 | 68 | GND |
| 10 | TXD4 | 30 | TXD1 | 49 | DTR1 | 69 | GND |
| 11 | TXD3 | 31 | RTS2 | 50 | TXD2 | 70 | GND |
| 12 | RTS3 | 32 | DTR2 | 51 | RTS1 | 71 | GND |
| 13 | DTR3 | 33 | CD2 | 52 | DTR4 | 72 | GND |
| 14 | RTS4 | 34 | DSR2 | 53 | CTS2 | 73 | GND |
| 15 | RI2 | 35 | CD1 | 54 | DSR1 | 74 | GND |
| 16 | CTS1 | 36 | RI1 | 55 | RXD1 | 75 | GND |
| 17 | RXD2 | 37 | RXD3 | 56 | RXD4 | 76 |  |
| 18 | CD4 | 38 | DSR4 | 57 | CTS4 | 77 |  |
| 19 | RI4 | 39 | CD3 | 58 | DSR3 | 78 |  |
| 20 | RI3 |  |  | 59 | CTS3 |  |  |

Table 5. EXM-26 RS-232 DB-78 Pin-out.

| Pin | Signal | Pin | Signal | Pin | Signal | Pin | Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | TXD8+ | 21 | CTS5+ | 40 | RTS5- | 60 | CTS8+ |
| 2 | RTS6- | 22 | TXD5+ | 41 | CTS6+ | 61 | TXD7+ |
| 3 | TXD6+ | 23 | GND | 42 | CTS6- | 62 | CTS7+ |
| 4 | RXD6+ | 24 | RXD6- | 43 | GND | 63 | RTS7- |
| 5 | CTS5- | 25 | RXD5+ | 44 | RXD5- | 64 | RTS8- |
| 6 | GND | 26 | CTS8- | 45 | RXD8+ | 65 |  |
| 7 | RXD8- | 27 | RTS8+ | 46 | RTS7+ | 66 |  |
| 8 | RTS6+ | 28 | RTS5+ | 47 | RXD7- | 67 |  |
| 9 | RXD7+ | 29 | CTS7- | 48 | GND | 68 | TXD1- |
| 10 | RTS4- | 30 | RTS1- | 49 | TXD1+ | 69 | TXD2- |
| 11 | RTS3- | 31 | CTS2+ | 50 | RTS2- | 70 | TXD3- |
| 12 | CTS3+ | 32 | TXD2+ | 51 | CTS1+ | 71 | TXD4- |
| 13 | TXD3+ | 33 | GND | 52 | TXD4+ | 72 | TXD5- |
| 14 | CTS4+ | 34 | CTS2- | 53 | RXD2+ | 73 | TXD6- |
| 15 | RXD2- | 35 | GND | 54 | CTS1- | 74 | TXD7- |
| 16 | RXD1+ | 36 | RXD1- | 55 | RTS1+ | 75 | TXD8- |
| 17 | RTS2+ | 37 | RTS3+ | 56 | RTS4+ | 76 |  |
| 18 | GND | 38 | CTS4- | 57 | RXD4+ | 77 |  |
| 19 | RXD4- | 39 | GND | 58 | CTS3- | 78 |  |
| 20 | RXD3- |  |  | 59 | RXD3+ |  |  |

Table 6. EXM-26 RS-422 DB-78 Pin-out.

## DB-25 Cable Connector

The optional quadracable or octacable shipped with the EXM-26 splits the DB-78 connector into four or eight identical DB-25 connectors. The pin-outs for the DB-25 are listed on the following page.

| Pin Number | Description |
| --- | --- |
| 2 | TXD (Transmit) |
| 3 | RXD (Receive) |
| 4 | RTS (Request to Send) |
| 5 | CTS (Clear to Send) |
| 6 | DSR (Data Set Ready) |
| 7 | GND (Ground) |
| 8 | CD (Carrier Detect) |
| 20 | DTR (Data Terminal Ready) |
| 22 | RI (Ring Indicator) |

Table 7. RS-232 DB-25 Pin-out.

| Pin Number | Description |
| --- | --- |
| 2 | RTS- (Request to Send) |
| 3 | RTS+ (Request to Send) |
| 4 | CTS+ (Clear to Send) |
| 5 | RXD+ (Receive) |
| 6 | CTS- (Clear to Send) |
| 7 | TXD- (Transmit) |
| 8 | GND (Ground) |
| 20 | TXD+ (Transmit) |
| 22 | RXD- (Receive) |

Table 8. RS-422 DB-25 Pin-out.