# 1.  Product Description

This manual contains the information required to install and use the EXM-28 Arcnet controller module.

The EXM-28 is an SMC-compatible Arcnet controller board. It is based on the COM90C66 Arcnet controller chip from SMC. The EXM-28 supports both memory-mapped and sequential I/O mapped access to the on-board Arcnet controller RAM buffer.

Data transfers are configurable for 8-bit or 16-bit data widths. The data rate is 2.5Mbps.

There are two customer options for network connection. The EXM-28 either comes with a BNC connector on the front panel for a coaxial network interface, or has two RJ-11 connectors provided for a twisted pair interface.

An on-board socket is provided for an optional remote-boot PROM available from third-party vendors.

The EXM-28 can be terminated via an on-board jumper for either star or bus topologies.

In addition to the Arcnet controller board features, the EXM-28 has an on-board 82C54 counter/timer chip. There is no external front panel access to the 82C54. Interface is through five bytes configurable in the I/O space. This chip has three individual timer/counters. The first two are clocked via an on-board clock signal. The third has its clock input cascaded from the output of the second counter/timer.

The EXM-28 has a programmable front panel LED. Two other diagnostic LEDs monitor network and board activity.

Multiple EXM-28 boards may be operating within the same system, provided each board's I/O address and any IRQ used are unique.

## Specifications

Table 1 defines the environmental and electrical specifications of the EXM-28.

| Characteristic |  | Value |
| --- | --- | --- |
| Environmental |  |  |
| Temperature | operating | 0° to 60° C (* see below) |
|  | storage | -40° to 85° C |
| Humidity | operating | 5% - 95% (non-condensing) |
|  | storage | 5% - 95% (non-condensing) |
| Vibration | operating | .015"displacement with 2.5 g peak (max) acceleration over 5 to 2000 Hz |
|  | storage | .030" displacement with 5 g peak (max) acceleration over 5 to 2000 Hz |
| Shock | operating | 30g 11 msec duration, half-sine shock pulse |
|  | storage | 50g 11 msec duration, half-sine shock pulse |
| Electrical |  |  |
| Current | maximum | 5V @ 1.0A |
|  | typical | 5V @ 0.7A |
| Other |  |  |
| Weight |  | 3.5 oz. (105 g.) |

Table 1. EXM-28 Environmental and Electrical Specifications.

* Upper temperature limit degrades 2° C per 1000 ft. elevation.  Maximum  
   elevation 10,000 ft.