# 2.  Installation

## Before You Begin

Before inserting the EXM-28 into your system, you must determine if you should use JP1 to terminate the board, and you must set the network node ID.

µ §

Figure 1. EXM-28 Board Layout (With BNC Connector).

### JP1

µ §JP1 is used to terminate the EXM-28, whether using the coaxial or twisted pair cables. Refer to Figure 1 for the location of JP1. The EXM-28 ships with the jumper connected to one pole, not terminated. When JP1 is jumpered, the board is terminated for both star and bus topologies.  Note that each station connected using a star topology must be terminated.  For bus topologies, only the last station and the server are terminated.

### Dip Switches

The set of eight dip switches is a mechanical 8-bit binary code which is used to set the network node ID. The switches are numbered in Figure 1.  Switch 1 is the LSB and sets a 1. Switch 2 sets a 2, Switch 3 sets a 4, and so on. Switch 8 is the MSB and sets a 128.

Valid IDs range from 1 to 256.  O (zero) has a special meaning to the COM90C66 (refer to the SMC literature for more information). Thus the EXM-28 dip switches must be changed and set to a different ID prior to installation.

Leaving a switch down clears it or zeros the bit. Pushing the switch up (off) sets the bit equal to 1. The EXM-28 leaves the factory set to 0 (all switches down).

## Installation

Before installing the EXM-28, unpack and inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-28, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Insertion in an EXM Carrier

Insertion of the EXM-28 into an EXM carrier is straightforward.  Remove a blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the 
EXM-28 into the card guides.  Firmly press the EXM-28 front panel to ensure that the module is properly seated in the subplane and secure it with the thumbscrews. DO NOT OVERTIGHTEN.

*	make sure that power to your system is off.  this exm is not designed to be inserted or removed from a live 
system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

*	insert it with adequate continuous force rather than tapping or hammering on it.

## Configuring the BIOS Setup

The EXM configuration data in the EPC to which the EXM-28 is connected needs to be modified to recognize and enable the card and select from the available options.  Invoke the BIOS setup function by pressing the CTRL-ALT-ESC keys simultaneously.

### Enabling the EXM module

Once in the setup program, a menu will be displayed specifying which function keys are available for further configuration.  Press the F2 function key to invoke the EXM menu.  The screen display resembles Figure 2 below.

ID OB1 OB2       
 Slot 0    FF  00  00
      1    DB  C1  00
      2    D3  61  08
      3    7D  05  00
      4    ED  01  00
      5    7C  E1  FF

Figure 2. EXM Setup Screen.

The setup screen displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.  The displayed data shows SLOT, ID, OB1 and OB2 for each installed EXM.

All slots not occupied by an EXM module should show an ID of FF and OB1/OB2 of  00 00  indicating that no EXM is present. A typical value for the EXM-28 is shown in bold letters in slot 2.

Slot, OB1 and OB2 are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See the diagram below to determine which EXM slot the EXM-28 occupies.  Note that dashed lines indicate EXM slots that may not be available on all systems.

EXM Slots

µ §

Figure 3. EXM Slot Numbering.

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-28 should be set to D3h.

OB1 is a hexadecimal value derived by combining the following:

| Memory Base and RAM offset (bits 7 - 3) | IRQ Select (bits 2 - 1) |  | CDEN Card Enable (bit 0) |  |
| --- | --- | --- | --- | --- |
| See Table 4. | 00 01 10 11 | IRQ9(2) IRQ3 IRQ5 IRQ7 |  | 0- disabled 1- enabled |

Table 2.  EXM-28 OB1 Field.

Bits 7 through 3 set the memory base address and RAM offset:

| bits 7-3 | RAM | PROM | bits 7-3 | RAM | PROM |
| --- | --- | --- | --- | --- | --- |
|  | Address | Address |  | Address | Address |
|  | Range | Range |  | Range | Range |
| 00000 | C0000-C07FF | C2000-C3FFF | 10000 | D4000-D47FF | D6000-D7FFF |
| 00001 | C0800-C0FFF | C2000-C3FFF | 10001 | D4800-D4FFF | D6000-D7FFF |
| 00010 | C1000-C17FF | C2000-C3FFF | 10010 | D5000-D57FF | D6000-D7FFF |
| 00011 | C1800-C1FFF | C2000-C3FFF | 10011 | D5800-D5FFF | D6000-D7FFF |
| 00100 | C4000-C47FF | C6000-C7FFF | 10100 | D8000-D87FF | DA000-DBFFF |
| 00101 | C4800-C4FFF | C6000-C7FFF | 10101 | D8800-D8FFF | DA000-DBFFF |
| 00110 | C5000-C57FF | C6000-C7FFF | 10110 | D9000-D97FF | DA000-DBFFF |
| 00111 | C5800-C5FFF | C6000-C7FFF | 10111 | D9800-D9FFF | DA000-DBFFF |
| 01000 | CC000-CC7FF | CE000-CFFFF | 11000 | DC000-DC7FF | DE000-DFFFF |
| 01001 | CC800-CCFFF | CE000-CFFFF | 11001 | DC800-DCFFF | DE000-DFFFF |
| 01010 | CD000-CD7FF | CE000-CFFFF | 11010 | DD000-DD7FF | DE000-DFFFF |
| 01011 | CD800-CDFFF | CE000-CFFFF | 11011 | DD800-DDFFF | DE000-DFFFF |
| 01100 | D0000-D07FF | D2000-D3FFF | 11100 | E0000-E07FF | E2000-E3FFF |
| 01101 | D0800-D0FFF | D2000-D3FFF | 11101 | E0800-E0FFF | E2000-E3FFF |
| 01110 | D1000-D17FF | D2000-D3FFF | 11110 | E1000-E17FF | E2000-E3FFF |
| 01111 | D1800-D1FFF | D2000-D3FFF | 11111 | E1800-E1FFF | E2000-E3FFF |

Table 3. Memory Base Address Settings.

IRQ - Bits 2 and 1 determine the IRQ. Possible selections are IRQ9, IRQ3, IRQ5, and IRQ7.

CDEN - Bit 0 enables or disables the EXM-28.

A typical OB1 value is 61h (01100001) representing RAM address range D0000 to D07FF, PROM address range D2000 to D3FFF, with IRQ9 and card enabled.

Note that there are many potential address conflicts in upper memory space (the  classic PC problem).  For example, the C-page addresses from C0000-C7FFFF are used to shadow video BIOS and is therefore not usable if a video card is present.   The rest of the C-page is not usable in a system with an EPC-3/EPC-4 or EPC-7, or any system with an EXM-16. When present, the EXM-17 uses the entire "D" page range from D0000-DFFFF. The "E" page (E0000-EFFFF) is used by VME and VXI systems to access the bus.

OB2 is a hexadecimal value derived by combining the following:

| Counter/ Timer I/O base address (bits 7 - 6) | PROM enable bit (bit 5) | Data transfer size (bit 4) | Reserved (bit 3) | Arcnet Base I/O address select (bits 2-0) |
| --- | --- | --- | --- | --- |
| 00 - disable 01 - 240h 10 - 300h 11 - 340h | 0 - disable 1 - enable | 0 - 8 bit 1 - 16 bit | 1 | See Table 6. |

Table 4.  EXM-28 OB2 Field.

Bits 2 through 0 set the base I/O address:

| Setting | ## Arcnet I/O Address Range |
| --- | --- |
| 0 0 0 | ## 0260 - 026F |
| 0 0 1 | ## 0290 - 029F |
| 0 1 0 | ## 02E0 - 02EF |
| 0 1 1 | ## 02F0 - 02FF |
| 1 0 0 | ## 0300 - 030F |
| 1 0 1 | ## 0350 - 035F |
| 1 1 0 | ## 0380 - 038F |
| 1 1 1 | ## 03E0 - 03EF |

Table 5. OB2 Settings for Base I/O Address.

For example, a typical OB2 value is 08h (00001000) representing Counter/Timer disabled, PROM disabled, 8 bit data transfer size, and 260h I/O base address. If the PROM bit is enabled (bit 5 = 1), the data transfer size must be 8-bit (bit 4 = 0).

Avoid these common address ranges to avoid possible conflicts in the system:

2E8-2EF	Serial port (COM4)

2F8-2FF		Serial port (COM2)

3E8-3EF	Serial port (COM3)

The EXM-28 requires up to 21 bytes in the I/O space (16 for the Arcnet and 5 for the counter/timer). Note that the Arcnet and counter/timer bytes are not necessarily contiguous with each other.  Refer to Chapter 4, Programming the EXM-28.

## Installing the Software

This section is intended for use by those who are using the card to connect to Novell NetWare when running DOS/Windows. RadiSys Corp. supplies only IPX and ODI drivers for the EXM-28, which are used for DOS and Windows environments. Other operating systems, such as QNX, may provide built-in drivers for the EXM-28 Arcnet card (QNX is explained briefly below). Be sure the driver is compatible with the COM90C66 chip from SMC.

All network software requires a network interface (driver) as a connecting layer or link between the network software and the network hardware (EXM-28).

µ §

Some operating systems provide both the network software and the network interface layers.  Some operating systems do not provide any of the layers. Before installing the EXM-28, determine how your particular operating system deals with networking; what is provided by the operating system and what you need to provide.  The table below shows how the QNX and DOS operating systems handle this issue.

| Operating System | Network Software | Network Interface |
| --- | --- | --- |
| DOS/Windows | Novell NetWare | ODI - ships with EXM-28 IPX - ships with EXM-28 |
| QNX | TCP/IP - built into OS | Built into OS |

Table 6. Operating Systems and Interface Software.

EXM-28 users of NetWare need to determine if they will be utilizing NetWare 4.0 or an earlier version, such as NetWare 3.11. Users of NetWare 4.0 and above should install the supplied ODI driver. Users of NetWare 3.11 and earlier should install the supplied IPX driver.

With the EXM-28 installed and configured, the next step is to install the software supplied on the distribution diskettes enclosed with the EXM-28. Follow the instructions below.

The distribution disk contains both ODI and IPX drivers.

### ODI

Users of NetWare 4.0 and above should use TRXNET.COM, the supplied ODI driver for the EXM-28. To use the ODI driver, you will also need the Novell workstation modules LSL.COM, IPXODI.COM, and NETX.COM. Consult your network administrator to obtain these files from your Novell system software.

On your EPC, create a directory to contain the EXM-28 ODI driver and the Novell workstation modules. For example, type MKDIR C:\EXM28.

Copy the files you obtained from your network administrator (LSL.COM, IPXODI.COM, and NETX.COM) into C:\EXM28. Now copy TRXNET.COM from the EXM-28 distribution diskette to the C:\EXM28 subdirectory.

Copy NET.CFG from the EXM-28 distribution diskette to your C:\ root directory. Edit this configuration file with the DOS editor (or any other text editor) and make sure that the settings for port, IRQ, and RAM base address match your hardware settings in OB1 and OB2.

At this point, you are ready to load the ODI driver. You can type the following commands, or invoke them in your AUTOEXEC.BAT file to automatically log on to the network when booting up:

C:\EXM28\LSL

C:\EXM28\TRXNET

C:\EXM28\IPXODI

C:\EXM28\NETX

Refer to Chapter 3, Operations, for information about how to cable your EXM-28 to the network. Once you have installed the network cable, contact your system administrator for information about your account, password, etc.

### IPX

Users of NetWare versions 3.11 must create the IPX driver. Refer to the instructions  in the Netware 3.11 Installation manual for more information. For users of NetWare 3.12 or later, refer to your Netware documentation for instructions.

Make the directory called for in the NetWare installation manual to contain your LAN drivers using the following command:

MKDIR [path]

Next, copy to the new directory the RXNETIO.LAN and RXNETIO.OBJ files provided on the EXM-28 distribution diskette. Also copy NETX.EXE (or NET.COM, both obtained from your Novell network administrator).

If not already created by the system administrator, create IPX.COM from RXNETIO.LAN and RXNETIO.OBJ using the Novell WSGEN program. Details are found in the NetWare 3.11 Installation manual.

The IPX.COM driver is configured for hardware setups with invocation options. To view the potential configurations IPX.COM can be set for, type the following:

IPX /D

A table of options displays, with the defaults indicated by an asterisk. If the defaults don't match your requirements, use the override command (IPX /O#) to select the number of another configuration. For help, type IPX /?

To start up the EXM-28 IPX driver, type these commands or put them into your AUTOEXEC.BAT file:

C:\EXM28\IPX

C:\EXM28\NETX

Refer to Chapter 3, Operations, for information about how to cable your EXM-28 to the network. Once you have installed the network cable, contact your system administrator for information about your account, password, etc.