# 3.  Operations

## Connecting to the Network

The EXM-28 can be ordered from the factory with either a BNC connector or two RJ-11 connectors, depending on the customer's requirements.

If using a coaxial network interface, connect the cable to the BNC connector on the front panel. A T-connector ships with the EXM-28 for use if the station is not the last in a bus topology.

If using a twisted-pair interface, connect the cable(s) to the RJ-11 connector(s). For terminating stations, only one cable connects, and it does not matter which connector is used.

For both coaxial and twisted-pair interfaces, if the station is to be terminated, JP1 must be set. Refer to Chapter 2, Installation, for more information.

Once you are connected to the network, contact your system administrator for information about your account, password, etc.

## BNC Connector

An industry-standard BNC connector is provided for coax network interface.

µ §

Figure 4. BNC Connector.

## RJ-11 Connectors

Two RJ-11 connectors provide the twisted pair network interface.  The pin-outs are defined in the following table:

| RJ-11 | Signal |
| --- | --- |
| 1 | No connect |
| 2 | No connect |
| 3 | Line 1 |
| 4 | Line 2 |
| 5 | No connect |
| 6 | No connect |

µ §µ §Table 7. RJ-11  Phone Jacks Pin-out.

## LEDs

There are three LEDs on the front panel: yellow, green, and red. The yellow and green LEDs are diagnostics that monitor board and network activity, respectively.

The red LED is programmable in the fifth byte of the counter/timer I/O space. Refer to Chapter 4, Programming the EXM-28, for more information.

## Boot PROM

The EXM-28 can accept an optional boot PROM for remote boot. The open socket is shown in Figure 1 on page 3. Note that the location of pin 1 on the PROM is designated in the figure.