# 4.  Programming 
the EXM-28

This chapter is intended for users who will be using the counter/timer or otherwise writing programs for the EXM-28.  The registers described below consist of the three standard EXM registers, five counter/timer registers, and 16 Arcnet registers.  Information about the Arcnet registers is found in the data sheet for the COM90C66 chip.

## Standard EXM Registers

The EXM-28 registers are described below:

|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 | I/O port |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Device ID Reg | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Config Option Byte 1 Reg | M4 | M3 | M2 | M1 | M0 | IRQ1 | IRQ0 | Cden | 102 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Config Option Byte 2 Reg | CTH | CTL | PROM | Size | Res | IOS2 | IOS1 | IOS0 | 103 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 5. EXM-28 Standard EXM Registers.

The registers listed in Figure 5 are standard EXM registers for device identification and configuration.  The EXM-28 responds to accesses to these ports only if EXM expansion interface line -EXMID is asserted (by writing the slot number to I/O address 96h).  An 8-bit read from I/O address 100h returns the device ID value D3.  Read/write configuration registers appear at I/O address 102h and 103h.

## Counter/Timer Registers

The four registers from the EXM-28 Base Address + 0h to Base Address + 3h correspond to the four registers of the 82C54 Programmable Counter/Timer.  Base Address + 4h is used for interrupt, LED and additional counter/timer control. (See the section Byte 5 later in this chapter.)

| Address | Register | Status |
| --- | --- | --- |
| Base Address + 0h | Counter 0 Register | Read/Write |
| Base Address + 1h | Counter 1 Register | Read/Write |
| Base Address + 2h | Counter 2 Register | Read/Write |
| Base Address + 3h | 82C54 Control Register | Write Only |

Table 8. 82C54 Programmable Interval Counter/Timer.

A brief description of each register is given below. For more detailed information, refer to the Intel data sheet on the 82C54.

Counter 0, Counter 1, Counter 2

These three counters are identical in operation. Counters 0 and 1 operate at 1 MHz, and are fully independent. The output of Counter 1 is cascaded into the clock input of Counter 2.  Each counter may operate in a different mode.

### Byte 5

The fifth byte in the I/O space is located at Base + 4h and is used for interrupt, LED, and counter/timer control, as described below. Note that the fifth byte is an externally implemented register and is not part of the 82C54 itself.

| Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LED | G2 | G1 | G0 | IRQH | IRQL | IIDH | IIDL |

| IRQH | IRQL | Interrupt |
| --- | --- | --- |
| 0 | 0 | none |
| 0 | 1 | IRQ5 |
| 1 | 0 | IRQ7 |
| 1 | 1 | IRQ15 |

IRQ settings

| IIDH | IIDL | Counter |
| --- | --- | --- |
| 0 | 0 | Counter 0 Interrupt |
| 0 | 1 | Counter 1 Interrupt |
| 1 | 0 | Counter 2 Interrupt |
| 1 | 1 | No interrupt pending |

Counter selection

All but the IIDH and IIDL bits (read only) are both readable and writeable. Setting the bit '1' activates the gate enable; clearing the bit (0) disables the gate. Because the output from counter 1 is cascaded into the input of counter 2, the gate for counter 1 is automatically enabled with the gate for counter 2 when the G2 bit is set. Refer to Figure 6. Interrupts from counter 1 are masked unless the G1 bit in the base + 4 register is also set.

Bits IRQH and IRQL control the interrupt used. The chosen interrupt will be generated whenever any of the counter/timers counts down to 0 (zero) and the appropriate Gx bit is a '1'. Interrupts will be ignored, and pending interrupts will be cleared when the Gx bit is cleared.  All the counter/timers will share the same interrupt. The counter/timer generating the interrupt can be identified by the IIDH and IIDL bits. A read of the register will reset these bits. If a second counter/timer generates a signal before a read of the register, it will be buffered until the first interrupt is cleared. If an individual counter/timer generates a second interrupt before the first has been cleared, it will be lost.

The LED bit, when set ('1') illuminates the front panel software programmable LED. When clear ('0'), the red LED is extinguished.

µ §

Figure 6. Counter/Timer Enable Diagram.

82C54 Control Word

The Control Word Register sets up the mode of the 82C54 and initializes all reads and writes to the counter.

Programming the 82C54 Counters

Counters are programmed by writing a Control Word into the Control Word Register, and then an initial count to the appropriate counter register. The Control Word specifies which counter is being programmed. For each counter, the Control Word must be written before the initial count is written. The initial count must follow the count format specified in the Control Word (LSB only, MSB only, or LSB and then MSB).

If a counter is programmed to read/write two-byte counts, the program must not transfer control between writing the first and second byte to another routine which also writes into that same counter.

Base + 3h

| Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SC1 | SC0 | RW1 | RW0 | M2 | M1 | M0 | BCD |

Table 9. Control Word Register.

| SC - Select Counter |  |  |  | M2 - M1 |  | M0 |  |  | Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | Counter 0 | 0 |  | 0 |  | 0 | Mode 0 |  |
| 0 | 1 | Counter 1 | 0 |  | 0 |  | 1 | Mode 1 |  |
| 1 | 0 | Counter 2 | x |  | 1 |  | 0 | Mode 2 |  |
| 1 | 1 | Read-Back | x |  | 1 |  | 1 | Mode 3 |  |
| RW1 - RW0 |  |  | 1 |  | 0 |  | 0 | Mode 4 |  |
| 0 | 0 | Counter Latch | 1 |  | 0 |  | 1 | Mode 5 |  |
| 0 | 1 | R/W LSB | Binary Counter Bits (bit 0) |  |  |  |  |  |  |
| 1 | 0 | R/W MSB | 0 |  | Binary Counter 16 Bits |  |  |  |  |
| 1 | 1 | R/W LSB 1st, then MSB | 1 |  | Binary Coded Decimal Counter (4 Decades) |  |  |  |  |

Table 10. 82C54 Control Word Format.

Programming the 82C54 Modes of Operation

There are six modes:

0. Interrupt on Terminal Count

1. Hardware Retriggerable One-Shot

2. Rate Generator

3. Square Wave Mode

4. Software Triggered Strobe

5. Hardware Triggered Strobe (Retriggerable)

Of the six modes, Mode 3, Square Wave Mode, is the most commonly used and is described below. For more information about the Counter/Timer and the rest of the available modes, refer to the Intel data sheet for the 82C54.

Mode 3: Square Wave Mode

Mode 3 is typically used for regular interval triggering.  Mode 3 is implemented as follows:

Even counts: OUT is initially high. The initial count is loaded on one CLK pulse and then is decremented by two on succeeding CLK pulses. When the count expires OUT changes value and the Counter is reloaded with the initial count. The above process is repeated indefinitely.

Odd counts: OUT is initially high. The initial count minus one (an even number) is loaded on one CLK pulse and then is decremented by two on succeeding CLK pulses. One CLK pulse after the count expires, OUT goes low and the Counter is reloaded with the initial count minus one. Succeeding CLK pulses decrement the count by two. When the count expires, OUT goes high again and the Counter is reloaded with the initial count minus one. The above process is repeated indefinitely. So for odd counts, OUT will be high for (N + 1)/2 counts and low for (N-1)/2 counts.

GATE = 1 enables counting; GATE = 0 disables counting. If GATE goes low while OUT is low, OUT is set high immediately; no CLK pulse is required. A trigger (accomplished by setting the gate bit) reloads the Counter with the initial count on the next CLK pulse. Thus the Gate input can be used to synchronize the Counter. After writing a Control Word and initial count, the counter is loaded on the next CLK pulse. This allows the counter to be synchronized by software also.

Writing a new count while counting does not affect the current counting sequence. If a trigger is received after writing a new count but before the end of the current half-cycle of the square wave, the Counter is loaded with the new count on the next CLK pulse and counting will continue from the new count. Otherwise, the new count is loaded at the end of the current half-cycle.

## COM90C66 Arcnet Controller Chip

The Arcnet controller chip used on the EXM-28 contains internal registers which may be accessed by the microprocessor. A detailed description of these registers is beyond the scope of this manual. For more information, refer to the SMC data sheet for the COM90C66 chip.

Under RadiSys' implementation of the EXM-28 COM90C66 Arcnet controller chip, when writing a device driver for this chip, do not change from 8-bit to 16-bit mode (or vice versa) by writing directly to the chip register. If it is necessary to change modes after chip initialization, the change should be made by changing the SIZE bit in register OB2 (103h). The card enable bit (CDEN, or bit 0) of register 102h must first be cleared to disable the card. Next change the SIZE bit in register 103h. Then re-assert the CDEN bit in register 102h. When the card is re-enabled in this way, the COM90C66 chip will be re-initialized to the correct value.

NOTES