# 1.  Product Description

This manual contains the information you need to install and use the EXM-29 RS422/RS485 serial-port interface.

The EXM-29 provides two RS-422/RS-485 serial ports. It is software compatible with the ICS FASTCOMM/422 dual port board, save for two exceptions. First, the 
EXM-29 is configured via programmable registers on the board, instead of via dip switches. Second, the EXM-29 does not have the capability of sharing interrupts across multiple boards. However, interrupt sharing between ports on the same board is available. Thus, once configured, the EXM-29 is capable of running any software developed for FASTCOMM/422 control, provided different boards use different interrupts.

Two DB-9 connectors on the front panel provide the port interfaces. In addition, LEDs on each port's transmit and receive lines provide a visual display of port activity.

Port input signals (RXD, CTS) can be terminated with on-board 100W resistors connected through a dip switch. Therefore, termination resistors can be switched in or out as needed.

Port outputs can be configured to be gated by the port RTS signal. CTS handshaking may be enabled or ignored for each port independently. Each port may be independently set for half-duplex mode (TXD and RXD signals connected together on the board).

Each port is controlled by a separate 82510 asynchronous serial controller, capable of running at baud rates up to 288K. Other features include 5-9 bit character formats, 4 byte independent transmit and receive FIFOs, and 16450 compatibility.

## Specifications

The following table defines the environmental and electrical specifications of the 
EXM-29.

| Characteristic |  | Value |
| --- | --- | --- |
| Environmental Specifications |  |  |
| Temperature | operating | 0 to 60°C ambient |
|  | storage | -40 to 85°C |
| Humidity | operating | 5% to 90% non condensing |
|  | storage | 5% to 95% non condensing |
| Altitude | operating | 10,000 ft (3,000 m) |
|  | storage | 50,000 ft (15,000 m) |
| Vibration | operating | 0.015 inch (0.38 mm) Peak to Peak displacement with 2.5 g peak (max) acceleration over 5 to 2000 Hz |
|  | storage | 0.030 inch (0.76 mm) P-P displacement with 5.0 g peak (max) acceleration over 5-2000 Hz |
| Shock | operating | 30 g, 11 ms duration, half-sine shock pulse |
|  | storage | 50 g, 11 ms duration, half-sine shock pulse |
| Electrical Specifications |  |  |
| Current | typical | +5V @ 0.370A |
|  | maximum | +5V @ 0.53A |

Table 1. EXM-29 Environmental and Electrical Specifications.