# 2.  Installation

Before installing your EXM-29, you should unpack and inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-29, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Insertion in an EXM Carrier

Insertion of the EXM-29 into an EXM carrier is straightforward.  In the case of the EXP-MC, first ensure that the subplane has been inserted into the backplane and that the EXP-MC has been inserted into the subplane and attached to the chassis.  Then remove any blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the EXM-29 into the card guides.  Firmly press the EXM-29 front panel to ensure that it has mated with the rear connector, and secure it with the thumbscrews.

*	make sure that power to your system is off.  the exm is not designed to be inserted or removed from a live system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

## Configuration Using BIOS Setup Screen

The EXM configuration data in the EPC to which the EXM-29 is connected needs to be modified to recognize and enable the card and select from the available options.  Invoke the BIOS setup function by pressing the CTRL-ALT-ESC keys simultaneously.

### Enabling the EXM module

Once in the setup program, a menu displays specifying which function keys are available for further configuration.  Press the F2 function key to invoke the EXM menu.  The screen display resembles Figure 1 below.

ID OB1 OB2       
 Slot 0    FF  00  00
      1    DB  C1  00
      2    D2  AB  FF
      3    DE  01  00
      4    ED  01  00
      5    DC  F5  91

Figure 1. EXM Setup Screen.

The setup screen displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.  The displayed data shows SLOT, ID, OB1 and OB2 for each installed EXM.

Note that all slots are listed on the screen even if the actual system configuration does not have all the possible EXM slots.  All slots not occupied by an EXM module should show an ID of FF and OB1/OB2 of   00  00  indicating that no EXM is present. A typical value for the EXM-29 is shown in bold letters in slot 2.

Slot, OB1 and OB2 are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See the diagram below to determine which EXM slot the EXM-29 occupies.  Note that dashed lines indicate EXM slots that may not be available on all systems.

EXM Slots

µ §

Figure 2. EXM Slot Numbering.

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-29 is D2h.

OB1 and OB2 are two bytes of option information.

OB1 is a hexadecimal value derived by combining the following:

| Resrvd (bit 7) | Port 2 IRQH (bit 6) | Port 2 IRQM (bit 5) | Port 2 IRQL (bit 4) | Port 1 IRQH (bit 3) | Port 1 IRQM (bit 2) | Port 1 IRQL (bit 1) | Card Enable (bit 0) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Set to 1 | See below | See below | See below | See below | See below | See below | 0 - disable 1 - enable |

Table 2. OB1 Configuration Bit Definition.

IRQ settings are displayed in the following table:

| Setting | IRQ |
| --- | --- |
| 000 | none |
| 001 | none |
| 010 | IRQ9(2) |
| 011 | IRQ3 |
| 100 | IRQ4 |
| 101 | IRQ5 |
| 110 | IRQ6 |
| 111 | IRQ7 |

Table 3. IRQ Settings for OB1.

For instance, a typical OB1 value is ABh (10101011), representing Port 2 set to IRQ9(2), Port 1 set to IRQ5, and card enabled.

OB2 is a hexadecimal value derived by combining the following:

| Resrvd (bit 7) | Port 2 RTS (bit 6) | Port 2 CTS (bit 5) | Port 2 RS485 (bit 4) | Resrvd (bit 3) | Port 1 RTS (bit 2) | Port 1 CTS (bit 1) | Port 1 RS485 (bit 0) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Set to 1 | See below | See below | 0 - RS422 1 - RS485 | Set to 1 | See below | See below | 0 - RS422 1 - RS485 |

Table 4. OB2 Configuration Bit Definition.

The OB2 register holds the RS-422/RS-485 select bits (bits 0 and 4) for each port;  the CTS handshaking enable bit for each port (bits 1 and 5); and the RTS gate enable bits for each port (bits 2 and 6). When configured for RS-485, the board is set up in half-duplex mode (i.e., the TX+ and TX- lines are connected to the RX+ and RX- lines, respectively).

The CTS handshaking bits (bits 1 and 5) enable or disable CTS handshaking. The default setting of 0 represents CTS enabled, and 1 represents CTS disabled.

The RTS gate enable bits (bits 2 and 6) disable or enable the RTS signal control of the port line drivers. The default setting of 0 disables the RTS gate enable bits, while a 1 enables the RTS gate enable bits. When enabled, the line drivers are enabled/disabled by the state of the RTS signal.

For instance, a typical OB2 value is FFh (11111111), representing RTS gate enable mode is enabled, CTS is disabled and RS-485 is enabled for Port 2 and Port 1.

### OB3

This register is located at 104h and holds the base I/O address select bits for the 8 bytes of I/O space needed by port 1 of the EXM-29.

µ §

These bits directly correspond to address lines SA3-SA10 on the system bus. SA15 is decoded as well ('0' expected), to prevent conflict with certain RadiSys VXI bus I/O registers.

Note that this register is not configurable from the BIOS setup screen, thus must be programmed after the system boots, if the default address of 3E8 (OB3 = 7Dh) is not acceptable. See the section on I/O Base Address Configuration for information both on the default address and programming considerations.

The contents of the OB3 register are NOT battery-backed. This register returns to the default value on power-up.

### OB4

This register is located at 105h and holds the base I/O address select bits for the 8 bytes of I/O space needed by port 2 of the EXM-29.

µ §

These bits directly correspond to address lines SA3-SA10 on the system bus. SA15 is decoded as well ('0' expected), to prevent conflict with certain RadiSys VXI bus I/O registers.

Note that this register is not configurable from the BIOS setup screen, but must be programmed after the system boots, if the default address of 2E8 (OB4 = 5Dh) is not acceptable. See the section on I/O Base Address Configuration for information both on the default address and programming considerations.

The contents of the OB4 register are NOT battery-backed. This register returns to the default value on power-up.

If you change the configuration of the ports, make sure they do not conflict with other used I/O addresses. In case of conflict, a hardware reset returns the port base address to the default.

## I/O Base Address Configuration

The EXM-29 port base addresses are set in Option Byte Registers 3 and 4. These bytes are not configurable from the BIOS setup screen, but must be set via software, unless the default values are acceptable.

The default values are 7Dh for Option Byte Register 3 (corresponding to an I/O base address of 3E8h for port 1), and 5Dh for Option Byte Register 4 (corresponding to an I/O base address of 2E8h for port 2).

Because these registers are part of the EXM configuration space, changes must be preceded by a write to the register at address 96h. This register must be written with the slot number in which the EXM-29 to be configured resides.

Therefore, changing the base address for a port is a two step process. First, address 96h is written with the appropriate slot number. Then, the appropriate Option Byte Register is written with the correct value. Refer to Figure 2 for slot numbering.

To facilitate this process, software is provided with the EXM-29 for easy configuration under DOS. The executable file EXM29CFG.EXE accepts up to four parameters (optional "quiet" mode for no message display, the slot number, the port 1 base address to set, and the port 2 base address to set). Invoking this executable from within the AUTOEXEC.BAT file on boot up essentially provides the same effect as if the I/O base addresses were configured from the BIOS setup screen.

Invocation of the EXM29CFG.EXE program is as follows:

exm29cfg [/q] [/slot:<val>] [/ob3:<val>] [/ob4:<val>]

Options may be preceded by either a forward slash, '/', or a hyphen, '-'. Options may be added in any order. Values are always separated from the option by the colon, ':'.

Integer or hexadecimal values must not be preceded or trailed by additional characters or padding. For example, the value 3E8h must be entered as '3E8', not '0x03E8' or '3E8h'. Zero must be entered as '0', not '00'. Options and values are case-insensitive.

On encountering this, the program exits and displays a summary of options without fully invoking the program.

Options are not required. All have default values. One special option is pre-defined. This is the '?' option. A quick review of the default values can be obtained by invoking the program with the summary option, '/?'.  Any option not explicitly set will be treated as though it were set with the default value.

Default values are:

OB3	3F8h

OB4	2F8h

slot	slot 0

Options:

/q -	This option directs the program not to display any messages while running 	or upon completion, except in the case of error. This is useful when the 	program is invoked from AUTOEXEC.BAT.

/slot -	This is the slot in which the EXM-29 to be configured resides. This should 	be an integer value.

/ob3 -	This guides the value for the Option Byte 3 register. 	The value associated 	with this option should be the I/O address, in hexadecimal, at which 	EXM-29 port 1 will be based. The program will take care of adjusting this 	address such that the appropriate value is loaded into the Option Byte 3 	register.

/ob4 -	This guides the value for the Option Byte 4 register. The value associated 	with this option should be the I/O address, in hexadecimal, at which 	EXM-29 port 2 will be based. The program will take care of adjusting this 	address such that the appropriate value is loaded into the Option Byte 4 	register.

Example:

Assuming an EXM-29 in slot 3, to configure the port 1 base

address at 2A0h, and the port 2 base address at 2B8h, the

program would be invoked as follows:

exm29cfg /slot:3 /ob3:2a0 /ob4:2b8