EXM-31 14.4K Baud V.32bis Modem 
Hardware Reference



RadiSys  Corporation

15025 S.W. Koll Parkway

Beaverton OR 97006

(503) 646-1800

FAX: (503) 646-1850

AT&T is a registered trademark of American Telephone and Telegraph, Inc.

EPC and RadiSys are registered trademarks of RadiSys Corporation.

May 1995

Copyright  1995 by RadiSys Corporation

All rights reserved.

Hardware Warranty

RadiSys Corporation ("RadiSys") warrants the EPC system and component modules to the original purchaser for two years from the product's shipping date.  If an EPC product fails to operate in compliance with its specification during this period, RadiSys will, at its option, repair or replace the product at no charge.  The customer is, however, responsible for shipping the product; RadiSys assumes no responsibility for the product until it is received.  This warranty does not cover repair of products that have been damaged by abuse, accident, disaster, misuse, or incorrect installation.

RadiSys' limited warranty covers products only as delivered.  User modification, such as the addition of memory arrays or other devices, may void the warranty, and if the product is damaged during installation of the modifications, this warranty does not cover repair or replacement.

This warranty in no way warrants suitability of the product for any specific application.

IN NO EVENT WILL RADISYS BE LIABLE FOR ANY DAMAGES, INCLUDING LOST PROFITS, LOST SAVINGS, OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES ARISING OUT OF THE USE OR INABILITY TO USE THE PRODUCT EVEN IF RADISYS HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES, OR FOR ANY CLAIM BY ANY PARTY OTHER THAN THE PURCHASER.

THE ABOVE WARRANTY IS IN LIEU OF ANY AND ALL OTHER WARRANTIES, EXPRESSED OR IMPLIED OR STATUTORY, INCLUDING THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE OR USE, TITLE AND NONINFRINGEMENT.  Repair or replacement as provided above shall be the Purchaser's sole and exclusive remedy and RadiSys' exclusive liability for any breach of warranty.

NOTES

Table of Contents

1.  Product Description	1

Specifications	2

2.  Installation	3

Insertion in an EXM Carrier	3

Configuring the BIOS Setup	4

Enabling the EXM module	4

3. Connector	9

RJ-11 Connector	9

4. Programming Interface	11

UART Registers	12

AT Commands	14

Initialization Strings	14

Basic AT Commands	14

A/ Repeat Last Command	14

A Answer Command	14

Bn Communication Standard Option	14

Cn Carrier Control Option (dummy command)	15

Dn Dial Command	15

En Off-Line Echo Command	16

Fn On-Line Echo Command (dummy command)	16

Hn Switch-hook Control	17

In Identification/Checksum Option	17

Ln Speaker Volume	17

Mn Speaker Control	18

Nn Select Negotiate Handshake	18

On Go On-Line	18

P Enable Pulse Dialing	19

Qn Results Code Display Option	19

Sn Selects S Register	19

Sn=x Writes to an S Register	19

Sn?  Read an S Register	20

T Enable Tone Dialing	20

Vn Result Code Form	20

Wn Select Extended Result Codes	20

Xn Select Result Codes/Call Progress Options	21

Yn Long Space Disconnect	21

Zn Recall Stored Profile	22

AT& (Ampersand) Commands	22

&Bn V.32 Autoretrain	22

&Cn Data carrier Detect Option	22

&Dn Data Terminal Ready Option	23

&F Load Factory Settings	23

&Gn Guard Tone Option	23

&Ln Dial Up./Lease Line Option	24

&Mn Asynchronous Mode	24

&Pn Dial Pulse Ratio	25

&Qn Communication Mode Option	25

&Rn CTS/RTS Option (non-CTS flow control mode)	26

&Sn Data Set Ready Option	26

&Tn Self-Test Commands	26

&Un Disable Trellis Coding	27

&Vn View Active Configuration and Stored Profile	27

&Wn Store Active Profile.	28

&Xn Synch Transmit Clock Source Option	28

Yn Select Stored Profile on Powerup	29

Zn=x Stored Telephone Number	29

AT% Commands	29

%An Set Auto-reliable Fallback Character	29

%Cn Data Compression	30

%Dn 108 Mode Select	30

%Fn Character Framing Select	30

%En Enable/Disable Autoretrain	30

%Gn Auto Fallback Enable (V.32bis only)	31

%O Escape Method	31

%V Command Set Switch	31

AT\ (back slash) commands	31

\An Block Size	31

\Bn Transmit Break	32

\Cn Set Autoreliable Buffer	32

\Gn Set modem Port Flow Control	33

\Jn Bits/s Rate Adjust	33

\Kn Set Break Control	33

/Nn Set Operating Mode	34

\Qn Set Serial Port Flow Control	34

\Tn Set Inactivity Timer	35

\Vn Modify Result Code Form	35

\Xn Set XON/XOFF Pass-through	35

AT- (Dash) Commands	36

-Cn Set Calling Tone in Data Mode	36

-Jn Set V.42 Detect Phase	36

-Kn MNP Extended Services	36

-Qn Fallback Modulation Speeds (MNP 10)	36

-V Display Number	37

AT” (Double-quote) Commands	37

“Hn V.42bis Compression Control	37

“On n = 6 - 256	37

“Mn MNP 10 Control	38

S Registers	39

S0  Ring to Autoanswer On	39

S1  Ring Count	39

S2  Escape Character	40

S3  Carriage Return <CR> Character	40

S4  Line Feed <LF> Character	40

S5  Back Space <BS> Character	40

S6  Wait Before Dialing	41

S7  Wait for Carrier	41

S8  Pause Time for Command or Dial Modifier	41

S9  Carrier Recovery Time	42

S10  Lost Carrier Hang Up Delay	42

S11  DTMF Dialing Speed	42

S12  Guard Time	42

S14  Bit Mapped Options	43

S16  Modem Test Options (read only)	43

S18  Modem Test Timer	44

S19  Autosync Protocol Support Mode	44

S20  Autosync Protocol Support Mode	44

S21  Bit-Mapped Options	45

S22  Bit-Mapped Options	45

S23  Bit Mapped Options	46

S25  Detect DTR Change	47

S26  RTS to CTS Delay Interval	47

S27  Bit-Mapped Option	47

S30  Inactivity Timer Value	48

S32  Wait for <CR> Time	48

S37  Desired DCE Speed	48

S95  Extended Result Code Bit Map	48

S108 Signal Quality Selector	49

S110 V.32/V.32bis/V.32terbo Selector	49

S112 DTE Speed Select During Data Transfer	50

V.25bis Commands Set	50

CIC Enable Autoanswer	50

CRN Dial Command	50

CRS Dial Stored Number	51

DIC Disable Autoanswer	51

PRN Store Telephone Number	51

RLD Display Delayed Numbers	52

RLN Display Stored Numbers	52

Standard V.25bis Indications	52

CFI Call Failure Indication	52

CNX Connect	53

DLC Delayed Call	53

INC Incoming Call	53

INV Invalid	53

LSD List of Delayed Call Numbers	53

LSN List of Stored Numbers (Dial Strings)	54

VAL Valid	54

Result Codes	54

5.  Support and Service	55

In North America	55

Technical Support	55

Bulletin Board	55

Repair Services	56

Warranty Repairs	56

Non-Warranty Services	56

Arranging Service	57

Other Countries	58

Figures

Figure 1. EXM Setup Screen 	  4

Figure 2. EXM Slot Numbering	  5

Figure 3. OB1 Register	  5

Figure 4. OB2 Register	  6

Tables

Table 1. EXM-31 Environmental and Electrical Specifications	  2

Table 2. OB1 Settings 	  6

Table 3. RJ-11 Pin-out	  9

Table 4. UART Registers	12

Table 5. Result Codes	54