# 1.  Product Description

This manual contains the information required to install and use the EXM-31 High Speed Modem expansion module.

The EXM-31 is a plug-in PC-compatible internal modem for computer systems with EXM expansion capabilities. As an internal modem, it includes an 16550-compatible serial port, and can be set to standard PC I/O COM1-COM4. It can also be configured to use one of eight interrupts.

The EXM-31 is based on the AT&T HSM144DC chip set. When configuring with standard PC software, use a classification such as “Generic AT&T Class 1 Fax, 14400 Data Modem.”  The EXM-31 supports the following features:

· Data Modes

	CCITT V.32bis: 14400 (TCM), 12000 (TCM), 7200 (TCM)

	CCITT V.32: 9600 (TCM), 4800 (QAM)

	CCITT V.22bis: 2400 (QAM)

	CCITT V.22: 1200 (DPSK)

	CCITT V.21: 300 (FSK)

	CCITT V.23: 75/1200 (FSK)

	Bell 212A: 1200 (DPSK)

	Bell 103: 300 (FSK)

· Fax Modes

	CCITT V.17: 14400 (TCM), 12000 (TCM), 9600 (TCM), 7200 (TCM)

	CCITT V.29: 9600 (QAM), 7200 (QAM)

	CCITT V.27ter: 4800 (DPSK), 2400 (DPSK)

	CCITT V.21 Channel 2: 300 (FSK)

·	V.42 error correction (LAPM and MNP)

·	V.42bis and MNP 5 data compression

·	V.42bis data compression over MNP

·	Extensive AT commend set

·	EIA/TIA 578 Class 1

·	One of the following options:

	Classic A: Hayes Auto Sync

	Classic B: Class 2 (August 1990) Fax Interface

·	Integrated 550 UART emulation

Note that there are two versions of this product: on is for international markets, and the other is a U.S. version. This is a build-time option.

## Specifications

Table 1 defines the environmental and electrical specifications of the EXM-31.

| Characteristic |  | Value |
| --- | --- | --- |
| Environmental |  |  |
| Temperature | operating | 0° to 60° C (* see below) |
|  | storage | -40° to 85° C |
| Humidity | operating | 5% - 95% (non-condensing) |
|  | storage | 5% - 95% (non-condensing) |
| Vibration | operating | .015"displacement with 2.5 g peak (max) acceleration over 5 to 2000 Hz |
|  | storage | .030" displacement with 5 g peak (max) acceleration over 5 to 2000 Hz |
| Shock | operating | 30g 11 msec duration, half-sine shock pulse |
|  | storage | 50g 11 msec duration, half-sine shock pulse |
| Electrical |  |  |
| Current | maximum | 5V @ 700 mA |
|  | typical | 5V @ 500 mA |

Table 1. EXM-31 Environmental and Electrical Specifications.

* Upper temperature limit degrades 2° C per 1000 ft. elevation.  Maximum  
   elevation 10,000 ft.