# 2.  Installation

Before installing the EXM-31, unpack and inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-31, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Insertion in an EXM Carrier

Insertion of the EXM-31 into an EXM carrier is straightforward.  Remove a blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the 
EXM-31 into the card guides.  Firmly press the EXM-31 front panel to ensure that the module is properly seated in the subplane and secure it with the thumbscrews. DO NOT OVERTIGHTEN.

*	make sure that power to your system is off.  this exm is not designed to be inserted or removed from a live 
system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

*	insert it with adequate continuous force rather than tapping or hammering on it.

## Configuring the BIOS Setup

Once the EXM-31 is inserted into the carrier, turn the system on and boot it up. The EXM configuration data in the EPC to which the EXM-31 is connected now must be modified to recognize and enable the card and select from the available options.  Invoke the BIOS setup function from the command line by pressing the CTRL-ALT-ESC keys simultaneously.

### Enabling the EXM module

Once in the setup program, a menu displays specifying which function keys are available for further configuration.  Press the F2 function key to invoke the EXM menu.  The screen display resembles the figure below.

ID OB1 OB2       
 Slot 0    DC  00  00
      1    DB  C1  00
      2    D0  07  DD      
      3    DE  00  39
      4    F5  05  00
      5    DC  F5  00

Figure 1. EXM Setup Screen.

The EXM Setup Screen displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.  An example of configuration data for the EXM-31 is shown in bold in slot 2. The displayed data shows SLOT, ID, OB1 and OB2.  These are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See the diagram below to determine which EXM slot the EXM-31 occupies.  Note that dashed lines indicate EXM slots that may not be available on all systems.

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

EXM Slots

Figure 2. EXM Slot Numbering.

Figure 2 lists two sample EXM slot configurations, but there are several chassis options available. Consult the specification sheet that shipped with your chassis if you are unsure about slot numbering for your particular system.

All slots not occupied by an EXM module should show an ID of FF and OB1/OB2 of   00  00  indicating that no EXM is present.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-31 should be set to D0h (D-zero).

OB1 is a hexadecimal value derived by combining the following:

| Reserved (bits 7-4) |  |  |  | INT Select (bits 3,2,1) |  | Card Enable (bit 0) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |  | see below |  | 0 disable 1 enable |

Figure 3. OB1 Register.

OB1 contains the interrupt select and the card enable bit.

The interrupt select is as follows:

| Bit 3 | Bit 2 | Bit 1 | Interrupt |
| --- | --- | --- | --- |
| 0 | 0 | 0 | IRQ9 |
| 0 | 0 | 1 | IRQ3 |
| 0 | 1 | 0 | IRQ4 |
| 0 | 1 | 1 | IRQ5 |
| 1 | 0 | 0 | IRQ7 |
| 1 | 0 | 1 | IRQ11 |
| 1 | 1 | 0 | IRQ12 |
| 1 | 1 | 1 | IRQ15 |

Table 2. OB1 Settings.

For example, a typical OB1 value is 07h (0000 0111) indicating the card is enabled at IRQ5.

Note that interrupts cannot be disabled. Make sure that the IRQ chosen for the 
EXM-31 does not conflict with any other modules that use an IRQ.  Some common IRQs to avoid are the following:

IRQ3		if already using COM2

IRQ4		if already using COM1

IRQ6		if using floppy disk controller

IRQ7		if using LPT1

OB2 is used to select the base address.

| Reserved bit 7 | Base Address Select (bits 6-0) |
| --- | --- |
| Set to 1 | see below |

Figure 4. OB2 Register.

Base Address specifies the base address of the UART I/O registers.  This value is matched against SA[9..3] to provide an I/O chip select.

The EXM-31 consumes a contiguous eight byte region of I/O space.  Some suggested address ranges are given below:

Address		Hex Value	OB2 Value

3F8-3FF		7F		FF

2F8-2FF		5F		DF

3E8-3EF	7D		FD

2E8-2EF	5D		DD

2A0-2A7	54		D4

2B0-2B7	56		D6

It is suggested that the address range of  000 - 107 not be used as the base address due to possible conflicts while configuring other EXMs or AT peripherals in the system. Some other common base I/O address ranges to avoid are the following:

1F0-1FF		IDE disk controller

278-27F		Parallel port (LPT2)

378-37F		Parallel port (LPT1)

3F0-3F7		Floppy disk controller

For example, a typical OB2 setting would be DD, setting the modem to use base address 2E8-2EF.

NOTES