# 4. Programming Interface

This chapter contains information needed to write custom software drivers for the EXM-31.  Anyone using the EXM-31 with software compatible with standard PC modems in general,  or specifically a “generic AT&T Class 1 Fax, 14400 Data Modem” can skip reading this chapter.

The EXM-31defines the following registers in the I/O space (the MSB is the left-most bit).

| Bit 7 | Bit 6 |  | Bit 5 | Bit 4 |  |  | Bit 3 |  | Bit 2 |  |  | Bit 1 |  |  | Bit 0 |  |  |  | I/O Port |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 |  | 0 | 1 |  |  | 0 |  | 0 |  |  | 0 |  |  | 0 |  |  |  | 100 |  |  |
| Device ID Register |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reserved |  |  |  |  |  | Interrupt Select |  |  |  |  |  |  |  |  |  | CDEN |  |  |  | 102 |  |
| Configuration Option Byte 1 Register |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | Base Address Select |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 103 |  |  |  |
| Configuration Option Byte 2 Register |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

These registers are standard EXM registers for device identification and configuration.  The EXM-31 responds to accesses to these ports only if EXM expansion interface line -EXMID is asserted.  An 8-bit read from I/O address 100h returns the value D0, the device ID of the EXM-31.  Read/write configuration registers appear at I/O address 102h and 103h.  The bit encodings are defined in chapter 2.

The upper four bits of register 102 are reserved.

Interrupt selects the interrupt to be generated.

Base address selects the place in the I/O space in which the 8-byte I/O port area of EXM-31 appears.

CDEN specifies whether the EXM is disabled or enabled.  If disabled, the EXM will not respond to any I/O or memory addresses and will not assert an interrupt output; it will only respond to reads from I/O port 100h and reads and writes from I/O port 102h, and then only if EXM expansion interface line -EXMID is asserted.

## UART Registers

The table below describes the 16C550 registers.

| Bit # | Register Address |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | OD=0 | OD=0 | 1D=0 | 2 | 2 | 3 | 4 | 5 | 6 | 7 | OD=1 | 1D=1 |
|  | RBR | THR | IER | IIR | FCR | LCR | MCR | LSR | MSR | SCR | DLL | DLM |
| 0 | Data bit 0 | Data bit 0 | ERBFI | 0 if IRQ pend | FIFO enable | WLS0 Bit 0 | DTR | DR | DCTS | Bit 0 | Bit0 | Bit 8 |
| 1 | Data bit 1 | Data bit 1 | ETBEI | INT ID (0) | RCVR FIFO reset | WLS1 Bit 1 | RTS | OE | DDSR | Bit 1 | Bit1 | Bit 9 |
| 2 | Data bit 2 | Data bit 2 | ELSI | INT ID (1) | XMIT FIFO reset | # of stop bits | Out 1 | PE | TERI | Bit 2 | Bit2 | Bit10 |
| 3 | Data bit 3 | Data bit 3 | EDSSI | INT ID (2) 2 | DMA mode select | parity enable (PEN) | Out 2 | FE | DDCD | Bit 3 | Bit3 | Bit11 |
| 4 | Data bit 4 | Data bit 4 | 0 | 0 | resvd | Even Parity Select (EPS) | Loop | BI | CTS | Bit 4 | Bit4 | Bit12 |
| 5 | Data bit 5 | Data bit 5 | 0 | 0 | resvd | Stick parity | 0 | THRE | DSR | Bit 5 | Bit5 | Bit13 |
| 6 | Data bit 6 | Data bit 6 | 0 | FIFO enable2 | RCVR Trig (LSB) | Set break | 0 | TEMT | RI | Bit 6 | Bit6 | Bit14 |
| 7 | Data bit 7 | Data bit 7 | 0 | FIFO enable2 | RCVR Trig (MSB) | DLAB | 0 | RCVR FIFO Err 2 | DCD | Bit 7 | Bit7 | Bit15 |

Table 4. UART Registers.

NOTES:

1	Bit 0 is the least significant bit. It is the first bit serially transmitted or 
	received.

2	Interrupt ID Bit (2): These bits are always 0 in NS16450 mode

The following abbreviations are used for the bits in the table:

ERBFI	Enable Received Data Available Interrupt

ETBEI	Enable Transmitter Holding Register Empty Interrupt

ELSI	Enable Receive Line Status Interrupt

EDSSI	Enable Modem Status Interrupt

WLS0	Word Length Select Bit 0

WLS1	Word Length Select Bit 1

DLAB	Divisor Latch Access Bit

DTR	Data Terminal Ready

RTS	Request To Send

DR	Data Ready

OE	Overrun Error

PE	Parity Error

FE	Framing Error

BI	Break Interrupt

THRE	Transmitter Holding Register

TEMT	Transmitter Empty

DCTS	Delta Clear to Send

DDSR	Delta Data Set Ready

TERI	Trailing Edge Ring Indicator

DDCD	Delta Data Carrier Detect

CTS	Clear To Send

DSR	Data Set Ready

RI	Ring Indicator

DCD	Data Carrier Detect

The following abbreviations are used for the registers:

RBR	Receiver Buffer Register (Read Only)

THR	Transmitter Holding Register (Write Only)

IER	Interrupt Enable Register

IIR	Interrupt Identification Register (Read Only)

FCR	FIFO Control Register (Write Only)

LCR	Line Control Register

MCR	Modem Control Register

LSR	Line Status Register

MSR	Modem Status Register

SCR	Scratch Register

DLL	Divisor Latch (LS)

DLM	Divisor Latch (MS)

## AT Commands

AT commands are issued from an asynchronous DTE, such as a PC, and control the modem’s operating and software configuration. The following section lists all AT commands supported by the AT&T HSM Complete Chip Set. The defaults for each command are in bold.

### Initialization Strings

Initialization strings are used to initialize the modem before beginning to dial a number. One of the most common is to return the modem to the factory default values:

AT&F&W0

This command loads the factory default settings stores them as the active user profile #0 into one of the two nonvolatile stored profile memory locations. Other commands can be used in tandem, in addition to, or mixed.

## Basic AT Commands

### A/	Repeat Last Command

This command re-executes last command string (not to be preceded with AT or followed by pressing the Return key.)

### A	Answer Command

This command instructs the modem to go off-hook and attempt to establish a connection without waiting for a ring.

### Bn	Communication Standard Option

This command is used to configure the modem to the desired communications standard settings at particular line speed.

B0	Selects CCITT V.22 when modem is at 1200 bits/s

Selects CCITT V.21 when modem is at 300 bits/s

B1	Selects Bell 212A when modem is at 1200 bits/s

Selects Bell 103 when modem is at 300 bits/s

B2	Selects V.23 originate mode receive 1200 bits/s, transmit 75 		bits/s;

Answer mode receive 75 bits/s, transmit 1200 bits/s

B3	Selects V.23 originate mode receive 75 bits/s, transmit 1200 		bits/s;

answer mode receive 1200 bits/s, transmit 75 bits/s

B15	Selects CCITT V.21 when the modem is at 300 bits/s

B16	Selects Bell 103 when the modem is a 300 bits/s

B30	Selects V.22bis when the modem is at 4800 bits/s

B41	Selects V.32 full duplex when the modem is at 4800 bits/s

B52	Selects V.32bis full duplex when the modem is at 7200 bits/s

B60	Selects V.32 full duplex when the modem is at 9600 bits/s

B70	Selects V.32bis full duplex when the modem is at 12000 bits/s

B75	Selects V.32bis full duplex when the modem is at 14400 bits/s

Result Codes:

OK	n=0, 1, 2, 3, 15, 16, 30, 41, 52, 60, 70, 75.

Error	Otherwise.

### Cn	Carrier Control Option (dummy command)

This command is used by some modems to control the transmit carrier. This chip set does not support C0 and will respond in error if this command is given. This chip set will accept C1 without error in order to ensure backward compatibility with communications software that issues this command.

C0	Transmit carrier always off (returns ERROR).

C1	Normal transmit carrier switching.

Result Codes:

OK	n=1.

Error	Otherwise.

### Dn	Dial Command

This command begins the dialing sequence. The string n (modifiers and telephone number) is entered after the D command.

Dial modifiers:

P	Pulse dial. Only digits 0-9 can be dialed in pulse dial mode.

R	Reverse dial mode. Originates call in answer mode.

S=n	Dial stored number. Dials the number stored in location n (0-3).

T	Tone DTMF dial. Any digit 0-9, *, #, A,B,C,D may be dialed

W	Wait for dial tone. Modem waits for dial tone before processing

the dial string.

,	Pause. Causes modem to pause before processing the next character

in the dial string.

!	Flash hook. This causes the modem to go on-hook for 0.5 s then 		return off-hook.

@	Wait for quiet answer. Wait for five seconds of silence after dialing

the number.

;	Return to idle state. Modem returns to command mode after dialing 		a number without disconnecting the call.

### En	Off-Line Echo Command

This command determines whether characters are echoed to the DTE from the modem when it is in the command state.

E0	Echo disabled.

E1	Echo enabled.

Result codes:

OK	n=0,1.

Error	Otherwise.

### Fn	On-Line Echo Command (dummy command)

This command is used by some modems to determine whether characters are echoed to the DTE from the modem when it is in the on-line state. This chip set does not support F0 and will respond in error if this command is given. This chip set will accept F1 without error in order to ensure backward compatibility with communications software that issues this command.

F0	Returns Error.

F1	On-line echo disabled.

Result Codes:

OK	n=1

Error	Otherwise

### Hn	Switch-hook Control

This command provides control over the line relay.

H0	Modem goes on-hook.

H1	Modem goes off-hook.

Result Codes:

OK	n=1.

Error	Otherwise.

### In	Identification/Checksum Option

This command instructs the modem to query its memory for information about itself.

I0	Reports product code.

I1	Reports a 3-digit ROM checksum

I2	Performs a ROM check.

I3	Report ROM components

I4	Modem capabilities.

Result Codes:

OK	n=0, 1, 2, 3, 4.

Error	Otherwise.

### Ln	Speaker Volume

This command determines the volume level of the speaker, when supported by the modem.

L0	Lowest speaker volume.

L1	Low speaker volume.

L2	Medium speaker volume.

L3	High speaker volume.

Result Codes:

OK	n=0, 1, 2, 3.

Error	Otherwise.

### Mn	Speaker Control

This command determines whether the speaker function of the modem is on or off.

M0	Speaker always off.

M1	Speaker on until carrier present.

M2	Speaker always on.

M3	Speaker off during dialing and on until carrier.

Result Codes:

OK	n=0, 1, 2, 3.

Error	Otherwise.

### Nn	Select Negotiate Handshake

This command controls automode enable/disable.

N0	When originating or answering, handshake only at the 			communication standard specified by S register 37 and Bn.

N1	When originating or answering, start handshake only at the 		communication standard specified by S register 37 and Bn. 		During handshake, fallback to a lower speed may occur.

Result Codes:

OK	n=0,1.

Error	Otherwise.

### On	Go On-Line

This command returns the modem to on-line mode or data mode.

O0	Returns modem to on-line mode or data mode.

O1	Begins an equalizer retrain sequence, then returns to on-line state.

O3	Issues rate renegotiation sequence, then returns to on-line state for 		V32bis only.

Return Codes:

OK	n=0,1,3

Error	Otherwise

### P	Enable Pulse Dialing

This command instructs the modem to use pulse dialing. Dialed digits will be pulsed until a T command or dial modifier is received.

Result Codes:

OK

### Qn	Results Code Display Option

This command controls whether the result codes are displayed to the DTE.

Q0	Result codes enabled.

Q1	Result codes disabled.

Result Codes:

OK	n=0,1

Error	Otherwise

### Sn	Selects S Register

This command selects a specific S register.

n = 0 ~ 112

### Sn=x	Writes to an S Register

This command writes a value to a specified register.

n = 0 ~ 112

x = 0 ~ 255

### Sn? 	Read an S Register

The contents of S register n are sent to the DTE as three decimal digits. This informational text response is formatted with <CR><LF> as determined by the V command currently in effect.

n = 0 - 112

### T	Enable Tone Dialing

This command instructs the modem to send DTMF tones while dialing. Dialed digits will be tone dialed until a P command or dial modifier is received.

Result Codes:

OK

### Vn	Result Code Form

This command determines whether result codes (including call progress and negotiation progress messages) are displayed as numbers or words.

V0	Numeric form.

V1	Verbose form.

Result Codes:

OK	n=0,1.

Error	Otherwise.

### Wn	Select Extended Result Codes

This command determines which result codes will be used to describe the type of connection and protocol that resulted from handshaking and negotiation.

W0	CONNECT result code reports DTE speed; if S register 95 = 0, 		disable all extended result codes.

W1	CONNECT result code reports DTE speed; enable the CARRIER 		and PROTOCOL extended result code based on S95.

W2	CONNECT result code reports DCE speed; if S register 95 = 0, 		disable all extended result codes.

Result Codes:

OK	n=0,1,2.

Error	Otherwise.

### Xn	Select Result Codes/Call Progress Options

This command enables tone detection options used in the dialing process. As these functions are chosen, the modem chip set’s result codes are also affected. This command is therefore frequently used to control the modem chip set’s responses. The primary function of this control is to control the modem chip set’s call response capabilities.

X0	Result codes 0-4 enabled. Busy and dial tone detect disabled.

X1	Result codes 0-5, 18-88 enabled. Busy and dial tone detect 			disabled.

X2	Result codes 0-6, 8-88 enabled. Busy detect disabled and dial tone

detect enabled.

X3	Result codes 0-5, 7-88 enabled. Busy detect enabled and dial tone 		detect disabled.

X4	All Result Codes 0-88 enabled. Busy and dial tone detect 			enabled.

Result Codes:

OK	n=0,1,2,3,4.

Error	Otherwise.

### Yn	Long Space Disconnect

This command determines if the modem will disconnect a call upon receiving a long space (1.6 sec break) signal. or transmit a 4.0 s space when initiating a disconnect.

Y0	Disables long space disconnect.

Y1	Enables long space disconnect.

Result Codes:

OK	n=0,1.

Error	Otherwise.

### Zn	Recall Stored Profile

This command instructs the modem chip set to go on hook and restore the selected stored profile. Any non-storable parameters previously set are returned to their factory settings.

Z0	Reset and recall user profile 0.

Z1	Reset and recall user profile 1.

Result Codes:

OK	n=0,1.

Error	Otherwise.

## AT& (Ampersand) Commands

### &Bn	V.32 Autoretrain

This command controls the modem action when the threshold set by S108 is encountered.

&B0	Hang up if threshold reached.

&B1	Retrain if threshold reached.

&B2	Do not hang up and do not retrain (i.e., tolerate any line).

Result Codes:

OK	n=0,1,2

Error	Otherwise

### &Cn	Data carrier Detect Option

This command controls how the state of the DCD or RLSD (receive line signal detect) relates to the carrier from the remote modem.

(If Q0 is selected)

&C0	State of carrier from remote modem is ignored. RLSD circuit is 		always on.

&C1	State of carrier from remote modem is tracked. RLSD circuit 			reflects the state of carrier

(If &Q1, &Q2, &Q3, or &Q4 are selected)

&Cn	has no effect. RLSD shows current state of carrier.

Result Codes:

OK	n=0,1.

Error	Otherwise.

### &Dn	Data Terminal Ready Option

This command interprets how the modem will respond to the state of the DTR signal and changes to the DTR signal.

&D0	DTR ignored.

&D1	Go to command mode on on-to-off DTR transition.

&D2	Hang up and go to command mode on on-to-off DTR 			transition. Autoanswer is disabled if DTR is low.

&D3	Reset on on-to-off DTR transition.

Result Codes:

OK	n=0,1,2,3.

Error	Otherwise.

### &F	Load Factory Settings

This command loads the configuration stored and programmed at the factory. This operation replaces all of the command options and S register settings in the active configuration with factory values stored in nonvolatile RAM.

&F	Recall factory settings as active configuration.

Result Codes:

OK	n=0.

Error	Otherwise.

### &Gn	Guard Tone Option

This command determines which guard tone, if any, to transmit while transmitting in the high band (answer mode). This command is only used in V22 and V22bis mode.

&G0	No guard tones.

&G1	550 Hz guard tone.

&G2	1800 Hz guard tone.

Result Codes:

OK	n=0,1,2.

Error	Otherwise.

&Kn	Select Flow Control

This command selects the flow control method the modem chip set provides to the DTE to prevent the modem’s buffer from overflowing with data. A data buffer holds the data until the modem is ready to transmit it. When the data buffer is full, flow control instructs the DTE to stop sending to the modem while the modem continues to send characters.

&K0	Disable flow control.

&K1	Reserved.

&K2	Reserved.

&K3	Enable bidirectional hardware flow control (RTS/CTS).

&K4	Enable bidirectional XON/XOFF flow control.

&K5	Reserved.

Result Codes:

OK	n=0,3,4

Error	Otherwise

### &Ln	Dial Up./Lease Line Option

This command is used to select between dial-up operations or leased-line operations.

&L0	Dial Line.

&L1	Leased Line.

Result Codes:

OK	n=0,1.

Error	Otherwise.

### &Mn	Asynchronous Mode

This command is used to select the communication mode.

On-Line State	Idle State

&M0	Async	Normal

&M1	Sync	Normal

&M2	Sync	Dial when DTR = 1

&M3	Sync	Talk/Data, DTR = 0/1

Result Codes:

OK	n=0,1,2,3

Error	Otherwise

### &Pn	Dial Pulse Ratio

This command determines the make/break ratio used during pulse dialing.

&P0	Make = 39%, Break = 61% for use in the United States.

&P1	Make = 33%, Break = 66% for use in some other countries.

Result Codes:

OK	n=0,1.

Error	Otherwise.

### &Qn	Communication Mode Option

This command will select the communication mode. It determines how the chip set will treat transmit and receive data while in the on-line state and establishes certain call setup procedures.

On-Line State	Idle State

&Q0	Async	Normal

&Q1	Sync	Normal

&Q2	Sync	Dial when DTR = 1, Hang up when DTR = 0

&Q3	Sync	Talk/Data, DTR = 0/1

&Q4	AutoSync if applicable

Result Codes:

OK 	n=0,1,2,3,4.

Error	Otherwise.

### &Rn	CTS/RTS Option (non-CTS flow control mode)

The functions of CTS and RTS are determined by this command.

&R0	CTS follows RTS (synchronous modes only).

&R1	CTS always true.

Result Codes:

OK	n=0,1.

Error	Otherwise.

### &Sn	Data Set Ready Option

This command controls the functions of DSR. DSR indicates when the modem is connected to a communications channel and is ready.

(If &Q0 is selected)

&S0	DSR circuit always on.

&S1	DSR circuit on during handshaking, and in test modes/off in idle 		mode and when carrier is lost.

(If &Q1, &Q2, &Q3 or &Q4 is selected)

DSR circuit on during handshaking, off in test or idle mode.

Result Codes:

OK	n=0,1

Error 	Otherwise

### &Tn	Self-Test Commands

These commands are used to configure and place the modem chip set in various test modes as defined by CCITT V.54. These tests are typically used to verify the analog and digital portions of a modem’s operation.

&T0	Terminate test.

&T1	Local analog loopback.

O = Low-frequency band.

A = High-frequency band.

&T3	Local activated Remote Digital Loopback (RDL).

&T4	Grant request from remote for RDL test.

&T5	Deny request from remote for RDL test.

&T6	Remote digital loopback test.

&T7	Remote digital loopback with self-test.

&T8	Local analog loopback with self-test.

Result Codes:

OK		n=0,3,4,5,7,8.

CONNECT	n=1,6.

Error		Otherwise.

### &Un	Disable Trellis Coding

This command is used to enable and disable trellis coding for V.32 9600 bps connections only.

&U0	Enable trellis coding.

&U1	Disable trellis coding.

Result Codes:

OK	n=0,1.

Error	Otherwise.

### &Vn	View Active Configuration and Stored Profile

This command is used to display the active and stored profiles (commands and S register settings) along with any stored telephone numbers. The following example illustrates typical &V information displayed for the AT&T HSM Complete Modem Chip Set.

Example:

ACTIVE PROFILE

B1 B116 B30 B41 B60 E1 L2 M1 N1 P Q0 V1 W0 X4 Y0 &B1 &C1 &D2 &G0 &J0

&K3 &L0 &P0 &Q0 &R0 &S0 &U0 &X0 &Y0

%A013 %C1 %E1 %G0 %01 \A3 \C0 \G0 \J0 \K5 \N3 \Q3 \T000 \V1 \X0 -J1 *H3 *O016

S00:000 S01:000 S02:043 S03:013 S04:010 S05:008 S06:002 s07:030

S08:002 S09:006 S10:014 S11:095 S12:050 S18:000 S25:005 S26:001

S37:000 S112:000

STORED PROFILE 0

B1 B16 B30 B41 B60 E` L2 M1 N1 P Q0 V1 W0 X4 Y0 &B1 &C1 &D2 &G0 &J0

&K0 &L0 &P0 &Q0 &R0 &S0 &U0 &X0

%A013 %C1 %E1 %G0 %01 \A3 \C0 \G0 \J0 \K5 \N3 \Q3 \T000 \V0 \X0 -J1 *H3 *O016

S00:000 S02:043 S06:002 S07:030 S08:002 S09:006 S10:014 S11:095

S12:050 S18:000 S25:000 S26:001 S37:000 S112:000

TELEPHONE NUMBERS:

&Z0 =		&Z1 =

&Z2 =		&Z3 =

Result Codes:

OK	n=0,1.

Error	Otherwise

### &Wn	Store Active Profile.

This command stores certain command options and S register values into one of the two nonvolatile stored profile memory locations.

&W0	Save active configuration as user profile 0.

&W1	Save active configuration as user profile 1.

Result Codes:

OK	n= 0 or 1

Error	Otherwise

### &Xn	Synch Transmit Clock Source Option

This command determines how the DTE transmit clock is generated while the modem is in synchronous modes.

&X0	The modem generates the transmit clock and applies it to pin 15 		(RS232)

&X1	The DTE generates the transmit clock on pin 24 (RS232) and the 		modem applies it to pin 15 (RS232).

&X2	The modem derives the transmit clock from the receive carrier 			signal and applies it to pin 15 (RS232).

Result Codes:

OK	n=0,1,2

Error	Otherwise

### Yn	Select Stored Profile on Powerup

This command selects which user profile will be recalled into the active profile (powerup).

&Y0	Recall user profile 0 on powerup.

&Y1	Recall user profile 1 on powerup.

Result Codes:

OK	n=0,1.

Error	Otherwise.

### Zn=x	Stored Telephone Number

This command is used to store up to four telephone numbers. The maximum number of digits per number is 36.

x = 0-9, A, B, C, D, #, *, T, P, R, W, @, comma, !, ; (dial string).

n = 0-3.

## AT% Commands

### %An	Set Auto-reliable Fallback Character

This command allows the user to specify the character that tells the modem to return (or fall back) from auto-reliable (MNP) mode to a normal connection on the answering modem.

Result Codes:

OK	n = 0-127 (ASCII character).

Error	Otherwise.

Default	013.

### %Cn	Data Compression

This command controls the MNP Class 5. The modem can only perform data compression on an error corrected link.

%C0	No compression.

%C1	MNP Class 5 compression.

Result Codes:

OK	n=0,1.

Error	Otherwise.

### %Dn	108 Mode Select

This command selects the 108 mode to user for V.25 bis.

0 = 	108/2 mode (default).

1 =	108/1 mode.

### %Fn	Character Framing Select

This command selects the V.25bis character framing.

0 = 	Asynchronous

1 =	HDLC

2 =	BISCYNC 0 = Default

### %En	Enable/Disable Autoretrain

This command controls whether or not the modem will automatically monitor the line quality and request a retrain when necessary.

%E0	Disable autoretrain

%E1	Enable autoretrain.

Result Codes:

OK	n=0,1

Error	Otherwise

### %Gn	Auto Fallback Enable (V.32bis only)

This command selects the auto fallback option.

%G0	Disabled.

%G1	Enabled.

Result Codes:

OK	n=0,1.

Error	Otherwise.

### %O	Escape Method

This command controls the escape method.

n = 0	Disables escape.

n = 1	+++AT method enabled.

n = 2	<BREAK> AT method.

n = 3	Enables both +++AT and <BREAK>AT.

### %V	Command Set Switch

This command switches from the AT command set to V.25bis when entered in V.25bis Async only.

## AT\ (back slash) commands

### \An	Block Size

This command sets the maximum block size during a MNP and LAPM connection. This option forces the modem chip set to transmit smaller blocks of data. This command is typically used for poor quality telephone lines where transmitting smaller blocks of data avoids retransmissions, thus improving throughput.

MNP				LAPM

\A0	Max 64 characters	Max 64 characters

\A1	Max 128 characters	Max 128 characters

\A2	Max 192 characters	Max 128 characters

\A3	Max 256 characters	Max 128 characters

Result Codes:

OK	n=0,1,2,3.

Error	Otherwise.

### \Bn	Transmit Break

This command sends a break to the remote modem. It is equivalent to using the <BREAK> key on your terminal. This command can be used if your keyboard does not have a <BREAK> key.

\B1 - \B9	Break length in 100 ms.

Default = 0	Break of 300 ms is transmitted.

### \Cn	Set Autoreliable Buffer

This command determines whether the modem chip set buffers the data it receives during link negotiation.

\C0	Does not buffer data.

\C1	Buffers data on answering modem for 4 seconds, until 200 			characters have been buffered, and then switches to normal mode 		and the buffered characters are passed to the serial port. If a SYN 		character is detected, reliable mode is attempted.

\C2	Does not buffer data on answering modem. Switches to normal 		mode upon receipt of autoreliable character and passes it to serial 		port.

Result Codes:

OK	n=0,1,2.

Error	Otherwise.

### \Gn	Set modem Port Flow Control

This command selects the modem chip set’s DE or port flow control. If the modem chip set is receiving data from the remote modem faster than it can be processed, enabling port flow control allows the modem chip set to handle the data efficiently.

\G0	Disable port flow control.

\G1	Set port flow control to XON/XOFF flow control.

Result Codes:

OK	n=0,1.

Error	Otherwise.

### \Jn	Bits/s Rate Adjust

This command allows the modem chip set and the DTE to exchange asynchronous data up to 57,600 bits/s, regardless of the speed of the data connection between the modem chip set and the remote modem.

\J0	Turn off feature.

\J1	Turn on feature.

Result Codes:

OK	n=0,1.

Error	Otherwise.

### \Kn	Set Break Control

This command selects the way in which the modem handles a break signal. This modem chip set can handle a break signal in one of three ways: expedited destructive, expedited, or queued break.

(In command state, if reliable mode, then transmit break, to remote)

0,1	Destructive/Expedited

2,3	Nondestructive/Expedited

4,5	Nondestructive/Nonexpedited

(In connect state, if direct mode then receive break at serial port)

0,2,4	Immediately send break and enter command state

1,3,5	Immediately send break through

(In connect state, receive break at modem port, send to serial port)

0,1	Destructive/Expedited

2,3	Nondestructive/Expedited

4,5	Nondestructive/Nonexpedited

Default	= 5

Result Codes:

OK	n=0,1,2,3,4,5

Error	Otherwise

### /Nn	Set Operating Mode

This command selects the type of data connedt5ion the modem chip set can establish.

Classic Feature			Gold Feature

\N0	Normal Mode		Normal Mode

\N1	Direct Mode		Direct Mode

\N2	MNP Reliable Mode	MNP Reliable Mode

\N3	V.42 Autoreliable Mode	MNP Autoreliable Mode

\N4	V.42 Reliable Mode	LAPM Reliable Mode

\N5				LAPM Autoreliable Mode

\N6				V.42 Reliable Mode

\N7				V.42 Autoreliable Mode

Result Codes:

OK	n=0,1,2,3,4.

Error	Otherwise.

### \Qn	Set Serial Port Flow Control

This command selects the flow control method the modem chip set provides to the DTE to prevent the modem’s buffer from overflowing with data. A data buffer holds the data until the modem is ready to transmit it. When the data buffer is full, flow control instructs the DTE to stop sending to the modem while the modem continues to send characters.

\Q0	Disables flow control.

\Q1	Enable bidirectional XON/XOFF flow control.

\Q2	Enable unidirectional hardware flow control.

\Q3	Enable bidirectional hardware flow control.

Result Codes:

OK	n=01,2,3.

Error	Otherwise.

### \Tn	Set Inactivity Timer

This command allows the modem chip set to disconnect if no data activity has occurred for the selected period of time.

n = 0 - 90	Length in minutes.

Default		000.

### \Vn	Modify Result Code Form

This command option selects the extended MNP/V.42 CONNECT responses.

\V0	Enable codes defined by ATV command.

\V1	Enable modified MNP codes.

Result Codes:

OK	n=0,1.

Error	Otherwise.

### \Xn	Set XON/XOFF Pass-through

This command allows or prevents the modem chip set from passing XON and XOFF characters through to the remote system.

\X0	Process flow control characters.

\X1	Process flow control characters and pass them through to the local 		or remote so they can process the characters.

Result Codes:

OK	n=0,1

Error	Otherwise

## AT- (Dash) Commands

### -Cn	Set Calling Tone in Data Mode

This command instructs the modem to generate a FAX calling tone when originating a data call.

-C0	Disables calling tone.

-C1	Enables calling tone.

Result Codes:

OK	n=0,1.

Error	Otherwise.

### -Jn	Set V.42 Detect Phase

This command determines whether the originating modem sends a sequence to the answering modem to determine the type of error correction it has enabled.

-J0	Disables the V.42 detect phase.

-J1	Enables the V.42 detect phase.

Result Codes:

OK	n=0,1.

Error	Otherwise.

### -Kn	MNP Extended Services

This command determines whether a LAPM connection will be converted to an MNP connection. This will occur only if AT-K1 is set on both sides.

-K0	Disables MNP extend services.

-K1	Enables MNP extend services.

Result Codes:

OK	n=0,1

Error	Otherwise

### -Qn	Fallback Modulation Speeds (MNP 10)

This command controls whether the modem can fall back from a V.32 or V.32bis MNP reliable connection to 2400 bits/s or 1200 bits/s.

-Q0	Disables fallback to both V.22bis (2400) and V.22 (1200)

-Q1	Enables fallback to V.22bis (2400)

-Q2	Enables fallback to V.22bis (2400 and V.22 (1200)

Result Codes:

OK	n=0,1,2

Error	Otherwise

### -V	Display Number

This command will display “Modem Firmware Version XXXX.”

## AT” (Double-quote) Commands

### “Hn	V.42bis Compression Control

This command controls V.42bis data compression over an error correction link.

“H0	Disable V.42bis

“H1	Enable V.42bis only when transmitting data

“H2	Disable V.42bis only when receiving data

“H3	Enable V.42bis for both directions.

Result Codes:

OK	n=0.1.2

Error	Otherwise

### “On	n = 6 - 256

This command selects the string size for V.42bis compression.

Default	= 16

### “Mn	MNP 10 Control

This command enables and disables MNP1 10 protocol for impaired or cellular control.

“M0	Disables MNP 10 protocol.

“M1	Enables MNP 10 protocol.

Result Codes:

OK	n=0,1

Error	Otherwise

## S Registers

The S registers are summarized below. Registers denoted with an asterisk (*) may be stored in one of the two user profiles by entering the &Wn command. One of these profiles may be loaded at any time by using the Zn command. Registers or register fields quoted as “reserved” are reserved for current or future use by the firmware, or are permanently overridden by PTT limitations.

All bit-mapped registers are read-only. The appropriate AT command which controls the relevant bits in the S register should be used to change the value.

The factory default values are stored in ROM and are loaded into the active configuration at powerup or by the ATZn command. In addition, the designated default profile can be changed by entering the &Yn command where n is one of the two possible user profiles.

All of the factory default values may be loaded at any time by entering the &F command. All factory defaults are for the 9.6 Data/FAX, 14.4 Data/FAX, 19.2 Data/FAX, 14.4 Data/FAX/Cell, and 14.4 Data/FAX/Voice unless otherwise specified.

### S0 	Ring to Autoanswer On

Sets the number of the rings required before the modem automatically answers a call. Setting this register to zero disables autoanswer mode.

Range:	0 - 255

Default:	000

Units:	Rings

### S1 	Ring Count

Sets the number of rings required before the modem answers a call. If no rings occur over an eight second interval, this register is cleared.

Range:	0 - 255

Default:	000

Units:	Rings

### S2	Escape Character

S2 holds the decimal value of the ASCII character used as the escape character. The default value corresponds to an ASCII +. A value over 127 disable the escape process, i.e., no escape character will be recognized.

Range:	0 - 255

Default:	043

Units:	ASCII

### S3	Carriage Return <CR> Character

Sets the command line and result code terminator character. Pertains to asynchronous operation only.

Range:	0 - 127, ASCII decimal

Default:	013 (carriage return)

Units:	ASCII

### S4	Line Feed <LF> Character

Sets the character recognized as a line feed. Pertains to asynchronous operation only. The line feed control character is output after the carriage return control character if verbose result codes are used.

Range:	0 - 127, ASCII character

Default:	010 (Line Feed)

Units:	ASCII

### S5	Back Space <BS> Character

Sets the character recognized as a backspace. Pertains to asynchronous operation only. The modem will not recognize the Backspace character if it is set to a value that is greater than 32 ASCII. This character can be used to edit a command line. When the echo command is enabled, the modem echoes back to the local DTE the Backspace character, an ASCII space character, and a second backspace character;  this means a total of three characters are transmitted each time the modem processes the Backspace character.

Range:	0 - 127, ASCII decimal

Default:	008 (Backspace)

Units:	ASCII

### S6 Wait Before Dialing

Sets the length of time, in seconds, that the modem must wait (pause) after going off-hook before dialing the first digit of the telephone number. The modem always pauses for a minimum of 2 seconds, even if the value of S6 is less than 2 seconds. The Wait for Dial Tone call progress feature (W dial modifier in the dial string) will override the value in register S6. This operation, however, may be affected by some ATX options according to country restrictions.

Range:	2 - 255

Default:	002

Units:	seconds

### S7	Wait for Carrier

Sets the time, in seconds, that the modem must wait before hanging up because a carrier is not detected. The timer is started when the modem finishes dialing (originate) or goes off-hook (answer). In originate mode, the timer is reset upon detection of an answer tone if allowed by country restrictions. This timer also specifies the wait for silence time for the @ dial modifier in seconds. S7 is not associated with the W dial modifier.

Range:	1 - 255

Default:	030 (090 for MNP 10)

Units:	Seconds

### S8	Pause Time for Command or Dial Modifier

Sets the time, in seconds, that the modem must pause when the *,* dial modifier is encountered in the dial string.

Range:	0 - 255

Default:	002

Units:	Seconds

### S9	Carrier Recovery Time

Sets the time, in tenths of a second, that the carrier must be present before the modem considers it valid and turns on RLSD. As this time is increased, there is less chance to detect a false carrier due to noise from the telephone line.

Range:	1 - 255

Default:	006 (0.6 seconds)

Units:	0.1 seconds

### S10 	Lost Carrier Hang Up Delay

Sets the length of time, in tenths of a second, that the modem waits before hanging up after a loss of carrier. This allows for a temporary carrier loss without causing the local modem to disconnect. When register S10 is set to 255, the modem functions as if a carrier is always present.

The actual interval the modem waits before disconnecting is the value in register S10 minus the value in register S9. Therefore, the S10 value must be greater than the S9 value or else the modem disconnects before it recognizes the carrier.

Range: 	1 - 255

Default:	014 (14 seconds)

Units:	0.1 seconds

### S11	DTMF Dialing Speed

Range:	50 - 255

Default:	095

Units:	0.001 seconds

### S12	Guard Time

Defines the maximum period, in 2-hundredths of a second, allowed between consecutive asynchronous escape characters (+) for the escape sequence to be considered valid.

Range:	0 - 255

Default:	050 (1 second)

Units:	0.02 seconds

### S14	Bit Mapped Options

Indicates the status of command options.

Default:	(AAh), (176d), (10101010b)

Bit 0	Reserved

Bit 1	Command echo (En)

0	E0 is selected

1	E1 is selected *

Bit 2	Quite Mode (Qn)

0	Q0 is selected*

1	Q1 is selected

Bit 3	Result codes (Vn)

0	V0 is selected

1	V1 is selected *

Bit 4	Dumb Mode

0	dumb mode off (normal)*

1	dumb mode on

Bit 5	Tone (T) Pulse (P)

0	T is selected

1	P is selected

Bit 6	Reserved

Bit 7	Originate/Answer

0	answer

1	originate *

### S16	Modem Test Options (read only)

Indicates the test in progress status.

Default:	000

Bit 0	Local analog loopback

0	local ALB disabled *

1	local ALB enabled (&T1)

Bit 1	Reserved

Bit 2	Local digital loopback

0	local DL disabled *

1	local DL enabled (&T3)

Bit 3	Remote digital loopback (RDL) status

0	RDL off *

1	RDL on

Bit 4	RDL requested (AT&T6)

0	RDL not active *

1	RDL request from distant end is in service

Bit 5	RDL w/ self-test

0	RDL w/ self-test disabled *

1	RDL w/ self-test enabled (&T7)

Bit 6	Local analog loopback (ALB) w/ self-test

0	ALB w/ self-test disabled *

1	ALD w/ self-test enabled (&T8)

Bit 7	Reserved

### S 18 	Modem Test Timer

Sets the length of time, in seconds, that the modem conducts a test (commanded by &Tn) before returning to the command mode. If this register value is zero, the test will not automatically terminate;  the test must be terminated from the command mode by issuing an &T0 or H command.

Range:	0 - 255 seconds

Default:	000

### S19 	Autosync Protocol Support Mode

Bit 1	0	Bisync protocol *

1	SDLC protocol

Bit 2	0	SDLC address detect disabled *

1	SDLC address detect enabled

Bit 3	0	NRZI bit encoding *

1	NRZ bit encoding

Bit 4	0	Selects mark idle *

1	Selects flag or sync idle

### S20	Autosync Protocol Support Mode

In SDLC mode:	SDLC address

In Bisync mode:	definition of Bisync sync character

### S21	Bit-Mapped Options

Indicates the status of command options.

Default:	(30h), (048d), (00110000b)

Bit 0	Reserved

Bit 1	Reserved

Bit 2	CTS behavior

0	&R0 is selected*

1	&R1 is selected

Bits 4,3	DTR behavior (&Dn)

00	&D0 is selected

01	&D1 is selected

10	&D2 is selected *

11	&D3 is selected

Bit 5	RLSD (DCD) behavior (&Cn)

0	&C0 is selected

1	&C1 is selected *

Bit 6	DSR behavior (&Sn)

0	&S0 is selected *

1	&S1 is selected

Bit 7	Long space disconnect (Yn)

0	Y0 is selected *

1	Y1 is selected

### S22	Bit-Mapped Options

Indicates the status of command options.

Default:	(76h), (118d), (01110110b)

Bits 0,1	Speaker Volume (Ln)

00	L0 is selected

01	L1 is selected

10	L2 is selected *

11	L3 is selected

Bits 2,3	Speaker Control (Mn)

00	M0 is selected

01	M1 is selected *

10	M2 is selected

11	M3 is selected

Bits 6,5,4  Limit Result Codes (Xn)

000	X0 is selected

001	Reserved

010	Reserved

011	Reserved

100	X1 is selected

101	X2 is selected

110	X3 is selected

111	X4 is selected *

Bit 7	Dial Pulse Ratio (&Pn)

0	&P0 is selected *

1	&p1 is selected

### S23 	Bit Mapped Options

Indicates the status of the command options.

Default:	0Fh), (015d), (00001111b)

Bit 0	Grant PDL

0	&T5 is selected

1	&T4 is selected *

Bits 3,2,1 Assumed DTE rate

000	0 - 600 bits/s

001	600 bits/s

010	1200 bits/s

011	2400 bits/s

100	4800 bits/s

101	9600 bits/s

110	19.2 Kbits/s

111	38.4 Kbits/s or 57.6 Kbits/s *

Bits 5,4	Assumed DTE Parity

00	Even parity *

01	space parity/no parity

10	odd parity

11	mark

Bits 7,6	Guard Tone (&Gn)

00	&G0 is selected *

01	&G1 is selected

10	&G2 is selected

11	Reserved

### S25 	Detect DTR Change

Sets the length of time that the modem will ignore DTR for before hanging up. Its units are seconds for synchronous mode 1 and one hundredths of a second for other modes.

Range:	0 - 255 (1 second for synchronous mode 1; 0.01 second otherwise)

Default:	005

### S26	RTS to CTS Delay Interval

Sets the time delay, in hundredths of a second, before the modem turns CTS ON after detecting an OFF-to-ON transaction on RTS when &R0 is commanded. Pertains to synchronous operation only.

Range:	0 - 255

Default:	001

Units:	0.01 seconds

### S27	Bit-Mapped Option

Indicates the status of command options.

Default:	(40h), (064d), (01000000b)

Bit 3, 1, 0 Synchronous/asynchronous selection (&Qn)

000	&Q0 is selected *

001	&Q1 is selected

010	&Q2 is selected

011	&Q3 is selected

100	&Q4 is selected

Bit 2	Leased Line control (&Ln)

0	&L0 is selected *

1	&L1 is selected

Bits 5,4	Internal Clock Select (&Xn)

00	&X0 is selected *

01	&X1 is selected

10	&X2 is selected

11	&X3 is selected

Bit 6	CCITT/Bell mode select (Bn)

0	B0 is selected

1	B1 is selected *

Bit 7	Reserved

### S30	Inactivity Timer Value

Range:	0 - 90

Default:	000

Units:	Minutes

### S32	Wait for <CR> Time

Range:	0 - 255

Default:	020

### S37	Desired DCE Speed

0	Attempt to connect at the last AT speed *

3	Attempt a 300 bits/s connection

5	Attempt a 1200 bits/s connection

6	Attempt a 2400 bits/s connection

7	Attempt a 4800 bits/s connection

8	Attempt a 7200 bits/s connection

9	Attempt a 9600 bits/s connection

10	Attempt a 12000 bits/s connection

11	Attempt a 14400 bits/s connection

12	Attempt a 16800 bits/s connection

13	Attempt a 19200 bits/s connection

### S95	Extended Result Code Bit Map

Default:	00h

Bit 0	Verbose CONNECT result code indicates the DCE speed (rather 		than the DTE speed)

Bit 1	Append /ARQ to the verbose CONNECT result code when an error 		control connection is established

Bit 2	Enable the CARRIER result code

Bit 3	Enable the PROTOCOL result code

Bit 4	Reserved

Bit 5	Enable the COMPRESSION result codes

Bit 6	Enable the PROTOCOL result codes 81-88 for MNP connections 		(in place of result code 80)

### S108	Signal Quality Selector

0	No limit

1	Low quality

2	Medium quality

3	High quality

Default:	001

### S109	V.32terbo Carrier Speed Selector

Bit 0	Reserved

Bit 1	4800 bits/s *

Bit 2	7200 bits/s

Bit 3	9600 bits/s

Bit 4	12.0 Kbits/s

Bit 5	14.4 Kbits/s

Bit 6	16.8 Kbits/s (if applicable)

Bit 7	19.2 Kbits/s (if applicable)

### S110	V.32/V.32bis/V.32terbo Selector

0	V.32 enabled

1	V.32bis enabled

2	V.32bis and automatic rate renegotiation enabled

3	V.32terbo enabled

4	V.32terbo and automatic rate renegotiation

Default:	002 (004 for 19.2 Data/FAX)

### S112	DTE Speed Select During Data Transfer

0	Last AT speed *

1	Reserved

2	300 bits/s

3	1200 bits/s

4	2400 bits/s

5	4800 bits/s

6	7200 bits/s

7	9600 bits/s

8	12.0 Kbits/s

9	14.4 Kbits/s

10	16.8 Kbits/s

11	19.2 Kbits/s

12	38.4 Kbits/s

13	57.6 Kbits/s

14	600 bits/s

Default:	000

## V.25bis Commands Set

The following commands are implemented, and parameters applicable to each command are noted following the description of the command.

### CIC	Enable Autoanswer

The modem will go on-line in answer mode canceling any DIC command previously issued. If no incoming call is present, the modem will issue the invalid message.

### CRN	Dial Command

The modem will go on-line, according to the dial string entered, and attempt to establish a connection.

0-9	Digits 0 through 9

A-D	TONE DIALING ONLY

*	The star sign: tone dialing only

#	The pound sign: tone dialing only

T	Select tone dialing: affects current and subsequent dialing

P	Select pulse dialing: affects current and subsequent dialing

<	Short dial pause: period controlled by S8: the modem will pause 		before dialing the digits following “<“

=	Long dial pause: period twice as long as the short dial pause

:	Wait for dial tone: the modem will wait for dial tone before dialing 		the digits following “:”

&	Flash: the modem will go off-line for 0.25 s. then return on line

### CRS	Dial Stored Number

The modem will go on-line, dial according to the dial string entered in the dial string memory addressed, and attempt to establish a connection.

0-2	Dial string memory address

### DIC	Disable Autoanswer

The modem, though configured for autoanswer, will disregard the incoming call. If there is no incoming call or autoanswer is not enabled, the modem will issue the invalid message, the command not being applicable to the subsequent incoming calls.

### PRN	Store Telephone Number

The modem will store the dial string specified into the dial string memory referenced.

0-2	Dial string memory address

:	Separator:  inserted between dial string memory address and dial 		string

x..x	Dial string: A string of dial characters. An empty string clears the 		identification memory referenced. The maximum length of the dial 		string is 18 characters. The following characters are accepted: 			 0-9, A-d, *, #, T, P, <, >, +, :, &.

### RLD	Display Delayed Numbers

The modem will return the list of numbers which have been delayed by country- dependent blacklisting procedures.

NO-	Phone Number	Delay

LSD1	12345		1

### RLN	Display Stored Numbers

The modem will return the dial string or the list of dial strings according to the parameter supplied.

0-2	Dial string memory address:  If no address is supplied, a full set of 		all stored dial strings will be returned.

## Standard V.25bis Indications

The modem provides the indications/responses listed, and parameters applicable to each message are noted following the description of the response.

### CFI	Call Failure Indication

The modem sends this message when a call fails to connect. A parameter is included to give the reason for the failure.

AB	No dial tone or the call abort timer expired with no call progress 		tone detected or during handshaking after answer tone detected

CB	Local circuit busy (phone off-hook)

ET	Busy (engaged tone) detected

FC	Requested number on forbidden call list (call not placed)

NS	No number stored at dial string memory requested (call not placed)

NT	Ringback detected: Ringback stopped:  Call abort timer expired 		with no answer tone detected

RT	Ringback detected: Call abort timer expired with ringback still 		detected

### CNX	Connect

The modem returns this message when a connection has been established. The message may be followed by a value indicating the connection speed (e.g., CNX 9600).

### DLC	Delayed Call

The modem returns this message when a call fails to connect and the number dialed is considered delayed. The message indicates the delay (in minutes) before the call may be reattempted as shown in the following example:

DLCn

This message will be preceded by the message indicating call failure.

### INC	Incoming Call

The modem will send this message when incoming ringing is detected on the line.

### INV	Invalid

The modem will send this message if the command line contains a syntax error, or if it is unable to execute the command.

### LSD	List of Delayed Call Numbers

The modem will return this message in response to the RLD command as shown by the example following:

NO	Phone Number	Delay

LSD 1	7503857609	2

LSD 2	1349579050	1

LSD 3	4146949385	1

LSD 4	3479784564	3

LSD 5	5529685740	1

### LSN	List of Stored Numbers (Dial Strings)

The modem will return this message in response to the RLN command.

LSN 0	Response to RLN

LSN 1

LSN 2

### VAL	Valid

The modem will send this message when it successfully executes the command issued and there is no other applicable response (message or circuit transition).

## Result Codes

The EXM-31 responds to commands with result codes, which may be numerical or English-language phrases.  Numerical responses consist of a numerical character followed by a CR (carriage return) character.  Phrase responses are preceded and followed by a CR/LF character sequence.  The result codes are shown in the following table. For additional result code information refer to S Register 95.

Digit
Code	Word Code	               Meaning

0	OK	Command line executed successfully

1	CONNECT	Connection successful

2	RING	Ringing signal detected

3	NO CARRIER	Carrier lost or not present

4	ERROR	Invalid command, checksum, command line, or

command-line length

5	CONNECT xxxx	Connection at stated bps

6	NO DIALTONE	No dial tone detected

7	BUSY	Busy signal detected

8	NO ANSWER	No silence detected when dialing a system not

providing a dial tone

Table 5. Result Codes.