# 1.  Product Description

This manual contains the information required to install and use the EXM-32 DeviceNet controller expansion module.

The EXM-32 is a DeviceNet controller board designed to interface to a DeviceNet or CAN field bus and is intended for use in a RadiSys-compatible EPC or EMC system.

There are two versions of the product: EXM-32DN and EXM-32CAN. The 
EXM-32DN is hardware compatible to DeviceNet and is defined as an isolated node with the transceiver and the network side of the optoisolators powered by the network.  A step down switching regulator converts the network 24V to 5V for the transceiver and network side of the optoisolators.  The EXM-32CAN is identical except that the transceiver and the network side of the optoisolators are powered by an isolated on-board 5V-5V DC-DC converter.

The list below presents a summary of capabilities offered by the EXM-32DN based on DeviceNet specifications:

	Supports trunkline-dropline configuration

	Allows node removal without network interruption

	Offers protection from wiring errors

	Supports selectable data rates of 125K, 250K, and 500K baud

	Includes overload protection

	Network Bus contains both signal and power lines in the trunkline

	Supports both network-powered and self-powered devices

The following capabilities are supportable via software:

	Supports duplicate Media Access Control (MAC) ID detection

	Includes provisions for efficient movement of I/O data

	Supports up to 64 nodes

	Facilitates application-to-application communications using object and connection-based software models

The optional EXM-32 SCAN Package includes software to access DeviceNet. The software diskette distributed in the SCAN package contains software written by S-S Technologies and an electronic version of the software user’s manual. Application software is required (from S-S, or user-developed) to access the EXM-32.

The following references may be of additional assistance in using the EXM-32:

1.	DeviceNet Specification, release 1.1, Allen-Bradley part # 956166-02.

2.	Intel386EX Embedded Microprocessor Hardware Reference, Intel Order #272485-000, April 1994, Intel Inc..

3.	Intel386EX Embedded Microprocessor Data Sheet, Intel Order #272420-003, March 1994, Intel Inc.

4.	82527 Serial Communications Controller Architecture Overview, Intel order # 272410-002, February 1995, Intel Inc.

5. 	EXM-32 SCAN Manual, RadiSys and S-S Technologies, December 1995.

## Specifications

Table 1 defines the environmental and electrical specifications of the EXM-32 for both CAN and DeviceNet.

| Characteristic |  | Value |
| --- | --- | --- |
| Environmental |  |  |
| Temperature | operating | 0° to 60° C (* see below) |
|  | storage | -40° to 85° C |
| Humidity | operating | 5% - 95% (non-condensing) |
|  | storage | 5% - 95% (non-condensing) |
| Vibration | operating | .015"displacement with 2.5 g peak (max) acceleration over 5 to 2000 Hz |
|  | storage | .030" displacement with 5 g peak (max) acceleration over 5 to 2000 Hz |
| Shock | operating | 30g 11 msec duration, half-sine shock pulse |
|  | storage | 50g 11 msec duration, half-sine shock pulse |

Table 1. EXM-32 Environmental and Electrical Specifications.

* Upper temperature limit degrades 2° C per 1000 ft. elevation.  Maximum  
   elevation 10,000 ft.