# 2.  Installation

Before installing the EXM-32, unpack and inspect it for shipping damage.

	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-32, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

	ensure that the installation process as described herein is also performed in a static-free environment.

## Insertion in an EXM Carrier

Insertion of the EXM-32 into an EXM carrier is straightforward.  Remove a blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the 
EXM-32 into the card guides.  Firmly press the EXM-32 front panel to ensure that the module is properly seated in the subplane and secure it with the thumbscrews. DO NOT OVERTIGHTEN.

	make sure that power to your system is off.  this exm is not designed to be inserted or removed from a live 
system.

	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

	insert it with adequate continuous force rather than tapping or hammering on it.

## Configuring the BIOS Setup

Once the EXM-32 is inserted into the carrier, turn the system on and start to boot it up. The EXM configuration data in the EPC to which the EXM-32 is connected now must be modified to recognize and enable the card and select from the available options.

Refer to your specific EPC hardware manual for more complete information about setting up an EXM mondule.

### Enabling the EXM module with Phoenix BIOS

If you are using the EPC-8, EPC-9, EPC-26A, or EPC-27, you will be using the Phoenix BIOS to configure your EXM-32. To access the BIOS Setup Menus, press the F2 during the start of the boot process. You cannot reach the BIOS from the command line.

The BIOS Main Menu has a sub-menu for EXM configuration:  the EXM menu. Press the right-arrow key until you reach the EXM menu.

Figure 1. Phoenix BIOS EXM Menu.

Refer to Figure 3 for a discussion of EXM slot numbering. Once you determine the slot number the EXM-32 occupies, enter the configuration information into the ID and option byte sections of the EXM table. Defaults for the EXM-13B are an ID of CF and OB1 set to 9B, with OB2 set to E0. Continue reading this chapter for more information.

### Enabling the EXM module with Award BIOS

If you are using an EPC-4, EPC-5, EPC-6, EPC-7, EPC-21, EPC-22, EPC-23, EPC-24, or EPC-26, you will be using the Award BIOS. Invoke the BIOS setup function from the command line by pressing the CTRL-ALT-ESC keys simultaneously.

Once in the setup program, a menu displays specifying which function keys are available for further configuration.  Press the F2 function key to invoke the EXM menu.  The screen display resembles the figure below.

ID OB1 OB2       
 Slot 0    DC  00  00
      1    DB  C1  00
      2    CF  9B  E0
      3    DE  00  39
      4    F5  05  00
      5    DC  F5  00

Figure 2. EXM Setup Screen.

The EXM Setup Screen displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.  An example of configuration data for the EXM-32 is shown in bold in slot 2. The displayed data shows SLOT, ID, OB1 and OB2.  These are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See the diagram below to determine which EXM slot the EXM-32 occupies.  Note that dashed lines indicate EXM slots that may not be available on all systems.

EXM Slots

Figure 3. EXM Slot Numbering.

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

Note that all slots are listed on the screen even if the actual system configuration does not have all the possible EXM slots.  All slots not occupied by an EXM module should show an ID of FF and OB1/OB2 of   00  00  indicating that no EXM is present.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-32 should be set to CFh.

OB1 is a hexadecimal value derived by combining the following:

| Reserved | IRQ Select (bits 6,5,4) | I/O Address (bits 3,2,1) |  | Card Enable (bit 0) |  |
| --- | --- | --- | --- | --- | --- |
| 1 | (see below) |  | (see below) |  | 0 disable 1 enable |

Figure 4. OB1 Register.

As shown above, OB1 contains the IRQ select, I/O address, and the card enable bit. The interrupt selection is as follows:

| Bit 6 | Bit 5 | Bit 4 | Interrupt |
| --- | --- | --- | --- |
| 0 | 0 | 0 | none |
| 0 | 0 | 1 | IRQ11 |
| 0 | 1 | 0 | IRQ12 |
| 0 | 1 | 1 | IRQ3 |
| 1 | 0 | 0 | IRQ4 |
| 1 | 0 | 1 | IRQ5 |
| 1 | 1 | 0 | IRQ15 |
| 1 | 1 | 1 | IRQ7 |

Table 2. IRQ Selection.

The I/O Address selection is as follows:

| Bit 3 | Bit 2 | Bit 1 | I/O Address |
| --- | --- | --- | --- |
| 0 | 0 | 0 | none |
| 0 | 0 | 1 | 100h |
| 0 | 1 | 0 | 110h |
| 0 | 1 | 1 | 120h |
| 1 | 0 | 0 | 200h |
| 1 | 0 | 1 | 220h |
| 1 | 1 | 0 | 300h |
| 1 | 1 | 1 | 320h |

Table 3. I/O Address Selection.

For example, a typical OB1 value is 9Bh (1001 1011) indicating the card is enabled at IRQ11and I/O address 220h.

Make sure that the IRQ chosen for the EXM-32 does not conflict with any other modules that use an IRQ.  Some common IRQs to avoid are the following:

IRQ3		if already using COM2

IRQ4		if already using COM1

IRQ6		if using floppy disk controller

IRQ7		if using LPT1

It is suggested that the address range of  000 - 107 not be used as the base address due to possible conflicts while configuring other EXMs or AT peripherals in the system. Some other common base I/O address ranges to avoid are the following:

1F0-1FF		IDE disk controller

278-27F		Parallel port (LPT2)

378-37F		Parallel port (LPT1)

3F0-3F7		Floppy disk controller

OB2 holds the memory size enabled on the EXM-32, and the memory start address, offset from a 1 MByte boundary (in the first 1 MByte by default, see below).

Memory address lines A19 to A14 are matched to bits A19 through A14 of the register during memory accesses to the EXM-32.

| A19 | A18 | A17 | A16 | A15 | A14 | Memory Size (bits 1,0) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | see below |

Figure 5. OB2 Register.

| Bit 1 | Bit 0 | Memory Size |
| --- | --- | --- |
| 0 | 0 | 64 Kbyte, 16-bit |
| 0 | 1 | 16 Kbyte, 8-bit |
| 1 | 0 | 32 Kbyte, 16-bit |
| 1 | 1 | 16 Kbyte, 16-bit |

Table 4. Memory Size Settings.

The EXM-32 responds to memory accesses from the host CPU as follows:

If 64 Kbytes of memory is configured, address lines A19 to A16 must match bits A19 through A16, and bits A15 and A14 are ignored. If 32 Kbytes of memory is configured, address lines A19 to A15 must match bits A19 through A15, and bit A14 is ignored. If 16 Kbytes of memory is configured, address lines A19 to A14 must match bits A19 through A14.

Under the 1MB boundary: Under the 1 MByte boundary, due to the driver architecture, 16-bit card conflicts, and other considerations, only one address range really makes sense. This is between D0000h and DFFFFh, even though the EXM-32 can be configured for other addresses (depending on video mode used, other 
64 Kbyte blocks may free up). For systems that cannot fit the entire 64 Kbytes of memory into the address space, there are configuration bits that direct the EXM-32 to act as if only 32 Kbytes or 16 Kbytes were installed. This also potentially opens the C0000h to CFFFFh or the A0000h to AFFFFh regions (again, depending on video usage). For applications with multiple boards, each may be configured in the same memory space. Individual boards may have their memory alternately enabled and disabled.

16 Bit Device: Because the EXM-32 can be a 16-bit device, care must be taken when using it in the same 128 Kbytes block as an 8-bit device. The EXM-32 dual port memory must be disabled (via the I/O port) before accessing the 8-bit device.

8-bit Device:  It is possible to specify the memory size to be 16 Kbytes, 8-bit which allows one to operate the shared memory as an 8-bit interface.  This should only be done to avoid conflicts that are otherwise unavoidable due to usage of another EXM card that cannot operate in 16-bit mode and must overlap the memory region of the EXM-32.

For example, a typical OB2 setting would be E0h, setting system to 64 Kbytes of memory using 16-bit accesses.

I/O port 104h (Option Byte 3 Register): This register is used to relocate the EXM-32 shared memory above the 1MB PC/AT boundary on the host CPU side. Memory address lines A23 to A20 (bits 3, 2, 1 and 0) are matched to bits A23 through A20 of the register during memory accesses to the EXM-32.

Note that this register is not configurable from the RadiSys CPU setup screen. Other than that difference, access is the same as to Option Bytes 1 or 2 (the slot number must be written to I/O address 96h before accessing the register). Because this register cannot be set at system boot, it has a default value of '0' in all non-reserved bit positions. This configures the EXM-32 memory under the PC/AT 1 MByte boundary. The purpose of this register is to allow the memory to be configured beyond that barrier if so desired.

The most significant bits (bits 7, 6, 5, and 4) are reserved for future use, and return '1' when read. A write to these bits has no effect.

NOTES