# 1.  Product Description

This manual contains the information required to install and use the EXM-33 Counter/Timer expansion module.

The EXM-33 provides timing, counting, frequency measurement, frequency generation, and pulse-width modulation. Uses include timing, speed measurement, event counting, and related tasks.

The EXM-33 uses an 82C54 Counter/Timer chip for counting and timing functions, and uses an 82C55 programmable peripheral interface chip to provide eight general purpose digital I/O signals. The 82C54 consists of three independent 16-bit counters with six programmable counter modes. The 82C55 consists of 24 programmable I/O pins in three ports.

The external onboard registers are accessible by the host processor and are used to set the base I/O address, enable the board, select interrupts, etc.

The faceplate of the EXM-33 contains a DB62 3-row I/O connector and a green LED that lights when the board is accessed.

Multiple EXM-33s can be installed in a single system, but interrupts may not be shared.

## Specifications

Table 1 defines the environmental and electrical specifications of the EXM-33.

| Characteristic |  | Value |
| --- | --- | --- |
| Environmental |  |  |
| Temperature | operating | 0° to 60° C (* see below) |
|  | storage | -40° to 85° C |
| Humidity | operating | 5% - 95% (non-condensing) |
|  | storage | 5% - 95% (non-condensing) |
| Vibration | operating | .015"displacement with 2.5 g peak (max) acceleration over 5 to 2000 Hz |
|  | storage | .030" displacement with 5 g peak (max) acceleration over 5 to 2000 Hz |
| Shock | operating | 30g 11 msec duration, half-sine shock pulse |
|  | storage | 50g 11 msec duration, half-sine shock pulse |
| Electrical |  |  |
| Current | maximum | 5V @ 50 mA |
|  | typical | 5V @ 50 mA |

Table 1. EXM-33 Environmental and Electrical Specifications.

* Upper temperature limit degrades 2° C per 1000 ft. elevation.  Maximum  
   elevation 10,000 ft.