# 2.  Installation

Before installing the EXM-33, unpack and inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-33, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Insertion in an EXM Carrier

Insertion of the EXM-33 into an EXM carrier is straightforward.  Remove a blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the 
EXM-33 into the card guides.  Firmly press the EXM-33 front panel to ensure that the module is properly seated in the subplane and secure it with the thumbscrews. DO NOT OVERTIGHTEN.

*	make sure that power to your system is off.  this exm is not designed to be inserted or removed from a live 
system.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

*	insert it with adequate continuous force rather than tapping or hammering on it.

## Configuring the BIOS Setup

Once the EXM-33 is inserted into the carrier, turn the system on and start to boot it up. The EXM configuration data in the EPC to which the EXM-33 is connected now must be modified to recognize and enable the card and select from the available options.

Refer to your specific EPC hardware manual for more complete information about setting up an EXM module.

### Enabling the EXM module with Phoenix BIOS

If you are using the EPC-8, EPC-9, EPC-26A, or EPC-27, you will be using the Phoenix BIOS to configure your EXM-33. To access the BIOS Setup Menus, press the F2 during the start of the boot process. You cannot reach the BIOS from the command line.

The BIOS Main Menu has a sub-menu for EXM configuration:  the EXM menu. Press the right-arrow key until you reach the EXM menu.

Figure 1. Phoenix BIOS EXM Menu.

Refer to Figure 3 for a discussion of EXM slot numbering. Once you determine the slot number the EXM-33 occupies, enter the configuration information into the ID and option byte sections of the EXM table. Defaults for the EXM-33 are an ID of CE and OB1 set to 03, with OB2 set to 03. This configures the EXM-33 at I/O port address 110h, programmed to cause an interrupt at IRQ3 generated from the 82C55. Continue reading this chapter for more information.

### Enabling the EXM module with Award BIOS

If you are using an EPC-4, EPC-5, EPC-6, EPC-7, EPC-21, EPC-22, EPC-23, 
EPC-24, or EPC-26, you will be using the Award BIOS. Invoke the BIOS setup function from the command line by pressing the CTRL-ALT-ESC keys simultaneously.

Once in the setup program, a menu displays specifying which function keys are available for further configuration.  Press the F2 function key to invoke the EXM menu.  The screen display resembles the figure below.

ID OB1 OB2       
 Slot 0    DC  00  00
      1    DB  C1  00
      2    CE  03  03
      3    DE  00  39
      4    F5  05  00
      5    DC  F5  00

Figure 2. Award BIOS EXM Setup Screen.

The Award BIOS EXM Setup Screen displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.  An example of configuration data for the EXM-33 is shown in bold in slot 2. The displayed data shows SLOT, ID, OB1 and OB2.  These are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See the diagram below to determine which EXM slot the EXM-33 occupies.  Note that dashed lines indicate EXM slots that may not be available on all systems.

EXM Slots

Figure 3. EXM Slot Numbering.

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

Note that all slots are listed on the screen even if the actual system configuration does not have all the possible EXM slots.  All slots not occupied by an EXM module should show an ID of FF and OB1/OB2 of   00  00  indicating that no EXM is present.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-33 should be set to CEh.

OB1 is a hexadecimal value derived by combining the following:

| Rsvd (bits 7,6,5,4) | I/O port Address (bits 3,2,1) | Card Enable (bit 0) |
| --- | --- | --- |
| 0000 | (see below) | 0 disable 1 enable |

Figure 4. OB1 Register.

As shown above, OB1 contains the I/O address and the card enable bit. The I/O address selection is as follows:

| Bit 3 | Bit 2 | Bit 1 | I/O Address |
| --- | --- | --- | --- |
| 0 | 0 | 0 | none |
| 0 | 0 | 1 | 110h |
| 0 | 1 | 0 | 120h |
| 0 | 1 | 1 | 130h |
| 1 | 0 | 0 | 140h |
| 1 | 0 | 1 | 150h |
| 1 | 1 | 0 | 160h |
| 1 | 1 | 1 | 170h |

Table 2. I/O Address Selection.

For example, a typical OB1 value is 03h (0000 0011) indicating the card is enabled at I/O address 110h.

OB2 is a hexadecimal value derived by combining the interrupt destination, the interrupt source selection, and three bits choosing the IRQ signal to be used:

| Rsvd (bits 7,6) | IRQ Select (bits 5,4,3) | Source (bit 2) | Destination (bits 1,0) |
| --- | --- | --- | --- |
| 00 | (see below) | 0 82C55 1 rsvd | 00 none 01 none 10 reset 11 interrupt |

Figure 5. OB2 Register.

The IRQ selection is as follows:

| Bit 5 | Bit 4 | Bit 3 | IRQ Selection |
| --- | --- | --- | --- |
| 0 | 0 | 0 | IRQ3 |
| 0 | 0 | 1 | IRQ4 |
| 0 | 1 | 0 | IRQ5 |
| 0 | 1 | 1 | IRQ6 |
| 1 | 0 | 0 | IRQ7 |
| 1 | 0 | 1 | IRQ9 |
| 1 | 1 | 0 | IRQ11 |
| 1 | 1 | 1 | IRQ15 |

Table 3. IRQ Selection.

Make sure that the IRQ chosen for the EXM-33 does not conflict with any other modules that use an IRQ.  Some common IRQs to avoid are the following:

IRQ3		if already using COM2

IRQ4		if already using COM1

IRQ6		if using floppy disk controller

IRQ7		if using LPT1

The EXM-33 can be programmed to cause either a reset or an interrupt to the system from one of two sources. Whether a reset or interrupt is generated is selected by bits 0-1 of the OB2 register. If an interrupt is selected, Bits 3-5 select which IRQ is used to signal the interrupt. They have no effect if bits 0-1 select “none” or “reset”.

Reserved bits return a ‘1’ when read. A write to these bits has no effect.