# 3. Connector

## External Interface Connector

The EXM-33 has a female 62-pin high-density (three-row) connector. The signal pinout is as follows:

| J2---Female DB62 High-Density (3-row) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pin | Signal | Description | Pin | Signal | Description | Pin | Signal | Description |
| 1 | CTC1_C0 | CTC1 Counter #0 Clock input | 22 | GND | Logic ground | 43 | N/C | not connected |
| 2 | CTC1_G0 | CTC1 Counter #0 Gate input | 23 | GND | Logic ground | 44 | N/C | not connected |
| 3 | CTC1_O0 | CTC1 Counter #0 Output | 24 | GND | Logic ground | 45 | N/C | not connected |
| 4 | CTC1_C1 | CTC1 Counter #1 Clock input | 25 | GND | Logic ground | 46 | N/C | not connected |
| 5 | CTC1_G1 | CTC1 Counter #1 Gate input | 26 | GND | Logic ground | 47 | N/C | not connected |
| 6 | CTC1_O1 | CTC1 Counter #1 Output | 27 | GND | Logic ground | 48 | N/C | not connected |
| 7 | CTC1_C2 | CTC1 Counter #2 Clock input | 28 | GND | Logic ground | 49 | N/C | not connected |
| 8 | CTC1_G2 | CTC1 Counter #2 Gate input | 29 | GND | Logic ground | 50 | N/C | not connected |
| 9 | CTC1_O2 | CTC1 Counter #2 Output | 30 | GND | Logic ground | 51 | N/C | not connected |
| 10 | CTC3_O0 | CTC3 Counter #0 Output | 31 | GND | Logic ground | 52 | +5V | Fused +5V |
| 11 | CTC3_O2 | CTC3 Counter #2 Output | 32 | GND | Logic ground | 53 | GND | Logic ground |
| 12 | CTC2_C0 | CTC2 Counter #0 Clock output | 33 | GND | Logic ground | 54 | CTC2_C0+ | CTC2 Counter #0 opto-isolated positive Clock input |
| 13 | CTC2_C1 | CTC2 Counter #1 Clock output | 34 | GND | Logic ground | 55 | CTC2_C0- | CTC2 Counter #0 opto-isolated negative Clock input |
| 14 | CTC2_C2 | CTC2 Counter #2 Clock output | 35 | GND | Logic ground | 56 | GND | Logic ground |
| 15 | PIO0 | Parallel I/O bit 0 | 36 | GND | Logic ground | 57 | CTC2_C1+ | CTC2 Counter #1 opto-isolated positive Clock input |
| 16 | PIO1 | Parallel I/O bit 1 | 37 | GND | Logic ground | 58 | CTC2_C1- | CTC2 Counter #1 opto-isolated negative Clock input |
| 17 | PIO2 | Parallel I/O bit 2 | 38 | GND | Logic ground | 59 | GND | Logic ground |
| 18 | PIO3 | Parallel I/O bit 3 | 39 | GND | Logic ground | 60 | CTC2_C2+ | CTC2 Counter #2 opto-isolated positive Clock input |
| 19 | PIO4 | Parallel I/O bit 4 | 40 | GND | Logic ground | 61 | CTC2_C2- | CTC2 Counter #2 opto-isolated negative Clock input |
| 20 | PIO5 | Parallel I/O bit 5 | 41 | GND | Logic ground | 62 | GND | Logic ground |
| 21 | PIO6 | Parallel I/O bit 6 | 42 | PIO7 | Parallel I/O bit 7 |  |  |  |

Table 5. External Interface Connector Pin-out.