# 1.  Product Description

The EXP-MX mass storage modules are 6U (VMEbus-size) modules that plug into the EXM expansion interface of a RadiSys VMEbus embedded PC. The EXP-MX mass storage module contains a disk controller board, a hard disk, and a 1.44 MB floppy diskette drive. Depending on the capacity and physical size of the internal hard disk, an EXP-MX module occupies either two or three VMEbus slots.

Front-panel connectors are provided for adding either a 3.5" or a 5.25" external floppy drive. There are also three indicator LEDs. Two indicate that the VMEbus +5V and +12V supplies are available (the +12V supply is needed for the hard disk drive).  The third LED is lit whenever a hard disk access is underway.

All EXP-MX models are fully MS-DOS compatible.  The EXP-MX uses the IDE (Integrated Device Electronics) interface.

EXP-MX models are compatible with all PC operating systems, not just MS-DOS.

The EXP-MX mates to an EXM expansion subplane that has a special right-most connector for the EXP-MX, like the BP-3, -4, -5, and -6. Only one EXP-MX module may be used in a system, and an EXP-MX cannot be used with either EXM-3 or EXM-9 disk controllers, or with the EXM-MX series disk drives.

## Specifications

The following table defines the typical environmental and electrical specifications of the EXP-MX. Individual drive types may vary.

| Characteristic |  | Value |  |
| --- | --- | --- | --- |
| Environmental |  |  |  |
| Temperature | operating |  | 5 to 50°C ambient |
|  | storage |  | -40 to 65°C |
| Humidity | operating |  | 8 to 85% non condensing |
|  | storage |  | 5 to 95% non condensing |
| Altitude | operating |  | 0 to 10,000 ft (3,000 m) |
|  | storage |  | 0 to 40,000 ft (12,000 m) |
| Vibration * | operating |  | 1 G (max) acceleration, 5-500 Hz (peak to peak) sine wave |
|  | storage |  | 2 G (max) acceleration, 5-500 Hz (peak to peak) sine wave |
| Shock * | operating |  | 10 G, 11 ms duration, half-sine shock pulse |
|  | storage |  | 70 G, 11 ms duration, half-sine shock pulse |
| Electrical |  |  |  |
| Power | maximum |  | 20 W for 15 seconds at power-up |
|  | typical |  | 10 W |
| Current | maximum |  | +5V @ 1.5A, +12V @ 1A |
|  | typical |  | +5V @ 1A, +12V @ 0.4A |
| Other |  |  |  |
| Weight | maximum |  | 35 oz (1000 g), over all models |

Table 1. EXP-MX Specifications.

* 	NOTE: Shock and Vibration as measured on the hard disk itself.  These values may not be equivalent to shock and vibration applied to the VMEbus chassis.