3.  Programming Interface

The table in this chapter defines the I/O Port addresses used by the EXP-MX modules.  Anyone just using the EXP-MX as a BIOS-controlled disk unit and interface can skip reading this chapter.

For an explanation of the floppy diskette controller registers, refer to documentation on the WD37C65 controller chip.  For an explanation of the EXP-MX only hard drive registers, refer to documentation on any standard PC/AT fixed-disk interface.

The floppy disk controller in the EXP-MX uses interrupt IRQ6 and DMA channel 2. The IDE disk controller in the EXP-MX uses interrupt IRQ14.

| Address | Description | Notes: |
| --- | --- | --- |
| 1F0 | Data Register (16 bits) | MX only |
| 1F1 | Error / Write Precompensation Register | MX only |
| 1F2 | Sector Count Register | MX only |
| 1F3 | Sector Number Register | MX only |
| 1F4 | Cylinder Low Register | MX only |
| 1F5 | Cylinder High Register | MX only |
| 1F6 | SDH Register | MX only |
| 1F7 | Status / Command Register | MX only |
| 2B0 | not available |  |
| 2B1 | not available |  |
| 2B2 | not available |  |
| 2B3 | not available |  |
| 2B4 | not available |  |
| 2B5 | not available |  |
| 2B6 | not available |  |
| 2B7 | not available |  |
| 3F2 | Floppy Operations Register |  |
| 3F4 | Floppy Command Register |  |
| 3F5 | Floppy Data Register |  |
| 3F6 | Alternate Status / Digital Output Register | MX only |
| 3F7 | Drive Address Register / Floppy Control Register | MX only |

Table 4.  EXP-MX  I/O Port Addresses.