National Instruments OS/2 1.4 Driver for the MC-GPIB.

Revision 1.4.

Copyright 1989, 1992  National Instruments Corporation.

All Rights Reserved.

-----------------------------------------------------------------

Note For Revision 1.4  NI-488 MC-GPIB Driver for OS/2

-----------------------------------------------------------------

This driver is specifically designed to work with the MC-GPIB boards.

It can also work with the new MC-GPIB board, which uses the NAT4882

chip instead of the NEC �PD7210 chip. However, the special features of

the NAT4882 chip are not utilized by this driver.

DOS Compatability Box

---------------------

Information in the "NI-488 OS/2 Software Reference Manual" about using

the NI-488 driver from the DOS compatability box has been updated and

is no longer accurate.  If you want to run a DOS GPIB application

in the compatability box, you must obtain Rev. 2.0 (or later) of our

NI-488 MC-GPIB driver for MS-DOS.

You cannot use DMA or interrupts when using the MS-DOS driver in the OS/2

DOS compatability box.  You must disable DMA and interrupts by using the

ibconf that comes with the NI-488 driver for MS-DOS. You must not call the

GPIB DOS driver from protected mode applications. The MS-DOS driver can be

installed for a single DOS session by selecting DOS_DEVICE from the DOS

Settings of that particular DOS session. Add the DOS driver by

typing its name on a new line. For more information on changing DOS

Settings for an open session, refer to the OS/2 2.0 "Master Help

Index". You can also install the MS-DOS driver for all DOS Sessions by

specifying the configuration command, 'DEVICE = ...', in the OS/2

config.sys file.

Programmed Input/Output

-----------------------

The minimum timeout value for a programmed I/O transfer is 30 msec.

The minimum timeout value for a DMA transfer is 1 msec.

CORRECTION:

NI-488 OS/2 Software Reference Manual. Page 1-2.

------------------------------------------------

Disregard the second 'Note' on page 1-2 under the section titled

'The NI-488 OS/2 Driver'. This note should read :

"If you are using OS/2, you must use the NI-488 driver for OS/2.

If you are in a DOS session, you must use the NI-488 driver

for MS-DOS with interrupts and DMA disabled."