INSTALLING DRIVERS FOR SCSI DISK DRIVES

PREPARATION

This file explains how to use files on this disk required to prepare the

following drives for use.

o  More than two fixed disk drives

o  Removable disk drives

o  Read/write optical disk drives

o  Wangtec tape drive(s)

o  CD-ROM devices

To install the device driver for these drives,  you need the following:

o  An MS-DOS boot partition on the fixed disk

o  A config.sys file in the root directory of the MS-DOS partition

o  A working copy of the NCR Software Support Diskette

NOTE: All versions of MS-DOS 3.3, 4.X or 5.0  support one or two SCSI fixed

disk drives installed inside or outside the system cabinet.

INSTALLATION

The following instructions explain how to install the device drivers on a

fixed disk.  This is the recommended installation.

1.   With the system turned on, place the working copy of the NCR Software

Support Diskette in drive A.

2.   Type the commands listed below to copy required files from the

NCR Software Support Diskette to the root directory of the fixed disk.

COPY A:\SCSI\DOS\CAMIDD.SYS C:\

COPY A:\SCSI\DOS\DISKIDD.SYS C:\

3.   Copy your config.sys file to a backup file called config.sav by typing:

COPY C:\CONFIG.SYS C:\CONFIG.SAV

If you make a mistake when editing the config.sys file, you can rename

config.sav to restore the original file.

INSTALLATION (continued)

4.   Use your favorite editor or the MS-DOS edlin utility to change or add the

following lines to the config.sys file.

DEVICE=CAMIDD.SYS

DEVICE=DISKIDD.SYS

NOTE: "DEVICE=CAMIDD.SYS" MUST be the first "DEVICE=" line of the

config.sys file.

5.   Reboot the computer by pressing the Ctrl, Alt, and Del keys at the same

time.  After reboot, a message appears on the screen when the device

driver has been installed.

PREPARING NEW MEDIA

To prepare new media for use by removable media drives and Read/Write optical

drives, use the LOWLFMT utility and follow the steps listed below.

NOTE: Skip this section if you have fixed disks only.

1.   With the system turned on, place the working copy of the NCR Software

Support Diskette in drive A.

2.   Type the command listed below to copy the required file from the

NCR Software Support Diskette to the directory on your fixed disk with

the MS-DOS files.

For example, if you use MS-DOS 3.3 and call the directory where the files

are stored DOSDIR3X, copy lowlfmt.exe to that directory.

Put the NCR Software Support Diskette in drive A and type the following

command:

COPY A:\SCSI\DOS\LOWLFMT.EXE C:\DOSDIR3X

PREPARING NEW MEDIA (continued)

3.   At the DOS prompt, type LOWLFMT

4.   Follow the screen prompts for the utility.

5.   After the low-level format is completed, use the fdisk utility described

below to partition the media.

6.   After the disk is partitioned, use the FORMAT command to initialize the

logical disk(s) created by the FDISK3, FDISK4 or FDISK5 utility.

NOTE: When using FDISK3 or FDISK4 to partition removable media (such as

Read/Write Optical drives), it is STRONGLY RECOMMENDED that you create

a SINGLE, EXTENDED partition that uses ALL of the disk space for a

SINGLE logical drive.

PARTITIONING

If you install more than two SCSI fixed disk drives, partition the additional

drives with one of the fdisk utilities on the NCR Software Support Diskette.

NOTE: The fdisk utility for MS-DOS 3.3 and 4.x partitions only the first and

second fixed disk drives. Although MS-DOS 5.0 fdisk supports more than

two fixed disk drives you have to use the one supplied on the NCR Soft-

ware Support Disk. This is important for retaining the logical drive

lettering!!

To use the fdisk utility on the NCR Software Support Diskette, follow the

steps listed below.

1.   Determine whether you are using MS-DOS 3.3, MS-DOS 4.X or MS-DOS 5.0

with the additional SCSI fixed drives.

PARTITIONING (continued)

2.   Copy the appropriate file from the NCR Software Support Diskette to

the directory on your fixed disk with the MS-DOS files.

MS-DOS VERSION              FILE

==============           ==========

MS-DOS 3.3             fdisk3.exe

MS-DOS 4.X             fdisk4.exe

MS-DOS 5.0             fdisk5.exe

For example, if you use MS-DOS 3.3 and call the directory where the files

are stored DOSDIR3X, copy fdisk3.exe to that directory.

Put the NCR Software Support Diskette in drive A and type the following

command:

COPY A:\SCSI\DOS\FDISK3.EXE C:\DOSDIR3X

PARTITIONING (continued)

3.  To partition the new disk drive(s), type the following command at the

DOS command prompt:

FDISK3

Follow screen instructions to partition the disk(s).

After the fixed disk is partitioned, use the format command to initialize

the logical disk(s) created by the FDISK3, FDISK4 or FDISK5 utility.

TAPE DRIVER INSTALLATION

The following instructions explain how to install the device drivers for tape

drives on a fixed disk.  This is the recommended installation.

1. Make certain that you have installed the CAMIDD.SYS and DISKIDD.SYS

drivers in your CONFIG.SYS file according to the instructions above.

2. Follow the Sytos Plus installation instructions to install Sytos Plus.

3. With the system turned on, place the working copy of the NCR Software

Support Diskette in drive A.

4. Type the command listed below to copy the required file from the NCR

Software Support Diskette to the proper directory of the fixed disk.

For the WANGTEC tape drive, type:

COPY A:\SCSI\DOS\NCM_525W.DRV C:\SYPLUS\SYFILES

TAPE DRIVER INSTALLATION (continued)

5. Before you use the tape drive, follow the directions in the Sytos Plus

User's Guide on how to "Add a new device".  This information is listed

in Chapter 5 in the "Utilities: Backup device setup" section.

CD-ROM Installation

The following instructions explain how to install the device drivers for

the CD-ROM device. The CD-ROM driver is contained in the A:\SCSI\DOS

sub-directory.

The following CD-ROM device driver is supplied:

Manufacturer            Filename

============            ========

Toshiba CD-ROM          CDRTOSH.SYS

1. Copy the CD-ROM device driver to your root directory.

COPY A:\SCSI\DOS\CDRTOSH.SYS C:\

CD-ROM Installation (continued)

2. To install the CD-ROM device drivers, you must first modify your config.sys

file to contain the following.

DEVICE=CAMIDD.SYS

DEVICE=DISKIDD.SYS

DEVICE=CDRTOSH.SYS /D:MSC000

3. Reboot your system in order for the changes to take effect.

4. To install the Microsoft Extensions that enable allocation of drive

lettering, file system, and invokes the CD-ROM device, copy the mscdex.exe

file from the diskette you received with your CD-ROM device order to the

root directory of your hard disk.

COPY A:\MSCDEX.EXE C:\

Type the following command at the DOS prompt to install the Microsoft

Extensions. For a more detailed description on the switches to the

CD-ROM Installation (continued)

Microsoft CD-ROM Extensions program refer to the README.DOC file on the

CD-ROM diskette.

MSCDEX /D:MSC000

IBM IMAGING ADAPTER

If you use the IBM Imaging Adapter in the DOS environment, then you must

install an additional device driver to ensure correct operation.  The

following instructions explain how to install the device driver on a fixed

disk.  This is the recommended installation.

1.   With the system turned on, place the working copy of the NCR Software

Support Diskette in drive A.

2.   Type the command listed below to copy required file from the NCR

Software Support Diskette to the root directory of the fixed disk.

COPY A:\SCSI\DOS\IIMAGE.SYS C:\

3.   Use the MS-DOS edlin editor or retype the config.sys file to add the

following line.

DEVICE=IIMAGE.SYS

NOTE: "DEVICE=IIMAGE.SYS" MUST be the last "DEVICE=" line of the config.sys

file.

#