******************************************************************************

I N F O R M A T I O N

******************************************************************************

This file contains information about the following topics:

E/Master Ethernet Adapter

Serial Port Limitations

Timer 2 Restrictions

2.88 MB Disk drive

Novell Netware

DOS Upper Memory Area

Memory Map

******************************************************************************

******************************************************************************

E/Master Ethernet Adapter

******************************************************************************

- The usage of the E/Master Ethernet Adapter within NCR 3220 and NCR 3321

is not recommented.

******************************************************************************

Serial Port Limitations

******************************************************************************

1. Serial port fails in two (2) Stop-Bit mode.

Use always one (1) Stop-Bit mode.

2. Serial port FIFO reset doesn't clear the receive buffer.

Overrun may occur when receiving data in normal mode and switching FIFO

mode.

3. Bit 3 to 0 of MSR are read/write-able.

******************************************************************************

******************************************************************************

Timer 2 Restrictions

******************************************************************************

1. 16 bit counter latch releases high byte after reading low byte

This may lead to incorrect count if the count has rolled over from LSB to

MSB during latch period. This may occur if the counter is being used in a

16 bit mode, after being used in 8 bit mode or 16 bit mode where only one

byte has been accessed.

2. Mode 0 and 4 not supported on Timer 2.

Do not  use mode 0 and 4 on timer 2.

3. Edge trigger mode not sensitive to edge.

This may result in inaccurate readings using this mode. Do not use the

edge trigger mode.

******************************************************************************

******************************************************************************

2.88 MB Diskette drives

******************************************************************************

- Problems may arise if older versions of operating systems are used,

which don't support the 2.88 MB diskette drives.

******************************************************************************

Novell Netware

******************************************************************************

The NCR 3220 system is certified as a workstation from Novell.

The certification for the NCR 3321 as server and workstation is in process.

Please note, that the SCSI drivers for Novell 2.x and 3.x included on the

Software Support Diskette were not Novell certified at the time it has been

released..

******************************************************************************

******************************************************************************

DOS Upper Memory Area

******************************************************************************

To use the Memory Page C (and D) for DOS Upper Memory Area run the Reference

Diskette and select "Relocate Memory Page C(and D) = No".

Note: The Automatic Configuration will set this pages to "Relocate = Yes"

******************************************************************************

******************************************************************************

Memory Map Standard Use

******************************************************************************

Condition in the Setup: Shadow BIOS    	= yes

Automatic Setup

Address

(Hex)

000000	����������������������������������ͻ

�     Interrupt Vector             �

000400	����������������������������������Ķ

�     BIOS Data Area               �

����������������������������������Ķ

�     Disk Operation System        �

�     (including BIOS Interface)   �

����������������������������������Ķ

�     RAM Memory                   �

�     (up to 640 KB)               �

0A0000	����������������������������������Ķ

continuation next page

0A0000	����������������������������������Ķ

�                                  �

�          Video RAM               � Memory Page A

�                                  � relocated

0B0000	����������������������������������Ķ

�                                  �

�          Video RAM               � Memory Page B

�                                  � relocated

0C0000	����������������������������������Ķ

�                                  � Memory Page C

�          ROM Extension           � relocated

�                                  �

0D0000	����������������������������������Ķ

�                                  � Memory Page D

�          ROM Extension           � relocated

�                                  �

0E0000	����������������������������������Ķ

continuation next page

0E0000	����������������������������������Ķ

�                                  � Memory Page E

�                                  � not relocated

�     System ROM BIOS 128KB        �

�     Shadowed into the RAM        � Memory Page F

�                                  � not relocated

�                                  �

100000	����������������������������������Ķ

�                                  �

�     Up to 15 MB                  �

�     RAM Extension                �

�     (I/O Channel Memory)         �

�                                  �

FE0000	����������������������������������Ķ

�                                  �

�     Duplicated Code Assignment   �

�     at Address 0E0000            �

�                                  �

����������������������������������ͼ

*******************************************************************************#