=========================================================================

INSTALLING SCSI DISK DRIVERS FOR NETWARE 286

=========================================================================

This section explains how to install device drivers for SCSI

fixed disk drives used with the Novell NetWare operating system.

The following two methods are outlined in the SFT/Advanced

NetWare 286 Installation manual.

1)  Standard Floppy Disk Method

2)  Network Drive Method

****  THE SCSI DEVICE DRIVERS FOR NETWARE SHOULD BE INSTALLED ON THE

FILE SERVER ONLY.

When you configure NetWare disk drivers, select a DMA channel and

a driver to use that DMA channel. Use the driver only with the

selected DMA channel; do not assign it to a second channel.

NETGEN allows you to select the device driver multiple times.  If

you select more than one DMA channel for the device  driver,  it

operates only on the lowest channel selected.

===========================================================================

How to Prepare for Installation

===========================================================================

The following are required.

-  A "bootable" flexible or fixed disk

-  A config.sys file with the statements FILES = 20

and BUFFERS = 15 in the root directory of the "boot" disk

-  A working copy of the NCR Software Support Diskette

-  The SFT/Advanced NetWare 286 Installation manual and diskettes

============================================================================

How to Use the "Standard Floppy Disk Method"

============================================================================

Follow  the steps listed below to install the SCSI device  driver

on the file server using the NetWare "Standard Floppy Disk

Method."

1.  Insert a formatted 3.5-inch diskette in drive A and make the

following volume label:

DSK_DRV_NCR

2.  Now insert the diskette which contains the files ncrsc9x.DSK and

ncrsc9x.OBJ from the NCR bulletin board into drive B. Copy these

two files to the diskette in drive A by typing the following

command:

COPY B:\ncrsc9x.OBJ A:

COPY B:\ncrsc9x.DSK A:

3.  Follow the instructions in the SFT/Advanced NetWare 286

Installation manual starting with the section "Selecting the

Network Configuration with NETGEN."

Install the Netware operating system according to the "Standard

Floppy Disk Method" until you get to the screen for selecting

disk drivers.

4.  At the prompt for disk driver options, select "Load and Select

Item."

5.  Put the disk with the volume label DSK_DRV_NCR into flexible disk

drive A and press Enter.

A screen message should appear when the volume has been copied.

6.  Follow screen prompts and the directions in the SFT/Advanced

NetWare 286 Installation manual to complete  installation  of

the NetWare operating system.

===========================================================================

How to Use the "Network Drive Method"

===========================================================================

The  NetWare "Network Drive Method" is used to establish another

server on the current network without having to use the longer

"Standard Floppy Disk Method".  For information on when to use

this method see the SFT/Advanced NewWare 286 Installation manual.

To install the SCSI driver on a system using  the "Network Drive

Method," follow the steps listed below.

1.  Connect the system to a file server that already has the  NetWare

files installed.

2.  Log-on to the file server through the system keyboard and  change

to the directory with the NetWare files.  Usually, this directory

will be named NetWare.

3.  Create a directory named dsk_drv_.ncr by typing:

MD DSK_DRV_.NCR

4.  Place the Diskette which contains the driver from the NCR bulletin

board into drive A.  Type the following command:

COPY A:\ncrsc9x.OBJ  DSK_DRV_.NCR

COPY A:\ncrsc9x.DSK  DSK_DRV_.NCR

5.  You should still be in the directory with the NetWare files.

Type NETGEN and follow directions on the screen and in the

SFT/Advanced NetWare 286 Installation manual until you reach the

prompt to select disk drivers.

6.  At the prompt for disk driver options, select "Load and Select

Item."

When the driver has been selected, a screen message appears

indicating the driver was loaded successfully.

7.  Follow screen prompts and the directions in the  SFT/Advanced

NetWare  286 Installation manual to complete  installation  of

the NetWare operating system.

#######