******************************************************************************

V G A

******************************************************************************

This file contains information for installing specific applications which

can make use of the VGA extended graphics modes. For all extended graphics

modes you need the HIGH_RES.SYS. The specific applications are:

Microsoft Windows 3.0

Microsoft Windows 3.1

AutoCAD Release10

AutoCAD Release11

Lotus123  3.0

Artline 2.0

Framework III

******************************************************************************

*****************************************************************************

WINDOWS 3.0 and 3.1 SUPPORT

*****************************************************************************

Windows 3 must first be installed. If it not, install the program

and select VGA as the display.

At the DOS prompt, change to the directory containing Windows 3.

C:>  CD\WINDOWS

Run the setup program by typing `setup' and pressing return.

C:\WINDOWS>  setup

*****************************************************************************

*****************************************************************************

WINDOWS 3.0 and 3.1 (CONT.)

*****************************************************************************

The setup program will show the list of options presently used for

your system similar to that shown below.

System Information

Computer:              MS-DOS or PC-DOS System

Display:               VGA

Mouse:                 Microsoft, or IBM PS/2

Keyboard:              101 or 102 key..

Keyboard Layout:       US

Language:              xxx

Network:               xxx

Use the up and down arrows to move the black bar to the line:

`Display:       VGA'     then press return.

*****************************************************************************

*****************************************************************************

WINDOWS 3.0 and 3.1 (CONT.)

*****************************************************************************

Use the up and down arrow keys to select display options. Press the

up and down keys until the selection reads:

`Other (Requires disk provided by a hardware manufacturer)' then

press return.

The setup program will ask to enter the drive and directory which

contains the Windows 3 drivers. Enter the drive and directory as

shown below.

For Windows 3.0 use Support Diskette 1 and type:

A:\VGA\WIN3    <return>

For Windows 3.1 use Support Diskette 2 and type:

A:\VGA\WIN31   <return>

*****************************************************************************

*****************************************************************************

WINDOWS 3.0 and 3.1 (CONT.)

*****************************************************************************

The setup program will display the choices for your computer. Use

the up and down arrow keys to select the resolution and press

return.

800 x 600 16 / 256 Color Display

1024 x 768 16 Color Display

640 x 480 256 Color Display

The Windows 3.1 Drivers support only the 256 Color.

After you have entered your choice, the setup program will complete

installation of the driver and will return to the prompt. Windows 3

can now be run in the new display resolution by typing `win'.

C:\WINDOWS> win    <return>

*****************************************************************************

*****************************************************************************

AUTOCAD RELEASE 10

*****************************************************************************

On the Software Support Diskette 2, in the \VGA\ACAD10 subdirectory

there is an ADI driver which supports enhanced VGA graphics modes.

R4V1600.EXE - supports 800x600 graphics mode,

In order to use these mode, you must have a Multiscan Monitor attached to

an EGA or VGA card capable of the resolution to be displayed.  Also, the

desired driver must be loaded before entering the AutoCAD program.  To load

the driver, copy the proper file, into your Auto

AutoCAD directory, set that directory to your default directory. At the DOS

command prompt, type the driver name (e.g. 'R4V1600').

****************************************************************************

****************************************************************************

AUTOCAD 10 (CONT.)

****************************************************************************

Alternately to install the driver automatically each time you turn on

or reboot your computer, add the driver name into the commands in your

AUTOEXEC.BAT file (e.g. 'C:\ACAD\R4V1600').

CONFIGURING AUTOCAD

The first time you use AutoCAD with the ADI driver, you need to configure

AutoCAD for an ADI display. Select 'Configure AutoCAD' from the AutoCAD

Main Menu. After you see the current configuration (if any) select

'Configure video display'. Select ADI as your new driver (installed at

interrupt 7Ahex).Save the new configuration and return to the Main Menu.

You can now begin an AutoCAD session using the enhanced graphics mode.

*****************************************************************************

*****************************************************************************

AUTOCAD RELEASE 11

*****************************************************************************

On the Software Support Diskette 2, in the \VGA\ACAD11 subdirectory

there is an ADI driver which supports enhanced VGA graphics modes.

In order to install it, you need to copy it to your default AUTOCAD

subdirectory. We suggest that you rename the current ADIDISP.EXP file

(if present) to ADIDISP.OLD

Replicate the NCR800P.EXP file by copying it to ADIDISP.EXP, which

is the default AUTOCAD name for the protected-mode display driver,

in order to have an extra copy.

For example :  D:

CD \ACAD

COPY A:NCR800P.EXP

REN ADIDISP.EXP ADIDISP.OLD (if present)

COPY NCR800P.EXP ADIDISP.EXP

ACAD

****************************************************************************

****************************************************************************

AUTOCAD 11 (CONT.)

****************************************************************************

Upon entering AUTOCAD, you will need to configure the display

device. The typical configuration procedure goes as follows for

AutoCAD Release 11 :

PROMPT :  5. Configure AutoCAD

ACTION :  Press 5

PROMPT : (current configuration shown)

ACTION :  Press RETURN

PROMPT :  3. Configure video display

ACTION :  Press 3

PROMPT :  Current video display is XXX, Change <N> ?

ACTION :  Press Y, RETURN

PROMPT :  (Choices shown)

PROMPT :  1. ADI P386 v4.0/4.1 display

ACTION :  Press 1, Adjust aspect ratio if needed, choose menu,

status, command

PROMPT :  1. Show configuration

ACTION :  Press 1

****************************************************************************

****************************************************************************

AUTOCAD 11 (CONT.)

****************************************************************************

ACTION :  Verify that video display is ADI P386 v4.0/4.1, that it

says VGA Driver, by SystemSoft and also VGA 800x600

ACTION :  Press RETURN

PROMPT :  0. Exit configuration

ACTION :  Press 0

PROMPT :  Save configuration <Y> ?

ACTION :  Press Y, RETURN

PROMPT :  Back to AutoCAD's main menu

For using the driver with AutoShade, proceed the same way as for

AutoCAD to copy it to your AutoShade subdirectory. Delete the file

SHADE.CFG and proceed to load AutoShade as you normally do.

AutoShade should ask you to specify the various drivers to use.

When asked for the display driver, select :

2. P386 Autodesk Device Interface display driver

*****************************************************************************

*****************************************************************************

LOTUS123 3.0

*****************************************************************************

In order to properly add the high-resolution drivers to Lotus 1-2-3,

please first install 1-2-3 according to Lotus' installation procedure.

Once 1-2-3 3.0 is installed and running under the standard VGA

configuration, please execute the following :

1. Make your Lotus 1-2-3 subdirectory the default directory.

2. Place the Software Support Diskette 2 into drive A.

3. Type COPY A:\VGA\LOTUS3\*.* <Enter>.

4. Rename the file INSTALL.DDF to INSTALL.SAV.

5. If you have installed Lotus 1-2-3 3.0, copy the file

INSTALL.N30 to INSTALL.DDF.

6. Run the 1-2-3 installation program (ins.exe or install.exe).

7. While in the installation program, choose

First - Time Installation

*****************************************************************************

*****************************************************************************

LOTUS123 3.x (CONT.)

*****************************************************************************

8. Select one of the options.

9. Follow the normal first time installation procedure until you

are prompted for disk 1.

10. Press the F9 key twice and select

End Install Program.

Upon exit from the installation program, 1-2-3 should be ready to use.

*****************************************************************************

***************************************************************************

ARTLINE 2.0

***************************************************************************

INSTALLATION OF ARTLINE 2.0 SCREEN DRIVERS

TO INSTALL A SCREEN DRIVER:

1. Follow the installation procedure to initially install ARTLINE with

one of the standard screen drivers (eg. VGA).

2. Once you have returned to the DOS prompt, create a DRIVER PACK

DISK. Using the DOS format utility, format a blank diskette by

entering the command:

FORMAT d: /V

where d: is the drive letter. The format utility will prompt

for a volume label by displaying:

Volume Label (11 characters, Enter for none?)

At the prompt, enter the following in upper case:

GEM DRIVRPK<CR>

*****************************************************************************

*****************************************************************************

ARTLINE 2.0 (CONT.)

*****************************************************************************

3. After the FORMAT copy the files from the

Software Support Diskette 2:

COPY A:\VGA\ARTLINE\DP.* d:

where d: is the drive letter.

The   Artline   2.0  800x600  16  color   video   driver

SDNCR600.VGA  for the 77c22 super VGA chip may be installed  from

the  "driver pack" disk. The "driver pack" disk is  labeled  'GEM

DRIVRPK'.   The video driver may be installed either  during  the

initial  Artline  install, or after the Artline system  has  been

previously  installed.   In  either  case   the   screen   driver

option   "Load additional  4.0  screen Driver Packs..." must   be

selected.  The prefered  method  of screen driver installation is

to   load   the standard 640x480 16 Color VGA driver  as  shipped

with the  Artline system,  then  execute  the Artline system   to

*****************************************************************************

*****************************************************************************

ARTLINE 2.0 (CONT.)

*****************************************************************************

assure   that   the  original  disks  function  correctly.  After

successful  test  of  the Artline system type  ALSETUP  .

The following is a condensed OEM install entry procedure:

setup       /* execute Artline setup procedure */

will see..."Loading control file Install.cfg"

Change existing Installation  <yes>

Continue

*Note:

All   the  prompts  must  be  answered  as  though   a   complete

installation  were being performed. If for example you  specified

installation of some Clip Art files in the original install,  and

you did not specify those files in the OEM specific install  then

the files would be removed from the disk. The update procedure of

Artline will not recopy all files already on the disk, but checks

to see if the file is current from the previous install.

*****************************************************************************

*****************************************************************************

ARTLINE 2.0 (CONT.)

*****************************************************************************

Change Source drive if necessary (where driver pack can be found)

Continue

Load Additional 4.0 Screen Driver Packs...

Insert 4.0 Screen Driver Pack to load

will see... "77c22 SuperVGA 16-Color (800x600)"

select pointing device

select printers

select plotters

select cameras

select Clip Art

*follow the rest of setup...

Place in the Driver Pack Disk when the screen  prompt

says to do so.

After  the  installation  is  complete  change  to the root

directory:

cd \

artline        /* execute Artline system */

*****************************************************************************

*****************************************************************************

FRAMEWORK III

****************************************************************************

To install FRAMEWORK III drivers for extended graphics or text modes:

1. Make the FRAMEWORK III Directory your default directory and copy all

files from the Software Support Diskette 2

COPY A:\VGA\FRAMEWK3\*.*

2. Install FRAMEWORK III with standard VGA 640x480 16 color graphics.

3. Execute FRAMEWORK III to assure installation process accurate.

4. Run  SETUPFW.EXE the FRAMEWORK III setup program.

5. Choose (2) "All other uses of setup program"

Place to get FWSETUP from (2) current directory.

Select (1) "Configuration...hardware, memory..."

Select (1) "Hardware" from the configuration options screen.

Select (1) "Hardware Screen driver".

Select (1) "The screen driver I need isn't listed here".

Select (2) "I want to enter my own driver name".

*****************************************************************************

*****************************************************************************

FRAMEWORK III (CONT.)

*****************************************************************************

Specify driver name of (1) of the following:

OEM214.SC for 800x600 graphics, 12 scanlines, 100 column, 50 row

OEM213.SC for 800x600 graphics, 14 scanlines, 100 column, 43 row

OEM212.SC for 800x600 graphics, 16 scanlines, 100 column, 37 row

OEM230.SC for 132 column, 25 line text, explode 800x600

OEM233.SC for 132 column, 43 line text, explode 800x600

Select (M) main menu, then (7) save all settings.

Setup should write FWSETUP to the current directory (2), and any key

to exit.

Type "FW" to start FRAMEWORK III with the extended mode driver.

#