New ADF Files for the SK-NET MC2+ Board

---------------------------------------

There are 2 new ADF files for the microchannel board SK-NET MC2+ in

the directory \ADF :

1.   @6afd.nov

2.   @6afd.adv

The file @6afd.nov contains the configuration options for the easy

installation of one or more MC2+ boards in the microchannel PC. By default,

the file @6afd.adf in the root of the driver diskette is a copy of this

file.

The file @6afd.adv contains the configuration options for the installation

of MC2 boards and MC2+ boards in one microchannel PC at the same time.

To use this file as the default ADF file, please make a backup copy of the

original driver diskette and store the original diskette in a safe place.

Replace the file @6AFD.ADF in the root directory of the backup diskette

by the file @6AFD.ADV in the \ADF directory.

-----------------------------

ATTENTION !

In general, a ADF file on a system diskette of a microchannel PC will

not be replaced by a file with the same name until the new file has a

newer date and time.

That means, if you want to change the default configuration file of

this driver diskette by the file @6afd.adv (for an advanced configuration),

copy the file \adf\@6afd.adv to \@6afd.adf and change the date and time

to a newer one.

Example:	COPY \ADF\@6AFD.ADV \@6AFD.ADF

FD \@6AFD.ADF /D:[actual date]

FD is the FileDate program of Norton Utilities, if you do not have access

to this program, please use a normal ASCII editor (MS-DOS EDIT.COM), load

and save the \@6AFD.ADF file to change the date to the actual date.