NDIS DOS DRIVER LOAD AND CONFIGURATION

_______________________________________

To install NDIS DOS Lan driver you need the following items:

* LAN Support Program (LSP), LSP 1.3 is the first version supporting FDDI

* LSP documentation .

* The Option Diskette delivered with the adapter .

DRIVER INSTALLATION USING LSP

______________________________

STEP 1

______

You need first to copy the adapter driver files from the adapter

driver installation diskette to system hard disk :

- Insert LSP diskette in drive A and follow LSP instructions.

- Type at the C:\ prompt A:\DXMAID end press Enter, then follow the

instructions of the LSP documentation.

- Insert the adapter driver installation diskette into the diskette

drive when requested by LSP.

- LSP asks you to give the path where Network driver files reside

on the driver installation diskette, enter :

A:\DOS

- Validate the path : LSP copies the files to hard disk

STEP 2

______

Then , you need to configure the adapter :

- Go to "View/Change parameters" options of the LSP.

- On panel "configure workstation" :

* you can override default parameters of adapter

STEP 3

______

When the installation is complete, the Installation Aid returns to DOS.

A new AUTOEXEC.BAT and a new CONFIG.SYS have been created. If you

already had an AUTOEXEC.BAT and a CONFIG.SYS they have been saved with

the same file name and a unique numeric extension.

STEP 4

______

Modify the new AUTOEXEC.BAT provided by LSP as follows :

* add at the first line :

C:\LSP\UPPSDLI

Note : Each time you use LSP the AUTOEXEC.BAT is modified so you need

to check if the UPPSDLI is still at the first line.

* If during the hardware installation you have changed the default

parameters you need to specify them to the device driver by using the

following UPPSDLI options :

DLI Options:

-u  unload resident DLI

-o  remote boot support

-p  max protocols (1..30), default 8

-c  max multicasts (1..20), default 8

-a  node address (hex), default ROM

-k  DECnet node address (e.g. -k8.133), no default

-x  service interrupt (hex), default 7Ch

Driver Options:

-i  IRQ number, default 2

-b  base address, default SK-G16:cc, Junior: lowest

-g  SK-G16/G8 base I/O port, default lowest

-j  force usage of SK-Junior or SK-Net adapter, default SK-G16

-s  number of receive buffers (1..9), default 6

(rest of the total 10 buffers used for transmission)

-m  SK-MC2+ medium connector (AUI, THIN, 10BT), default as configured

Supported Adapters:

SK-Net, SK-NET Junior, SK-NET MC2, SK-NET MC2+,

SK-NET G16, SK-NET G16TP, SK-NET G8

STEP 5

______

Reboot your workstation to load and start the adapter .

Warning : If you have installed your adapter at the interrupt level 9

check that no other adapter in the host uses interrupt level 2. When these

two interrupts are simultaneous used you can have software problems.

End of installation.

NDIS/DOS SPECIFIC PARAMETERS

----------------------------

You will find hereafter the parameters you can modify via LSP and

their possible values.

Example section for the NDIS/DOS driver in PROTOCOL.INI:

[UPPSND_MOD]

DriverName    = "UPPSND$"

NDISREVISION  = "2.0.2"

NETADDRESS    = "020000000B03"

TOPOLOGY      = "ETHERNET"

FORCEFEEDBACK = "YES"

[UPPSND_MOD] is the name of the driver section in the PROTOCOL.INI file.

Drivers that depend on this driver must refer to this name with the option:

BINDINGS = "UPPSND_MOD"

The following driver specific parameters are defined:

_____________________________________________________

DriverName = "UPPSND$"	 : Defines the logical name of the driver.

This parameter is mandatory.

Topology = "ETHERNET"    : Defines the topology the board is running on.

Always use "ETHERNET" on the Ethernet topology.

Possible values : "FDDI", "Ethernet",

"TokenRing", "FromDLI"

Default value   : "FromDLI"

ForceFeedback = Yes      : Forces the driver to feedback a packet to the

DLI driver, even if the packet was processed (or

an application mistakenly indicates it, like

BANYAN VINES or WOLLONGONG TCP do in some cases).

Other DLI clients can now use this packet

correctly.

NDISRevision = "2.0.2"   : The UPPSND NDIS driver is loaded by default

as a NDIS 2.0.1 driver. This can be changed to

2.0.2 with the NDISREVISION parameter.

NetAddress = <addr>[L|M] : The logical net address can be changed with the

NETADDRESS parameter.

<addr> has to be a 12 character string in HEX.

L and M are optional parameters,

L means the address is in LSB mode (least significant

bit, standard on Ethernet).

M means the address is in MSB mode (most significant

bit, standard on Token-Ring).

The driver must know the topology, that's why

this parameter must not be used together with

the parameter TOPOLOGY = FROMDLI .

At least one bit of the address must be set.

The multicast-bit must not be set.

Please note: The net address can only be set in

this way, if the UPPSND.DOS driver is the

first client that is registered by the DLI driver.

Default value : ROM address of the board.