NDIS Device Driver for Microsoft LAN Manager

May 1994

Software version 2.10

To install the driver, you need the following items:

*       The MS LAN Manager Setup program

*	The UPPS Disk

Follow these steps:

1.  Start the MS LAN Manager Setup program.

2.  In the Main Menu, select "Configuration".

3.  In the Configuration Menu, select "Network Drivers".

4.  Select the option "Remove Config".  The entries of previously

installed adapters in the CONFIG.SYS and PROTOCOL.INI file will

be deleted.

5.  Select "Add New Config".

6.  When the message "Network Adapter Drivers" appears, select the

option "Other Drivers".

7.  Follow the instructions given in the LAN Manager Setup program.

The file LTE.MSG offers nationalized messages if installed in the same

directory as UPPSND.DOS.  Currently there are english (LTE001.msg) and

german versions (LTE049.MSG) available.  IF LTE.MSG is not installed,

messages show up in english.

For more information, refer to the part "Network Device Drivers" in the

Microsoft LAN Manager Installation and Configuration Guide.

NDIS DOS DRIVER PARAMETERS

You will find hereafter the parameters you can modify and

their possible values.

Note:

All hardware dependent parameters are set in the parameter line of the

DLI driver.

The DLI driver has to be loaded BEFORE any other network driver in

the AUTOEXEC.BAT file.

Example section for the NDIS/DOS driver in PROTOCOL.INI:

[UPPSND_NIF]

DriverName    = "UPPSND$"

Topology      = "Ethernet"

ForceFeedback = "Yes"

[UPPSND_NIF] is the name of the driver section in the PROTOCOL.INI file.

Drivers that depend on this driver must refer to this name with the option:

Bindings = "UPPSND_NIF"

The following driver specific parameters are defined:

DriverName = "UPPSND$"   : Defines the logical name of the driver.

This parameter is mandatory.

Topology = "Ethernet"	 : Defines the topology the board is running on.

Possible values : "FromDLI", "Ethernet",

"FDDI", "TokenRing"

Default value   : "FromDLI"

ForceFeedback = Yes	 : Forces the driver to feedback a packet to the

DLI driver, even if the packet was processed (or

an application mistakenly indicates it, like

BANYAN VINES or WOLLONGONG TCP do in some cases).

Other DLI clients can now use this packet

correctly.

NDISRevision = "2.0.2"	 : The driver is loaded by default as an NDIS 2.0.1

driver. This can be changed to 2.0.2 with the

NDISREVISION parameter.

NetAddress = <addr>[L|M] : The logical net address can be changed with the

NETADDRESS parameter.

<addr> has to be a 12 character HEX string.

L and M are optional parameters,

L means the address is in LSB mode (least significant

bit, standard on Ethernet).

M means the address is in MSB mode (most significant

bit, standard on Token Ring).

The default on FDDI depends on the selected

emulation.

The driver must know the topology, that's why

this parameter must not be used together with

TOPOLOGY = FROMDLI.

At least one bit of the address must be set.

The multicast bit must not be set.

Please note: The net address can only be set in

this way, if the UPPSND.DOS driver is the

first client that is registered by the DLI driver.

Default value : address given by DLI parameter -a

or ROM address of the board if -a parameter

is omitted.

End of instructions