SK-Net NetWare Server Driver

Installation

January 1994

Software version 3.10

SkDrv.lan is a driver for Novell NetWare 3.11, 3.12 and 4.0x.

This version fixes, in conjunction with a new .adf file, problems with

interrupts 10 and 11 (now 9 and 10) on SK-NET MC2+ adapters.

The driver corresponds to the Driver Specification Version 3.1 as specified in

document version 1.00 from March 15, 1993 with changes in October 1993.

Before starting under NetWare 3.11 you should load Patchman.nlm

(version 2.30 or higher) and LslEnh.nlm (version 1.01 or higher).

On NetWare 3.12 or 4.0x you must not load them.

MSM.nlm resp. MSM31x.nlm (version 2.20 or higher) and EtherTSM.nlm

(version 2.20 or higher) are loaded automatically if they are located in the

same directory or in the directory SYSTEM.

Under NetWare 3.11 you should use MONITOR.NLM version 1.75 or higher! You

should not use earlier versions!

On a microchannel machine with an SK-NET MC2+ inserted you may use the keyword

MEDIUM (with the possible values AUI, THIN and 10BT) to select the used

interface (the value configured with the configuration utility is changed

temporarily).

On an EISA machine you may use the keyword IGNORE_CONFIG (without values)

to use other parameters than you selected in the EISA configuration utility.

Please use this keyword only if you encounter problems with the selected EISA

configuration.

In order to avoid EISA configuration mismatches, we highly recommend that you

use the appropriate Board Configuration File (.cfg) and  configuration utility

to set the PC system resources.