+===============================================================+

|	SKETHO.SYS SK-NET OS/2 ODI Driver Installation Guide	|

+===============================================================+

v2.00

June 22, 1994

Copyright by Schneider & Koch & Co. Datensysteme GmbH. All rights reserved.

SKETHO.SYS v2.00 is an

*** Ethernet ODI Driver (MLID) for NetWare OS/2 Workstations. ***

It is intended to work with OS/2 1.x, 2.0, and 2.1.

This documentation gives useful hints on how to install and configure this

driver to get ODI Driver Interface (MLID) support on OS/2 workstations.

DUE TO CHANGES IN THE DRIVER SOFTWARE OF VERSION v2.00, THE HANDLING

IS SOMEWHAT DIFFERENT FROM THE HINTS THAT ARE GIVEN IN YOUR SK-NET

INSTALLATION GUIDE COMING ALONG WITH THE ADAPTER HARDWARE.

Major difference to versions 1.x is the GENERIC NATURE of the new driver:

With SKETHO.SYS v2.00, it is not required to have different SK-NET

drivers for SK-NET ISA, EISA, and Micro Channel adapters. Instead,

SKETHO.SYS v2.00 supports all of the following SK-NET network adapters:

ISA-type:		SK-NET G16, SK-NET G16/TP, SK-NET G8

EISA-type:		SK-NET G32+

Micro Channel type:	SK-NET MC2, SK-NET MC2+

NOTE:	The SKETHO.SYS v2.00 driver does NOT support ALL kind of

SK-NET Ethernet adapters. E.g., SK-NET JET adapters will

be ignored by this driver. SK-NET JET drivers may be found on

the SK-NET JET driver disk.

SKETHO.SYS supports frame types

- Ethernet II (type code),

- Ethernet 802.2,

- Ethernet SNAP, and

- Ethernet 802.3.

Default will be Ethernet 802.3.

_____________________________________________________________________________

CONTENTS

1	Installing the SK-NET SKETHO.SYS Driver

1.1	Installing SKETHO.SYS on OS/2 1.x

1.2	Installing SKETHO.SYS on OS/2 2.x

2	SK-NET Adapter Configuration - The NET.CFG Configuration File

2.1	How SKETHO.SYS Will Search For an SK-NET Adapter in Your Workstation

2.2	Adapter Configuration in ISA, EISA, and Micro Channel Workstations

2.2.1	How to Configure SK-NET G32+ EISA Adapters

2.2.2	How to Configure SK-NET G16, G16/TP, and G8 ISA Adapters

2.2.3	How to Configure SK-NET MC2 and MC2+ Micro Channel Adapters

2.3	Custom NET.CFG Options

3	Sample Driver Startup Screen Output

3.1	Startup Error Messages

4	Sample NET.CFG, CONFIG.SYS, and PROTOCOL.INI Files

_____________________________________________________________________________

The driver is built with Novell MSM Support v1.07 and Novell Ethernet

Support v1.06 according to Novell's Open Data-Link Interface Developer's

Guide for NetWare OS/2 Workstation Drivers v2.0.

Add Novell's ODINSUP.SYS driver to support applications that require the

NDIS interface. For more information concerning NDIS and ODINSUP, please

refer to the NetWare Requester for OS/2 manual.

Driver name for NET.CFG or PROTOCOL.INI is SKETHO.

*****************************************************************************

1	Installing the SK-NET SKETHO.SYS Driver

*****************************************************************************

The first step in installing SK-NET NetWare Requester support for OS/2

workstations is to install the SK-NET adapter hardware in your workstation

and, on EISA or Micro Channel workstations, configure this hardware using

the EISA or Micro Channel Configuration Utility, respectively. All SK-NET

adapters come along with configuration files that will guide you through

this installation process.

For more information about this installation and configuration, refer to the

adapter's manual.

NOTE:	The driver will NEVER use DMA even if the adapter would offer

DMA capabilities.

After this, reboot your system and start the installation of the driver

software. All that has to be done is to copy SKETHO.SYS to the NetWare

Requester directory and to load the driver in the CONFIG.SYS file. The

NetWare Requester for OS/2 Installation Utility coming along with the

NetWare Requester for OS/2 may help you doing this.

NOTE:	During OS/2 startup, the SKETHO.SYS driver may be prevented from

loading by pressing the ALT key.

=============================================================================

1.1	Installing SKETHO.SYS on OS/2 1.x

=============================================================================

Installing SKETHO.SYS on OS/2 1.x is in three steps:

1.	First, you will have to install the NetWare Requester for OS/2 1.x.

If you have the NetWare Requester already installed, skip to step 2.

Start the NetWare Requester for OS/2 1.x Installation Utility

and follow the installation instructions.

The Installation Utility will ask you to chose a Network Interface

Card Driver to be loaded. At this point, the Installation Utility

will give you a couple of Novell MLID drivers to choose from.

Chose, e.g., the Novell NE2000 LAN Driver NE2000.SYS and complete

the installation process. You need not reboot the system.

2.	If you have the NetWare Requester for OS/2 1.x installed on your

workstation, you should manually

- copy the SKETHO.SYS driver from the Schneider & Koch driver disk

to the NetWare Requester directory (usually C:\NETWARE), and

- edit CONFIG.SYS file in the root directory of your OS/2 boot

drive to load SKETHO.SYS instead of the driver used so far.

Example: If you had installed the NE2000.SYS driver, there is

a line

DEVICE=C:\NETWARE\NE2000.SYS

that has to be replaced by

DEVICE=C:\NETWARE\SKETHO.SYS

(supposed your NetWare Requester directory is C:\NETWARE).

Sample NetWare Requester CONFIG.SYS file entries are given in

chapter 4 at the end of this documentation.

3.	If required, configure the SKETHO.SYS driver using a NET.CFG LINK

DRIVER section. Information on how to configure the SKETHO.SYS

driver using the NET.CFG configuration file is given in chapter 2

of this documentation.

Reboot the workstation to activate the changes.

=============================================================================

1.2	Installing SKETHO.SYS on OS/2 2.x

=============================================================================

The NetWare Requester for OS/2 2.x Installation Utility is quite comfortable.

You may install the SKETHO.SYS driver along with the NetWare Requester for

OS/2; or, if you have the NetWare Requester already installed at your

workstation, you may easily switch from any driver used so far to the

SKETHO.SYS driver.

To install SKETHO.SYS from a floppy drive, you will need to have a floppy

disk with the SKETHO.SYS driver in the root (e.g., the WSDRV_1 NetWare

Requester Client Drivers for OS/2 disk, or the Schneider & Koch driver

disk). Otherwise, you may copy the SKETHO.SYS driver into the directory

where you install the NetWare Requester from, or into the directory where

you want to install the NetWare Requester into (usually, C:\NETWARE).

If there is already a NetWare Requester installed at your workstation, you

may as well copy SKETHO.SYS into the NetWare Requester directory (e.g.,

C:\NETWARE).

1A.	First, you will have to install the NetWare Requester for OS/2 2.x.

If you have the NetWare Requester already installed, skip to step 1B.

Start the NetWare Requester for OS/2 2.x Installation Utility

and follow the installation instructions. CONFIG.SYS will have to

be changed, and all the NetWare Requester files and the SKETHO.SYS

driver will have to be copied to your hard disk.

The Installation Utility will ask for the ODI LAN driver to be

loaded. Check the list if it contains the SKETHO.SYS driver; the

Installation Utility will search the directory/drive where the

NetWare Requester is installed from.

Installing from a floppy drive, you may have to insert a floppy

disk into the drive with the SKETHO.SYS file located in it's root.

If the driver is not contained in the list, type "SKETHO.SYS"

manually. The Installation Utility will fail to copy the SKETHO.SYS

driver to the NetWare Requester directory. (Do not try to type

"A:\SKETHO.SYS" or "OS2\SKETHO.SYS" or the like; installation will

not work this way.)

Anyway, proceed in installing. The Installation Utility will copy

the NetWare Requester files to the NetWare Requester directory and

prepare CONFIG.SYS to correctly load the SKETHO.SYS driver and

NetWare Requester drivers.

Skip to step 2.

1B.	If you had already installed the NetWare Requester for OS/2 on your

workstation, start the NetWare Requester for OS/2 2.x Installation

Utility, too. As source drive to load the driver file, chose your

floppy drive.

Now, instead of performing the full installation process, you just

have to update your CONFIG.SYS file to load SKETHO.SYS. Take the

respective choice, and insert into the floppy drive a disk with

the SKETHO.SYS file located in it's root. Chose SKETHO.SYS from the

list. If the driver is not contained in the list, type "SKETHO.SYS"

manually.

The Installation Utility will update your CONFIG.SYS file.

2.	If the Installation Utility had copied the SKETHO.SYS driver to

the NetWare Requester directory during step 1A, or if you did

manually, skip to step 3.

If not, you will need to perform another run through the

Installation Utility. As before, start "Requester on Workstation"

in the "Installation" menue. As source drive to load the driver

file, chose your floppy drive.

Now you just have to copy the SKETHO.SYS ODI LAN driver file to the

NetWare Requester directory. Take the respective choice, and insert

into the floppy drive a disk with the SKETHO.SYS file located in

it's root. Chose SKETHO.SYS from the list.

The Installation Utility will copy SKETHO.SYS to the NetWare

Requester directory.

(Of course, you may later on copy SKETHO.SYS from the Schneider &

Koch driver disk to your NetWare Requester directory manually, too.)

3.	Now, if required, you may configure the SKETHO.SYS driver. Chose

"This Workstation" from the "Configuration" menue of the NetWare

Requester for OS/2 2.x Installation Utility. Information on how

to configure the SKETHO.SYS driver using the NET.CFG configuration

file is given in chapter 2 of this documentation.

Exit the Installation Utility and reboot the workstation to activate the

changes.

*****************************************************************************

2	SK-NET Adapter Configuration - The NET.CFG Configuration File

*****************************************************************************

SKETHO.SYS is a driver for a couple of SK-NET ethernet adapters running on

three different types of bus systems: ISA, EISA, and Micro Channel. The

situation is somewhat complicated by the fact that ISA adapters are running

in EISA workstations, too.

The valid NET.CFG options differ on distinct types of SK-NET adapters

supported by this driver. The following table gives an overview of the

hardware option keywords that are available for the different adapters:

Adapter			available hardware options

SK-NET G8, G16, G16/TP	INT <n>, MEM <Base Address>, PORT <I/O port>

SK-NET G32+		SLOT <n>

SK-NET MC2, MC2+	SLOT <n>

* For information about the possible values for these hardware

* options, refer to the adapter's manual.

*

* The size of shared memory used will be set by the driver.

*

* For information about other option keywords in the LINK DRIVER

* section of the NET.CFG file, e.g., FRAME and PROTOCOL, refer to the

* NetWare Requester for OS/2 manual.

On startup, the driver will search for an SK-NET adapter automatically.

Anyway, it is also possible to tell the driver about the adapter it should

use. This is done by specifying in NET.CFG, in the LINK DRIVER SKETHO

section, one of the following:

- For ISA adapters (SK-NET G16, G16/TP, and G8), set

PORT <I/O port>

to tell the driver that it should use an SK-NET ISA adapter that

is configured, by DIP-switches, to I/O port <I/O port>.

- For EISA adapters (SK-NET G32+), set

SLOT <slot number>

to tell the driver that it should use an SK-NET EISA adapter that

resides in EISA slot <slot number>.

- For Micro Channel adapters (SK-NET MC2, MC2+), set

SLOT <slot number>

to tell the driver that it should use an SK-NET Micro Channel

adapter that resides in Micro Channel slot <slot number>.

In most cases, it will not be necessary to specify an adapter's I/O port

or slot number in the NET.CFG file. Instead, the driver will find the

SK-NET adapter during adapter search.

Of course, specification of I/O port rsp. slot number may be required if

- you have more than one SK-NET adapter in your workstation, so

the driver cannot decide which one to use, or

- it's an ISA adapter (SK-NET G16, G16/TP, or G8) that has its

I/O port DIP-switches set to other than default I/O port 390h.

The following sections take a closer look to NET.CFG options. Section 2.1

deals with the adapter search strategy of the driver, section 2.2 provides

detailed informations about SK-NET adapter configuration in different

types of workstations. Section 2.3 introduces additional, custom NET.CFG

options that may be used for special purposes.

=============================================================================

2.1	How SKETHO.SYS Will Search For an SK-NET Adapter in Your Workstation

=============================================================================

If the driver finds neither I/O port nor slot number in the NET.CFG file,

it will take one of the following actions to find an SK-NET adapter:

- On ISA, the driver will check default I/O port 390h for SK-NET

ISA adapters.

- On EISA, the driver will first check default I/O port 390h to

check for an ISA type of SK-NET adapter, and then scan EISA slots

1 to 15 for SK-NET EISA adapters.

- On Micro Channel, the driver will scan Micro Channel slots 1 to 8

searching for SK-NET Micro Channel adapters.

If you encounter any problems with the driver searching for an SK-NET

adapter, you should add one of the following settings to the LINK DRIVER

SKETHO section of your NET.CFG file:

- For SK-NET ISA adapters, i.e. SK-NET G16, G16/TP, G8, specify

the I/O port that is configured on the adapter board. E.g., if

the adapter I/O port DIP-switches are set to I/O port 320h:

LINK DRIVER SKETHO

PORT 320

- For SK-NET EISA and Micro Channel adapters, i.e. SK-NET G32+,

MC2, MC2+, specify the slot number where the adapter resides.

E.g., if the adapter resides in slot 5:

LINK DRIVER SKETHO

SLOT 5

Whenever you specify an I/O port in an ISA adapter's LINK DRIVER SKETHO

section, the driver will just check this specified I/O port for SK-NET

ISA adapters SK-NET G16, G16/TP, and G8.

Whenever you specify a slot number, but no I/O port, the driver will just

check this specified slot for SK-NET EISA or Micro Channel adapters.

=============================================================================

2.2	Adapter Configuration in ISA, EISA, and Micro Channel Workstations

=============================================================================

The following sections take a closer look on how to use NET.CFG entries

configuring SK-NET adapters in EISA, ISA, and Micro Channel workstations.

-----------------------------------------------------------------------------

2.2.1	How to Configure SK-NET G32+ EISA Adapters

-----------------------------------------------------------------------------

EISA adapters running in EISA workstations will not learn their configuration

from the NET.CFG file. Instead, there is an EISA configuration utility that

will write the actual configuration to the computer's CMOS configuration RAM.

See your computer's documentation for details concerning EISA configuration.

SK-NET adapters come along with an EISA configuration file providing the

range of possible adapter configurations.

SK-NET G32+ is an SK-NET adapter designed for EISA computers. For this type

of boards, the EISA configuration utility will provide configuration details

to the driver. Thus the only thing to be done in the NET.CFG file is to

specify the number of the EISA slot where the adapter is installed:

LINK DRIVER SKETHO

SLOT 4

tells the driver to check EISA slot 4 for an SK-NET EISA adapter.

Other hardware configuration details, as interrupt line and shared memory

area, will be set by the EISA configuration utility directly.

-----------------------------------------------------------------------------

2.2.2	How to Configure SK-NET G16, G16/TP, and G8 ISA Adapters

-----------------------------------------------------------------------------

If you want to specify an SK-NET G16, SK-NET G16/TP, or SK-NET G8 adapter,

you should set the I/O port of the adapter in NET.CFG (according to the

DIP-switch settings on the board):

LINK DRIVER SKETHO

PORT 320

will cause the driver to look for SK-NET G16, G16/TP, and G8 adapters on

I/O port 320h. Adapters residing on other I/O ports will not be accessed

by this driver instance.

If you do not specify an I/O port in NET.CFG, the driver will assume the

default I/O port 390h and will look for an adapter there.

Other hardware configuration details, as interrupt line and memory base

and range, will have to be given using INT and MEM keywords if they differ

from the driver's default values. Default values are:

INT 5		- is interrupt 5,

MEM CC000	- is 16k Shared Memory starting at 000CC000h,

PORT 390	- is I/O port 390h.

Example:

LINK DRIVER SKETHO

FRAME ETHERNET_II

PROTOCOL IPX 8137 ETHERNET_II

PORT 320

MEM C0000 400

INT 11

will be a correct NET.CFG setting if

the adapter will have to transfer IPX packets using the ETHERNET_II

frame type,

the I/O port DIP-switches of the adapter are set to port 320h,

the adapter's 16kByte Shared Memory shall be set to 000C0000h,

and the adapter shall use interrupt 11.

NOTE:	On modern PC systems, interrupts 2 and 9 are identical. Thus for

SK-NET G16, G16/TP, and G8, specification of both INT 2 and INT 9

will be valid and will activate the same interrupt line.

*** SK-NET ISA Adapters in EISA Workstations ***

Of course, it is possible to install an SK-NET ISA adapter (as SK-NET G16,

SK-NET G16/TP, or SK-NET G8) in an EISA workstation. But, in fact, the

EISA configuration utility will not to set hardware configuration details

on ISA boards.

Instead, you will have to configure the adapter using NET.CFG entries

in exactly the same way as in an ISA computer.

We highly recommend you to additionally set the configuration of the

adapter in the EISA system configuration utility to tell the EISA system

about the hardware resources that the adapter requires. For this to be

done, SK-NET ISA adapters come along with an EISA configuration file

providing the range of possible adapter configurations.

NOTE:	THE SETTINGS OF EISA CONFIGURATION AND NET.CFG CONFIGURATION

SHOULD BE IDENTICAL TO PREVENT THE SYSTEM FROM CRASHING DUE TO

CONFIGURATION ERRORS!

-----------------------------------------------------------------------------

2.2.3	How to Configure SK-NET MC2 and MC2+ Micro Channel Adapters

-----------------------------------------------------------------------------

Micro Channel adapters will not learn their configuration from the NET.CFG

file. Instead, there is a Micro Channel configuration utility that will

deal with configuration. See your computer's documentation for details

concerning Micro Channel configuration.

SK-NET adapters come along with a Micro Channel configuration file providing

the range of possible adapter configurations.

SK-NET MC2 and SK-NET MC2+ are adapters designed for Micro Channel computers.

For these types of boards, the Micro Channel configuration utility will

provide configuration details to the driver. Thus the only thing to be done

in the NET.CFG file is to specify the number of the Micro Channel slot where

the adapter is installed:

LINK DRIVER SKETHO

SLOT 4

tells the driver to check Micro Channel slot 4 for an SK-NET adapter.

Other hardware configuration details, as interrupt line and shared memory

area, will be set by the Micro Channel configuration utility directly.

NOTE:	SK-NET MC2+ adapters allow for interrupts 3, 5, 9, and 10, whereas

SK-NET MC2 may work with interrupts 3, 5, 10, and 11.

=============================================================================

2.3	Custom NET.CFG Options

=============================================================================

This section describes an additional, custom NET.CFG option that may be

used on SK-NET G32+ or SK-NET MC2+ adapters to temporarily switch between

adapter media interfaces

Thin Ethernet (BNC connector jack, for Cheapernet),

AUI (DIX connector plug, for Thick Ethernet = Yellow Cable), and

10Base-T (RJ45 connector, for Twisted Pair Cabling).

The NET.CFG keyword is MEDIUM, followed by the choices THIN, AUI, or 10BT,

respectively.

Example:

LINK DRIVER SKETHO

MEDIUM THIN

will cause the adapter to activate the BNC Cheapernet cable connector.

IN ORDER TO AVOID CONFIGURATION ERRORS WE HIGHLY RECOMMEND TO USE THE

APPROPRIATE CONFIGURATION UTILITY OF THE PC SYSTEM TO SWITCH BETWEEN

MEDIUM CONNECTORS ON SK-NET G32+ AND SK-NET MC2+.

*****************************************************************************

3	Sample Driver Startup Screen Output

*****************************************************************************

The following section contains sample driver output as it may appear on

workstation startup.

The following is the SKETHO.SYS driver startup output that may result

from NET.CFG

LINK DRIVER SKETHO

FRAME ETHERNET_II

PROTOCOL IPX 8137 ETHERNET_II

PORT 320

settings:

_____________________________________________________________________________

SK-NET Generic Ethernet MLID  v2.00 (940622)

(C) Copyright 1993, 1994 Schneider & Koch. All Rights Reserved.

Searching for SK-NET adapters:

Checking for SK-NET G8/G16 adapters on I/O port given in NET.CFG...

Found SK-NET G8/G16 adapter! Parsing NET.CFG parameters...

Adapter 1

Int 5, Port 0320, DMA Not Used, Mem 000CC000

Max Frame 1514 Bytes, Line Speed 10 Mbps, Node Address 00005A123456

Board 1, Frame ETHERNET_II

_____________________________________________________________________________

Of course, this output may vary on different adapters and configuration

settings. Additional messages may be given to inform you about current

driver activities and hardware settings:

Searching for SK-NET adapters:

Checking for SK-NET G8/G16 adapters on I/O port given in NET.CFG...

Checking for SK-NET G32+ adapters on the slot given in NET.CFG...

Checking for SK-NET MC2/MC2+ adapters on the slot given in NET.CFG...

Found SK-NET G8/G16 adapter! Parsing NET.CFG parameters...

Found SK-NET G32+ adapter:

Found SK-NET MC2 adapter:

Found SK-NET MC2+ adapter:

Found more than one SK-NET adapter!

NOTE: Interrupt 3 may be disabled (DIP-switch) or in use by COM2!

NOTE: Interrupt 4 is available on SK-NET G8 only and may be in use by COM1.

NOTE: Interrupt 11 requires SK-NET G16 or G16/TP in a 16-bit slot.

Transceiver type is configured to be Thin Ethernet (BNC).

Transceiver type is configured to be 10BaseT (Twisted Pair).

Transceiver type is configured to be AUI (Thick Ethernet).

NET.CFG node address is multicast; will be ignored.

If you have more than one SK-NET ethernet adapter installed in one

workstation, each instance of the driver will output informations of

its own.

=============================================================================

3.1	Startup Error Messages

=============================================================================

If there are any initialization errors, an error code will be printed

past the above output, followed by a short error message. In case of

an error, some of the resource values given in the output may not yet be

set correctly. E.g., if no adapter had been found on I/O port 100h

given in NET.CFG, the output would look like this:

_____________________________________________________________________________

SK-NET Generic Ethernet MLID  v2.00 (940622)

(C) Copyright 1993, 1994 Schneider & Koch. All Rights Reserved.

Searching for SK-NET adapters:

Checking for SK-NET G8/G16 adapters on I/O port given in NET.CFG...

Adapter 1

Int Not Used, Port 0100, DMA Not Used, Mem Not Used

Max Frame 1514 Bytes, Line Speed 10 Mbps, Node Address FFFFFFFFFFFF

Board 1, Frame ETHERNET_II

MLI0207:  Did not find an SK-NET ethernet adapter.

SYS1201: The device driver "C:\NETWARE\SKETHO.SYS" specified in the DEVICE

command on line 73 of the CONFIG.SYS file was not installed. Line 73 is

ignored.

Press Enter to continue.

_____________________________________________________________________________

In this case, the Int, Mem and Node Adress values given are invalid since

the driver does not set appropriate values for a non-existing adapter.

The driver will not load and in consequence, the LSL, IFS.SYS, and NetWare

Requester drivers will not load, too.

The following list holds some example error messages that may occur on

SKETHO.SYS startup:

MLI0207: Did not find an SK-NET ethernet adapter.

MLI0208: Please select an adapter slot in your NET.CFG file.

MLI0209: NET.CFG INT number is invalid for this board.

MLI0210: Shared Memory configuration error.

MLI0211: NET.CFG PORT setting is invalid for G8/G16.

MLI0212: NET.CFG SLOT setting is invalid.

MLI0214: Error during board init.

MLI0215: Adapter not responding!

MLI0216: Adapter RAM access error!

MLI0217: Selected INT is in use and is not shareable.

MLI0218: SK-NET Ethernet OS/2 ODI Driver terminated by ALT-Key.

If one of these errors occurs, take a look into the following table for

possible reasons and ways to come around them:

ERROR	POSSIBLE CAUSE			WHAT SHOULD BE DONE

MLI0207 The driver did not find an	Check the hardware configuration

SK-NET ethernet adapter which	(board DIP-switch settings resp.

it was able to use.		EISA / Micro Channel system

configuration). See section 2 for

hints about using NET.CFG.

MLI0208 The driver found more than one	Select one of the adapters by

SK-NET adapter during EISA or	setting a SLOT <slot number> entry

Micro Channel slot search.	into the LINK DRIVER SKETHO section

Thus it could not decide which	of your NET.CFG file.

one to use actually.

MLI0209 The interrupt line chosen in	Choose a valid interrupt line.

your NET.CFG file is not valid	For informations about available

for the kind of adapter used.	values, refer to the adapter manual.

MLI0210 The Shared Memory area chosen	Choose a valid Shared Memory area.

in your NET.CFG file or set by	For informations about available

the (EISA / Micro Channel)	values, refer to the adapter manual.

system configuration utility

is not valid for the kind of

adapter used.

MLI0211 The I/O port value given in	Set a valid I/O port address in your

your NET.CFG file is not	NET.CFG file (keyword PORT).

valid for the kind of		For informations about available

adapter used.			values, refer to the adapter manual.

MLI0212 The slot number given in	Set a valid slot number in your

your NET.CFG file is not	NET.CFG file (keyword SLOT).

a valid slot number.		EISA slots may range from 1 to 15,

Micro Channel slots from 1 to 8.

MLI0214 This default error message should not occur on well-configured

OS/2 systems.

MLI0215 This error may occur if the	Use another interrupt line out

configured interrupt line is	of the available values stated in

in use by another device, or	the adapter manual.

if the adapter hardware does

not work correctly.

On G8, G16, or G16/TP, a	To use interrupt 3 on G8, G16, or

DIP-switch may disable		G16/TP, enable the INT 3 DIP-switch.

interrupt 3.

MLI0216 The Shared Memory of the	Some VGA adapters force 16-bit RAM

adapter does not work as	access to their RAM segment and thus

expected.			may cause this message to appear on

8-bit ISA adapters. In this case,

try other Shared Memory settings.

If nothing works, it might be

a hardware or bus system problem.

MLI0217 The interrupt line selected	Use another interrupt line out

by your NET.CFG or system	of the available values stated in

configuration setting is	the adapter manual.

already in use.

MLI0218 The ALT-key had been pressed	The driver will load only if the ALT-

during startup of the driver.	key is not pressed during startup.

*****************************************************************************

4	Sample NET.CFG, CONFIG.SYS, and PROTOCOL.INI files

*****************************************************************************

This section contains some sample NET.CFG, CONFIG.SYS, and PROTOCOL.INI

files.

----- SAMPLE NET.CFG: -----

Link Driver SKETHO

PORT 320

MEM D0000

INT 9

Protocol ODINSUP

BIND SKETHO

----- SAMPLE PROTOCOL.INI for using ODINSUP: -----

[PROTMAN]

DRIVERNAME = PROTMAN$

[NETBEUI_NIF]

DRIVERNAME = NETBEUI$

BINDINGS = SKETHO

[SKETHO]

----- SAMPLE CONFIG.SYS Entries: -----

These CONFIG.SYS entries will automatically be set by the Netware OS/2

Requester installation procedure.

REM --- NetWare Requester statements BEGIN ---

DEVICE=C:\NETWARE\LSL.SYS

RUN=C:\NETWARE\DDAEMON.EXE

rem Now comes our SK-NET Ethernet driver.

DEVICE=C:\NETWARE\SKETHO.SYS

rem The following line is required only if you want NDIS support.

DEVICE=C:\NETWARE\ODINSUP.SYS

DEVICE=C:\NETWARE\IPX.SYS

DEVICE=C:\NETWARE\SPX.SYS

RUN=C:\NETWARE\SPDAEMON.EXE

rem DEVICE=C:\NETWARE\NMPIPE.SYS

rem DEVICE=C:\NETWARE\NPSERVER.SYS

rem RUN=C:\NETWARE\NPDAEMON.EXE NP_COMPUTERNAME

DEVICE=C:\NETWARE\NWREQ.SYS

IFS=C:\NETWARE\NWIFS.IFS

RUN=C:\NETWARE\NWDAEMON.EXE

rem DEVICE=C:\NETWARE\NETBIOS.SYS

rem RUN=C:\NETWARE\NBDAEMON.EXE

DEVICE=C:\NETWARE\VIPX.SYS

DEVICE=C:\NETWARE\VSHELL.SYS

REM --- NetWare Requester statements END ---

=============================================================================