NDIS Device Driver for DEC Pathworks

Use the NDIS driver UPPSND.DOS, if you are using DEC Pathworks with

SK-NET Boards.

INSTALLATION

------------

Insert the first diskette of DEC Pathworks into the floppy drive A: (or B:)

and start NETSETUP.

Follow the instructions of NETSETUP.

When NETSETUP asks for a network adapter, select NDIS.

Type in the correct path to the NDIS driver on the driver diskette

( A:\PCSA\ ) and the correct path and name of the PROTOCOL.INI STUB file

( A:\PCSA\PROTOCOL.INI ).

Change the default value for the HEURISTICS parameter from 0 to 12.

HEURISTICS = 12

This is neccessary, if network adapters others than DEPCA card are used.

HEURISTICS is a parameter in the [DATALINK] section of the file PROTOCOL.INI .

Answer the following questions of NETSETUP. Finally Pathworks for DOS

should be installed successfully.

The DLI driver (UPPSDLI.COM, UPPSJET.COM or UPPSTR.COM) has to be loaded BEFORE

starting DEC Pathworks for DOS !!!

NDIS DOS DRIVER PARAMETERS

__________________________

You will find hereafter the parameters you can modify and

their possible values.

Note:

All hardware dependent parameters are set in the parameter line of the

DLI driver.

The DLI driver has to be loaded BEFORE any other network driver in

the AUTOEXEC.BAT file .

Example section for the NDIS/DOS driver in PROTOCOL.INI:

[UPPSND_NIF]

DriverName    = "UPPSND$"

TOPOLOGY      = "ETHERNET"

[UPPSND_NIF] is the name of the driver section in the PROTOCOL.INI file.

Drivers that depend on this driver must refer to this name with the option:

BINDINGS = "UPPSND_NIF"

The following driver specific parameters are defined:

_____________________________________________________

DriverName = "UPPSND$"   : Defines the logical name of the driver.

This parameter is mandatory.

Topology = "ETHERNET"    : Defines the topology the board is running on.

Possible values : "FDDI" , "Ethernet" ,

"TokenRing" , "FromDLI"

Default value   : "FromDLI"

Emulate = "TokenRing"    : Defines the emulation that is used by the

driver on FDDI.

Possible values : "TokenRing" , "Ethernet"

Default value   : "TokenRing"

BanyanSupport = Yes      : Supports BANYAN VINES on Token-Ring, because

BANYAN VINES Version 4 does not support a

special NDIS driver on Token-Ring.

ForceFeedback = Yes      : Forces the driver to feedback a packet to the

DLI driver, even if the packet was processed (or

an application mistakenly indicates it, like

BANYAN VINES or WOLLONGONG TCP do in some cases).

Other DLI clients can now use this packet

correctly.

NDISREVISION = "2.0.2"   : The UPPSND NDIS driver is loaded by default

as a NDIS 2.0.1 driver. This can ba changed to

2.0.2 with the NDISREVISION parameter.

NETADDRESS = <addr>[L|M] : The logical net address can be changed with the

NETADDRESS parameter.

<addr> has to be a 12 character HEX string.

L and M are optional parameters,

L means the address is in LSB mode (least significant

bit, standard on Ethernet).

M means the address is in MSB mode (most significant

bit, standard on Token-Ring).

The default on FDDI depends on the selected

emulation.

The driver must know the topology, that's why

this parameter must not be used together with

TOPOLOGY = FROMDLI .

At least one bit of the address must be set.

The multicast bit must not be set.

Please note: The net address can only be set in

this way, if the UPPSND.DOS driver is the

first client that is registered by the DLI driver.

Default value : address given by UPPSFE parameter

-a or ROM address of the FDDI board if -a parameter

is missing.

NETSTATUSMASK = "mmmmmmmmmmmmmmmm" :

With NETSTATUSMASK you can switch off the Network

Status Events on FDDI bit by bit. 'm' has to be

'0' (off) or '1' (on) .

Use this parameter only for testing (e.g. MtTool) .

End of instructions

For further information about the UPPSND.DOC driver please read the

DOC file in the directory \MSLANMAN.DOS\DRIVERS\ETHERNET\UPPSND .