SK-NET Ethernet Driver for Windows NT

Installation

March 1994

Software version 1.00

SKETHNT.SYS is the driver for the SK-Net G8, SK-Net G16, SK-Net G16/TP,

SK-Net MC2, SK-Net MC2+ and SK-NET G32+ ethernet adapters under the first

release of Windows NT on Intel processors.

The driver corresponds to the NDIS 3.0 Specification as delivered with

the first release of Windows NT.

Files needed

In the root of the install diskette

DISK3		Indicator.

Preferrably in the root

!SKD0200.cfg	New EISA config file for SK-NET G32+

!SKDE000.cfg	EISA config file for SK-NET G16

!SKDE300.cfg	EISA config file for SK-NET G8

!SKDE400.cfg	EISA config file for SK-NET G16/TP

In a single directory

OEMNADSK.HLP	Help file for SK-NET drivers.

OEMSETUP.INF	Setup script for ethernet driver.

SKETHNT.SYS	The driver.

How to install

For the SK-NET G32+ you MUST use the new config file supplied.

On an EISA machine the SK-NET ISA adapters MUST be configured using

the appropriate EISA configuration file.

After having configured your adapter, please start Windows NT. There,

start NETWORK from the CONTROL PANEL, choose ADD ADAPTER and then

<OTHER> and type in the path to OEMSETUP.INF.  Choose the driver

presented and select the appropriate configuration.  Then choose OK

and restart your system.  (Protocol software should be installed.)

NOTE:	The ISA cards (SK-NET G8, SK-NET G16, SK-NET G16/TP) will not run

in SIEMENS EISA machines which do parity checks in the shared memory

(e.g. PCE-4T/33, PCE 5/S, ...). Please use an SK-NET G32+ in these

machines.

Troubleshooting

If you encounter problems using the SK-NET Ethernet adapter under

Windows NT, please re-read carefully the instructions above and the

documentation supplied with the system. Additionally, you should look

into your system log file (using the event viewer) and check it for

entries from SKETHNT. The last DWORD of the logging data is the slot

number (or, if not present the IO port) of the adapter that caused the

error. The DWORD before is a unique number for some errors that appear

in several places. Following is a list of some events, possible reasons

(may be incomplete) and suggestions to fix your problem.

Event ID:	5003 -	Could not find an adapter.

Reason(s):	Wrong slot or IO Port

Adapter physically not installed.

EISA configuration information disappeared.

Help:		Check slot/port settings.

Look if adapter is physically present.

Look if (ISA) adapter is (EISA) configured.

---------------

Event ID:	5004 -	Could not connect to the interrupt number

supplied.

Reason(s):	The selected interrupt is already in use.

Help:		Select free interrupt.

---------------

Event ID:	5005 -	Has encountered an internal error and has

failed.

Reason(s):	???

Help:

---------------

Event ID:	5007 -	Timed out during an operation.

Reason(s):	Wrong interrupt, dma channel or base address.

Cable not connected.

Help:		Start a driver under DOS (with the same resource usage)

and look if it works.

If your special problem is not listed or the error persists, please

send us your system log file or at least the event id number and the

logging data (as dwords) of the entries written by SKETHNT.