DATA TRANSFER TEST (NSPD_E.EXE)

(c) Copyright Cabletron Systems, Inc. 1990, 91, 92

The Data Transfer Test (NSPD_E.EXE) is an application written

specifically for testing the integrity of an ethernet network.  The "ping"

test is compatible with both Cabletron Systems' LAN MD and Cabletron

Systems' Macintosh Node Server.

NOTE:  The Data Transfer Test is meant to be used for diagnostic purposes

only and may be volatile to a network which may be running another

application.

To start the Data Transfer Test, which is located in the DIAGS

directory on the floppy diskettes provided with your Cabletron Systems

DNI Card, type NSPD_E at the system prompt and hit <RET>.

NOTE:  The Cabletron Systems Generic Packet Driver, ExxPD.EXE, must be

loaded prior to executing the Data Transfer Test.  The file is

located on the floppy diskettes provided with your Cabletron

Systems DNI Card.  Follow the directions included in the file

csipd_e.doc to install the driver.

When used correctly, the Data Transfer Test may be a valuable

tool.  The test can verify that the connection between the DNI card and

the network is functional and it can test the integrity of the communication

channels between nodes on the network.  The test also includes a function

which allows the user to create traffic on the connected network.

Under normal circumstances the Data Transfer Test should be set up

as follows.  The test requires that the diagnostic be loaded onto at least

two systems that are connected to the network, at least one of which should

be declared as a server.  Up to 14 nodes may act as a server.  All of the

other nodes on the network which are running the program should be declared

as being workstations.  Once all of the nodes have been declared as being

a server or workstation, activate the servers by hitting <RET>.  Now activate

the workstations in the same manner.  A message will appear telling the user

to wait while a list of active servers is constructed.  After 30 seconds,

provided an active server was found, a menu will be displayed and the

server to which communication is desired may be selected.  If an active

server was not found, the workstation will attempt to find a server up to

five times before an error is displayed.

To change the size of the packets the workstation is transmitting, use

the '-' and '+' keys.  Packet sizes can be incremented in intervals of 10

bytes between the lengths of 50 and 1500 bytes.  A packet size of 46 bytes

may be transmitted as well.  To send packets of variable size, hit the '-'

or '+' keys until the word "variable" is displayed.  The workstation will

transmit packets starting with a length of 46 bytes and will increment the

packet length by one byte each time a new packet is transmitted.  Packets

of random size may be transmitted as well.  Hit '-' or '+' until the word

"random" is displayed.  This will tell the workstation to transmit packets of

random length.

A transmit delay feature is included with the workstation.  By hitting

the '#' key, the delay inserted between packet transmitions may be incremented

on a scale of 0 - 10.  If a transmit delay of 0 is selected, no delay will be

inserted.  If a delay of 10 is selected, the longest possible delay will be

inserted.

If a bad packet is received by the workstation, the bad byte will be

displayed at the bottom of the screen along with the corresponding byte from

the packet that was transmitted.  This will allow for a visual comparison to

be made.

The software also allows the user to designate the node as a

transmitter.  When activated, the transmitter will transmit packets

continuously.  The packet size and transmit delay features, included when

the node is declared as a workstation, are included with this feature as

well.  The transmit delay that is inserted between packets is three times

longer than that which is inserted when acting as a workstation.  This feature

is useful to those that would like to generate traffic on the network and

does not require another node to be running this software.

The receiver function is included for use by a technician for low

level testing.

The Data Transfer Test is menu driven and includes complete help

screens to which the user may refer.