SSAFLASH Installation for RS/6000 SP                        11/14/96

----------------------------------

This describes how to implement the SSAFLASH process

on an IBM RS/6000 Scalable POWERParallel System (9076). This

involves obtaining the installation diskettes and then

installing the new microcode on the nodes of an SP2 system.

In an SP, you need to perform on each node what would normally be done on

a standalone RS/6000b but without the benefit of a diskette drive in

the node. To do this, you must distribute the code to all

the affected nodes, then execute it there. There are several ways of doing

this and it depends on your experience and preferences which will work

best for you.

First, obtain the latest version of SSAFLASH from AIXTOOLS. The microcode

included there is updated at times, so it is best to check. Also the

TOOLS shadow you access may not be up to date. If you suspect it is not,

use a server such as YKTVMV. This seems to be reasonably up to date.

Read and understand the README and instructions that came with

SSAFLASH.

In all cases, the following steps need to be done.

1. Obtain/create three diskettes.

ADAP2037.bff

DISK8381.bff

6561.bff

Refer to the SSAFLASH README for instructions to create the

diskettes.

SSAFLASH also contains dasd microcode.

The various levels are described in the SSAFLASH README file

2. Check to see if you need to install the microcode fix.

3. Transfer the contents of the three diskettes to the Control

Workstation.

4. Distribute the files to the nodes to be tested

5. Install the files on the nodes

6. Shutdown and Reboot the nodes

7. Check that the new microcode has been applied to the adapters.

---------------------------------------------------------

NOTE: Refer to the SSAFLASH README for instructions to obtain the diskettes.

-------------------------------------------------------------------------

INSTALLATION PROCEDURE

----------------------

1) Login as "root" to the Control Workstation:

login root

NOTE:     All the following commands shown will be run from a window on the

Control Workstation. It should not be necessary to telnet or rlogin

to the individual SP nodes. You should be familiar with the "dsh"

and "pcp" commands. If not, please refer to the SP Administration

Guide or Commands Reference.

2) Make and go to a work directory to make cleaning up easier when you're done.

mkdir /tmp/SSAFLASH

cd /tmp/SSAFLASH

3) Export this directory to "everybody".

exportfs -i /tmp/SSAFLASH

4) Prepare a Working Collective file to permit you to use the "dsh"

command to install to all the affected nodes in parallel.

Note:   You may also use the "dsh -w host1,host2,host3... command"

format to explicitly exercise groups of nodes rather than

use the working collective if you only need to do a few nodes.

o      Make a file containing a list of the nodes that you want

to perform the update on (in preparation for the dsh command)

(Press ENTER after each command)

cat > group1

nodename1

nodename2

nodename3......

CTRL-D

export WCOLL=./group1

- Or use your favorite editor (e.g. vi) to create the file.

o  You may test this to ensure it is working the way you

expect by typing " dsh date "

o  You should get back a list that looks something like:

nodename1:   Wed Apr 10 10:37:46 EDT 1996

nodename2:   Wed Apr 10 10:37:46 EDT 1996

nodename3:   Wed Apr 10 10:37:47 EDT 1996

nodename4:   Wed Apr 10 10:37:48 EDT 1996

If not, examine your nodelist file ./group1 and

also ensure that the Kerberos ticket is current to permit

"rsh" to be performed. If not, you may need to refresh it.

Refer to the SP Administration Guide (GC23-3897) for further

help. The System Administrator should also be able to help

you with Kerberos initialization. If not, please consult

your support center.

5) Check to see if the new microcode needs to be installed:

This code replaces all previous levels and will function on all

previous and current releases of the adapter hardware.

To determine what level of microcode your machine has installed,

type the following command from an AIX prompt:

dsh lscfg -vl ssa*   (where * is the device number)

or

dsh lscfg -vl ssa* | pg

If the data returned matches one of the following adapters and

the "ROS Level and ID..." is:

2001               2002              2904

(2-way FC6214)   (8-way FC6216)   (RAIDFC6217)

then this new level of microcode is already applied. If you see

a lower number, you will require this package.

To determine what level of microcode your drives have installed,

issue the following command at the AIX prompt:

dsh lscfg -vl pdisk* -v   (where * is the device number)

or

dsh lscfg -vl pdisk* -v | pg

If the "ROS level and ID ..." is :

6561 or  8372  or  8181 (SEE SSAFLASH README FILE)

then this new microcode package is already loaded. If you see a

lower number, then you will require this package.

6) Transfer the installation code from diskette to the Control Workstation

a. Insert a diskette you prepared previously into

the Control Workstation's drive.

Get the SSAFLASH package from Omnidisk Aixtools, Store in reader,

Transfer file as binary from reader to a: drive on MoST.

b. Transfer the file from the diskette to the Control Workstation:

bffcreate -d /dev/fd0 -t /tmp/SSAFLASH -X -q -v all

If you created your diskettes from a DOS machine, then there is

an additional step:

Must use the dosread command. (The majority of CE's have the MoST

thinkpads and will be copying the EC to a DOS-formatted diskette.

Insert the adap2029.bff diskette in the CW reader and enter:

dosread adap2037.bff /tmp/disk1

Then do the same for the 6561.bff diskette:

dosread 6561.bff /tmp/disk2

The next diskette:

dosread DISK8381.bff /tmp/disk3

bffcreate -d /tmp/disk1 -t /tmp/SSAFLASH -X -q -v all

bffcreate -d /tmp/disk2 -t /tmp/SSAFLASH -X -q -v all

bffcreate -d /tmp/disk3 -t /tmp/SSAFLASH -X -q -v all

c. Repeat the above command for each of the 3 disks.

- this will transfer the files from the diskette

to the directory and update the ".toc" file.

d. You may check that they got transferred correctly using

the following:

installp -l -d /tmp/SSAFLASH

You should see results similar to:

>> installp -l -d /tmp/SSAFLASH

Option Name                 Level                     I/U Q Content

====================================================================

ssadiskmc.mc                03.02.0000.0061            I  N usr

#   SSA DASD Microcode

ssadiskmcode.DCHC.obj       03.02.0000.0013            I  N usr

#   SSA DCHC Disk ROS microcode

ssadiskmcode.DFHC.obj       03.02.0000.0016            I  N usr

#   SSA DFHC Disk ROS microcode

ssamcode.base.obj           03.02.0000.0020            I  N usr

#   SSA Adapter microcode

ssamcode.enh.obj            03.02.0000.0020            I  N usr

#   SSA Enhanced Adapter microcode

ssamcode.raid.obj           03.02.0000.0029            I  N usr

#   SSA RAID Adapter microcode

7) Ensure that the appropriate directory is exported to "everybody".

showmount -e

For example, the results should include /tmp/SSAFLASH

(among several possible others).

8) NFS Mount the installation directory over /mnt on the nodes:

dsh mount <controlwks>:/tmp/SSAFLASH /mnt

- Where <controlwks> is the name of the Control Workstation known

to the nodes on the system Ethernet or its IP address.

9) You may wish to test your setup before commiting to the install

by doing:

dsh installp -l -d /tmp/SSAFLASH

You should see results similar to those in step 6 except it will

be repeated for all the nodes in your group.

10) Perform the installation on the applicable nodes, typing:

dsh installp -acXd /mnt all | tee SSAFLASH.log

(The SSAFLASH.log file will contain the messages that occurred

during the install for all the nodes.)

10a) IF YOU HAVE AIX 4.1.5 installed, with the SSA RAID Adapter, you

will have to use the folowiing to install the dasd microcode:

dsh ssaload -u

- this will update all the disks to the latest level.

OR

dsh -w <nodename> ssaload -d <pdiskname> -f <codefilename>

- This will update selected disks to a selected level on a selected node.

NOTE:  Refer to your SSA Adapter User's guide (SA33-3272)

for details about the "ssaload" instruction.

11) Shutdown and Reboot

dsh "shutdown -Fr &"

12) Check to see that the new microcode has been applied to the adapters

and disks:

dsh lscfg -vl ssa*

or

dsh lscfg -vl ssa* | pg

Check - ROS LEVEL AND ID = SEE SSAFLASH README FILE

dsh lscfg -vl pdisk* -v

or

dsh lscfg -vl pdisk* -v | pg

Check - ROS LEVEL AND ID = SEE SSAFLASH README FILE

13)  Cleanup after you finish.

a)      dsh /usr/sbin/umount /mnt

This command is not necessary since a reboot of the nodes should

clear the mount but it won't hurt anything if entered.

b)      exportfs -u /tmp/SSAFLASH

NOTE: You might want to omit the following steps in case you plan on

later installating more disks or adapters.

c)      rm  /tmp/SSAFLASH/*

d)      rmdir /tmp/SSAFLASH

------------------------------------------------------------------------------

References:

-----------

1. GC23-3897-01 IBM RISC System/6000 Scalable POWERparallel Systems

Administration Guide

2. GC23-3900-01 IBM RISC System/6000 Scalable POWERparallel Systems

Command and Technical Reference

3. SA33-3272

��####SSAFLASH                                                               # #