SSA MICROCODE

INSTALLATION INSTRUCTIONS

INSTALL SSAFLASH

MICROCODE FOR SSA 4-PORT ADAPTER

FEATURE CODE 6214

MICROCODE FOR ENHANCED SSA 4-PORT ADAPTER

FEATURE CODE 6216

MICROCODE FOR SSA RAID ADAPTER

FEATURE CODE 6217/6218

MICROCODE FOR SSA DISKS DFHC/DCHC

FIELD USE ONLY

SEP 11,1997

SSD 7133 PE

MP26/19 Havant

external 01705-486363

internal (7)-215732

NWARWICK at WINVMD

Written By:   D Sinclair/N Warwick

Approved By:  EC Management Team

1.0  USING THE NEW  ADAPTER AND DRIVE MICROCODE DISKETTES

___________________________________________________________________

This document describes how to install the new microcode for the SSA Adaptors

and SSA Dasd.  The microcode is supplied on two diskettes:

o   Adapter microcode diskette containing file adap2137.BFFBIN

o   Disk Drive microcode diskette containing file DISK8381.BFFBIN

o   Disk Drive microcode diskette containing file 6561.BFFBIN

2.0 INSTALLING THE ADAPTER MICROCODE

The SSA Adapter Microcode diskette contains the latest level of microcode

code for the SSA 4-Port Adapter, feature number 6214, the Enhanced SSA

4 Port Adapter, feature number 6216, the SSA Raid Adaptor, feature

number 6217 and 6218.

To load the microcode:

1.  Log in as root

2.  Insert the adap2137.bff Adaptor Microcode diskette in the drive rfd0

3.  Enter the command: "installp -ac all"

4.  Run cfgmgr to download the adaptor microcode to every adaptor

installed in the system.

5.  If the adapter is on an SSA loop that includes an adapter in another

system, you must update the  adapter  microcode in that other

system. See  2.1, "Installing the Adapter Microcode into Adapters in

Other Systems."

2.1 INSTALLING THE ADAPTER MICROCODE INTO ADAPTERS IN OTHER SYSTEMS

It is recommended that on an SSA loop, all SSA adapters of a particular type

use a common level of microcode.  If the adapters are all in one system,

they automatically receive a common level of microcode.    If the adapters

are in different systems, you should update the adapter microcode in

those systems as soon as is possible.

Where appropriate:

1.  Log in as root.

2.  Insert the SSA Adapter Microcode diskette into the drive rfd0.

3.  Enter the command: "installp -ac all"

4.  Run cfgmgr to download the adaptor microcode to every adaptor

installed in the system.

5.  ATTENTION:  In this step, do not run Advanced Diagnostics.

On each system that is connected to a particular SSA loop, go into System

Verification mode, and run diagnostics to ensure that  all  the  adapters

are  in  a valid loop.   (For example, a loop that contains an SSA 4-Port

Adapter can include only one other adapter.)

3.0    INSTALLING THE DISK8381.bff DISK DRIVE MICROCODE

______________________________________________________

This SSA Disk Drive Microcode diskette contains the latest level of

microcode (8372 and 8181) for SSA Disk Drives.

This level of code must be installed

on all AIX 4.1.5/4.2.1 systems containing the SSA 4 Port Raid Adaptor.

NOTE This code can only be downloaded with the service aids if the

4.1.5. or 4.2.1. SSA device drivers are installed.

3.1  Downloading the Microcode to the etc/microcode Directory of the

Specific Machine:

______________________________________________________

1.  Insert the DISK8381.bff Disk Drive Microcode diskette

in the drive rfd0

2.  Log in as root

3.  Enter the command: "installp -ac all"

3.2  Downloading the Microcode from the etc/microcode Directory of the

Specific Machine to the SSA DASD

______________________________________________________

The following steps should be followed for all systems in the loop:

1.

a/ Stop all io activity on the SSA drives you are loading uCode to.

b/ Unmount all filesystems on the SSA drives you are loading uCode to.

c/ varyoff all volume groups on the associated SSA drives

2.  Run the following command to load the latest available

microcode to all drives that require it.

NOTE  If there are other systems in the same SSA loop then ONLY perform

this step on one system in the loop.DO NOT perform it concurrently

on multiple systems in the loop.

ssadload -u

3.3 INSTALLING THE DISK DRIVE MICROCODE INTO DISKS IN OTHER SYSTEMS

______________________________________________________

Once a disk has been downloaded from one system, it is not necessary

to download the same code from other systems on the same SSA loop.

However, it is sensible to install the latest disk drive code onto

all hosts.  Where appropriate:

1.  Log in as root

2.  Insert the DISK8381.bff  Disk Drive Microcode diskette

in the drive rfd0.

3.  Enter the command: "installp -ac all"

3.4  INSTALLING THE 6561.bff DISK DRIVE MICROCODE

___________________________________________________

This microcode diskette provides the 6561 level of code for

DFHC type SSA disk drives.  If you are able to install the

DISK8381.bff SSA disk microcode as described in 3.1.1 you

can omit installation of the 6561.bff SSA disk microcode, since the

DISK8381.bff SSA disk microcode supercedes the 6561.bff SSA

disk microcode.

To load the microcode:

1.  Insert the 6561.bff Disk Drive Microcode diskette in the drive rfd0

2.  Log in as root

3.  the following steps should be followed for all systems in the loop:

a/ Stop all io activity on the SSA drives you are loading uCode to.

b/ Unmount all filesystems on the SSA drives you are loading uCode to.

c/ varyoff all volume groups on the associated SSA drives

4.  Enter the command: "installp -ac all"

5.  When you have finished installing all the microcode (adapter and disk)

then reboot or run "cfgmgr" for the changes to take effect.

If there are other systems in the same loop perform the following :

3.5 INSTALLING THE DISK DRIVE MICROCODE INTO DISKS IN OTHER SYSTEMS

___________________________________________________________________

Once a disk has been downloaded from one system, it is not necessary

to download the same code from other systems on the same SSA loop.

However, it is sensible to install the latest disk drive code onto

all hosts.  Where appropriate on each system:

5.1.  Log in as root

5.2.  Insert the 6561.bff Disk Drive Microcode diskette in the

drive rfd0

5.3.  Enter the command: "installp -ac all"

3.6 ACTIONS TO BE TAKEN WHEN NEW DISKS ARE ADDED TO THE SYSTEM

________________________________________________________________

Since the disk download mechanism used for disk8381.bff is a manual

process it is necessary to repeat from step (2) in 3.2 whenever new

disks are added to a particular system.  This ensures all

attached disks are kept at the latest level of code.

Disk drive code installed from 6561.bff is installed automatically

on SSA disks where required at boot time.