"INSTALL.DOC" File

Installation Notes for the

National Instruments NI-488 MC-GPIB Driver for OS/2

Version 1.4

Copyright 1989, 1992 National Instruments Corporation.

All Rights Reserved.

This file contains important information about installing the

National Instruments NI-488 driver for OS/2. The term "OS/2" refers to

OS/2 1.2, 1.3 and 2.0.

OS/2 Version Warnings

---------------------

Due to a problem with the virtual memory manager in earlier

revisions of OS/2 1.2, you may experience swapping problems

when using the NI-488 driver for OS/2.  The driver must perform a

long-term lock on parameter and buffer addresses supplied via

OS/2 API DosDevIOCtl.  Under certain conditions, long-term lock

fails.  Microsoft suggests using short-term lock when

long-term lock fails.  However, if too many short-term locks are

used simultaneously, OS/2 cannot do any swapping or memory

management.  If a large portion of memory is locked, you will

get a lot of swapping and the system could lock up.  If you

experience this problem, upgrade to IBM OS/2 1.3 or to 2.0.

Files

-----

The following installation files are on this distribution diskette:

INSTALL.CMD     Protect mode installation command file

INSTALL.DOC     Installation documentation

GPIB.DDP        Device driver profile file

The following driver and utility files are on this distribution

diskette:

GPIB.SYS       NI-488 driver for OS/2

IBCONF.EXE     Protect mode configuration utility

IBIC.EXE       Protect mode interactive NI-488 interface

TESTA.EXE      Protect mode installation test

TESTB.EXE      Protect mode configuration test

The following OS/2 Application Programming Interface (API) files are

on this distribution diskette:

NICTL.DOC      Using the C macro, nictl

NICTL.H        C macro include file

NICODE.H       C include file for function codes for DosDevIOCtl

NICODE.BAS     BASIC include file for function codes for

DosDevIOCtl

NICODE.FOR     FORTRAN include file for function codes for

DosDevIOCtl

NICODE.PAS     Pascal include file for function codes for

DosDevIOCtl

The following example programs and makefiles use the OS/2 API functions:

DCSAMP2.C      C sample program using device functions

DCSAMP2.MAK    Makefile for C

BCSAMP2.C      C sample program using board functions

DBSAMP2.BAS    BASIC sample program using device functions

DBSAMP2.MAK    Sample makefile for BASIC

BBSAMP2.BAS    BASIC sample program using board functions

DFSAMP2.FOR    FORTRAN sample program using device functions

DFSAMP2.MAK    Sample makefile for FORTRAN

BFSAMP2.FOR    FORTRAN sample program using board functions

DPSAMP2.PAS    Pascal sample program using device functions

DPSAMP2.MAK    Sample makefile for Pascal

BPSAMP2.PAS    Pascal sample program using board functions

The following NI-488 interface files are on this diskette:

NI488.DOC      Documentation for porting NI-488 functions

NI488.C        Translate NI-488 to OS/2 API functions (source)

NI488M.OBJ     Medium model translate object

NI488L.OBJ     Large model translate object

DECL.H         C include file for declarations

DECL.BAS       BASIC include file for declarations

CBIB.C         NI-488 interface (C source) for BASIC

CBIB.OBJ       NI-488 interface (object) for BASIC

BBIB.BAS       BASIC interface to NI-488

DECL.FOR       FORTRAN include file for declarations

DECL.PAS       Pascal include file for declarations

The following examples use NI-488 Interface functions:

DCSAMP.C       C sample program using device functions

BCSAMP.C       C sample program using board functions

BCSAMP.MAK     Sample makefile for C

DBSAMP.BAS     BASIC sample program using device functions

BBSAMP.BAS     BASIC sample program using board functions

BBSAMP.MAK     Sample makefile for BASIC

DFSAMP.FOR     FORTRAN sample program using device functions

BFSAMP.FOR     FORTRAN sample program using board functions

BFSAMP.MAK     Sample makefile for FORTRAN

DPSAMP.PAS     Pascal sample program using device functions

BPSAMP.PAS     Pascal sample program using board functions

BPSAMP.MAK     Sample makefile for Pascal

Installing the NI-488 Driver for OS/2

-------------------------------------

Your startup disk must have 450K of free space to hold a copy

of the NI-488 software contained on the distribution diskette.

Use the OS/2 DISKCOPY utility to make backup copies of your

diskette.  Put the original diskette in a safe place.

The NI-488 software can be installed using either DDINSTAL

or INSTALL.

If you start up from the C drive (hard disk), you can install

the NI-488 driver for OS/2 using the OS/2 DDINSTAL command.

This command uses the device driver profile file,

GPIB.DDP, on the NI-488 OS/2 distribution diskette.

The NI-488 OS/2 distribution diskette has a command file,

INSTALL.CMD, that installs the NI-488 software for OS/2 to a

startup disk.

Refer to Chapter 3, "Software Installation and Configuration"

, of the "Getting Started with Your MC-GPIB and NI-488

OS/2 Driver" manual that you received with your distribution

diskette for more information about using DDINSTAL or INSTALL.

If you are installing the OS/2 driver under OS/2 version

1.2, 1.3 or 2.0 refer to the section "Installation Under

OS/2" later in this file.

Installation Under OS/2

------------------------

If you install the software using DDINSTAL, an additional step

is necessary to complete the installation process.  Disregard the

installation procedure described in Chapter 3, "Software

Installation and Configuration", of your "Getting Started

with Your MC-GPIB and NI-488 OS/2 Driver" manual and complete the

following steps:

Step 1.  Preparation

Your C drive must have 450K free space to hold a copy of the

NI-488 software for OS/2 contained on the distribution diskette.

Start up your computer.

Step 2.  Run DDINSTAL

After you start up your computer, complete the following

steps:

a. Insert the NI-488 OS/2 distribution diskette in drive A.

b. Type DDINSTAL at the OS/2 command prompt to install the

software. Press the <Enter> key.

c. You are prompted to insert the Device Support Supplement

diskette in drive A.  Since the NI-488 OS/2

distribution diskette is the Device Support

Supplement diskette, simply press the <Enter> key.

d. You are prompted to select a device.  Press the spacebar

and then press the <Enter> key.

DDINSTAL creates a directory called MC-GPIB2 on the C: disk

and copies the files on the distribution disk into this

directory.

DDINSTAL automatically adds the following statement to the

config.sys file:

DEVICE=C:\MC-GPIB2\GPIB.SYS