National Instruments NI-488.2M OS/2 Driver for the MC-GPIB.

Version 2.0.

Copyright 1993  National Instruments Corporation.

All Rights Reserved.

Thank you for purchasing the MC-GPIB interface board and NI-488.2M

OS/2 driver.

We appreciate communicating with the people who use our products.  We are

also very interested in hearing about the applications you develop using

our products.  To make it easy for you to communicate with us, Appendix E

in the "NI-488.2M OS/2 Software Reference Manual" contains forms for you

to complete.  Completing these forms before contacting National Instruments

facilitates communication.

Important Changes in NI-488.2M driver for OS/2

-----------------------------------------------

In order to conform with the IEEE 488.2 specifications and to

take advantage of the 32-bit features of OS/2 2.0, we have

incorporated many improvements and changes in the NI-488.2M driver for

OS/2. The following is a list of all the new features and changes that

have been incorporated in this release. It is important that you read

this list and become familiar with these new features and changes so

that you can understand how they may affect your existing applications.

1.    The NI-488.2M driver for OS/2 works only with National Instruments

IEEE 488.2 hardware. If you have an IEEE 488.1 board contact

National Instruments for hardware upgrade information.

2.    In order to be fully compliant with the IEEE 488.2 protocol,

the NI-488.2M driver works with a new set of functions called the

NI-488.2M routines. Please refer to the Function Reference Manual

for a complete description of these routines.

3.    The NI-488.2M MC-GPIB driver automatically configures

the base address, dma and interrupt settings to match the

corresponding hardware settings.

4.    Device status bits have their own defintions which are

different from board status bits. This, may affect

device level WAIT functions in your existing applications.

Refer to the Software Reference Manual for a description

of the device and board status bits.

5.     The  language interface file is a Dynamic Link Library (DLL)

instead of an OBJ file. Please refer to the Software Reference

Manual for more information on how to link your C applications

with the language interface.

6.     The NI-488.2M driver can handle Automatic Serial Polling. Please

refer to the Software Reference Manual for more information on

autopolling.

7.     The  NI-488.2M driver can handle interrupt disabling. Please refer

to the Software Reference Manual for information on disabling

interrupts through ibconf. Also, refer to the ibconfig function

in the Function Reference Manual for information on disabling

interrupts dynamically.

8.     The NI-488.2M driver can handle Asynchronous I/O for NI-488M calls.

Please refer to the functions ibwrta, ibrda and ibcmda in

the Function Reference Manual for a complete description of

Asynchronous I/O functions.

9.     The  NI-488.2M driver can handle I/O transfers larger than 64K.

However you need to use NI-488M or NI-488.2M calls to take

advantage of this feature. Due to an OS/2 problem, the OS/2

API calls  DosRead or DosWrite with a buffer larger than 64K

fails at the kernel even before the call is passed on to the

device driver.

10.    Some new NI-488M functions have been implemented in the NI-488.2M

driver. They are : ibask (device and board), ibconfig (device

and board), ibdev (device only), iblines (board only), ibln

(device and board), ibsrq, ibstop (device and board). Please

refer to the Function Reference Manual for a complete

description of these calls.

11.    ibdma and the API calls that disable or enable DMA are valid only

for board-level calls.

12.    The NI-488.2M driver has some new features which you may

find useful. Please refer to board-level and device-level

ibconfig functions in the Function Reference Manual for more

information about these features.